| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-46 Filename: q15d46.doc Generated: 15 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Gunnar Hellström National Post and Telecom Agency Box 5398 S-102 49 Stockholm, Sweden | Tel: Fax: Email: | +46 751 100 501 +46 8 556 002 06 gunnar.hellstrom@omnitor.se |
| Title: | Test sequence Irene |  |  |
| Purpose: | Information |  |  |

# Sign language test sequence "Irene"

This paper and accompanying video sequence presents a video clip with sign language intended to be used as test material for video coding. It contains sign language performed with natural speed.

The sequence is named "Irene" after the signing person. It is in Swedish sign language and copied from Swedish Educational TV. It shows the same head-to-stomach view that is usually used in personal use of videophones for signing. It is recorded in PAL with 25 fps. The intention is to provide it on request on CD, in YCbCr 4:2:0 format in CIF and QCIF 25 fps.

The clip contains a fair amount of fingerspelling, the feature of sign language that from experience is most challenging for videophone communication. Finger movements are so rapid that significant hand positions in some cases are recorded in one frame only, and any framerate reduction below 25 means a loss in language contents.

The clip "Irene", contains a number of eye blinks that are typical grammatical components of sign language used as sentence delimiters. They are short, in many cases only occurring on one or two  frames.

From the clips it can clearly be seen that the quality requirements stated before are valid.

- Over 20 fps is needed for full perception of sign language

- 12.5 - 15 fps gives some usability

- CIF is needed for smooth perception

- QCIF gives some usability but also some uncertainty.

- Blur in fingers, and face worse than corresponding to QCIF disturbs perception.

- The video delay from camera to receiving screen s.b. below 0,8 sec to be ueable, that should be below 0.4 sec to be good.