| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-48 Filename: q15d48.doc Generated: 14 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Michael Gallant, Faouzi Kossentini University of British Columbia 2356 Main Mall, Vancouver, B.C. Canada, V6T 1Z4 | Tel: Fax: Email: | (604)-822-4988 (604)-822-5949 mikeg@ee.ubc.ca |
| Title: | Clarification of Test-Model Description of Fast-Search Motion Estimation |  |  |
| Purpose: | Proposal |  |  |

_____________________________

The current description of the integer pixel fast search in the test model document proposal Q15-C-15, section 2.1.2, does not indicate that the search is started at the predicted motion vector or that the (0,0) vector is also checked.    Also it states that up to four untested motion vectors may be tested in each search iteration. In fact four motion vectors are only tested for the first layer of the search, in all remaining layers only from one to three motion vectors have to be tested.    Also, it is not specified what is done if the motion vector moves beyond the permitted range of MV values.

We propose that the following description replace the current description in the test model document.

### 2.1.2 Integer Pixel Fast Search Motion Estimation

An efficient alternative to the full search can be implemented as described in this section.

The search center is the median predicted motion vector as defined in 6.1.1 and F.2 of the standard. The (0,0) vector, if different than the predicted motion vector, is also searched and favored, as described in section 2.1.1 of this document.

The algorithm proceeds by sequentially searching diamond-shaped layers, each of which contains the four immediate neighbors of the current search center. Layer i+1 is then centered at the point of minimum SAD of layer i. Thus successive layers have different centers and contain at most three untested candidate motion vectors, except for the first layer around the predicted motion vector, which contains four untested candidate motion vectors.

The search is stopped only after (1) all candidate motion vectors in the current layer have been considered and the minimum SAD value of the current layer is larger than that of the previous layer or (2) after the search reaches the boundary of the allowable search region and attempts to go beyond this boundary.