| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-50 Filename: q15d50.doc Generated: 20 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Michael Gallant, Guy Cote, Faouzi Kossentini University of British Columbia 2356 Main Mall, Vancouver, B.C. Canada, V6T 1Z4 | Tel: Fax: Email: | (604)-822-4988 (604)-822-5949 mikeg@ee.ubc.ca |
| Title: | Proposal for New H.263++/H.26L Simulation Conditions |  |  |
| Purpose: | Proposal |  |  |

_____________________________

We propose that

- The rate-distortion based coder described in Document Q15-D-49 be used to generate anchor bitstreams for future core experiments

- Annexes D (+/- 32 pixels), F, J, I and T be used to generate anchor bitstreams for future core experiments

- In addition, we propose the following modifications/clarifications to the suggested simulation conditions presented in Document Q15-C-47

- Bit-rate be measured as the average of all bits per frame including the first INTRA frame.

- PSNR be measured as the arithmetic mean of the PSNR values for each color component of each frame.

- Objective comparisons based on varied frame skip coding be removed, possibly replaced by objective comparisons based on constant bit-rate coding.