| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-51 Filename: q15d51.doc Generated: 15 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Guy Côté, Michael Gallant and Faouzi Kossentini Dept. of Electrical and Computer Engineering University of British Columbia 2356 Main Mall Vancouver, B.C., V6T 1Z4 Canada | Tel: Fax: Email: | (604)-822-4988 (604)-822-5949 guyc@ece.ubc.ca |
| Title: | Description and Results for Rate-Distortion Based Quantization |  |  |
| Purpose: | Information and Test Model Proposal |  |  |

_____________________________

# Introduction

This document presents results for rate-distortion based quantization compatible with the H.263V.2 syntax.    We also propose the inclusion of rate-distortion quantization to the Test Model for the high complexity mode of the Test Model.

# Rate-Distortion Quantization

The proposed method is similar to the one first reported in document Q15-A-49 (Entropy Constrained results, when choosing the decisions levels only).    However, the proposed method allows an expanding dead-zone providing better rate-distortion results.    Moreover, the choice of the Lagrangian parameter is easy to compute and does not require additional iterations. The decision levels for quantization are given by the general equation (see document Q15-D-31):

|LEVEL| = [|COF| + (f-p) × QP] / (2 × QP).

We perform rate-distortion based quantization by minimizing the following Lagrangian function:

where f∈(0,2] in steps of 0.1 is used to locate the quantization thresholds (see document Q15-D-31). SSD is the sum of the squared differences between the original block sand its reconstruction s'and is given by:

SSD(s,f)=i=1,j=116,16s(i,j)−s'(i,j,f)2,

and R(f) is the number of bits associated with choosing fincluding bits for the macroblock header and coefficients for all six DCT blocks.    The Lagrangian parameter is determined by:

λQUANT=QP2,

where QP is the macroblock quantization parameter.

# Experimental Results

Objective results are first presented using the same conditions as document Q15-D-34 (Adaptive Quantization) and further results are presented in terms of rate-distortion curves according to the conditions and first table of document Q15-C-47, Suggested Simulation Conditions for H.263ver3/H26L Experiments until the Tampere meeting (QCIF resolution only).    The rate-distortion quantization method is applied to INTRA and INTER macroclocks.    Rate-distortion based quantization gives up to 0.5 dB improvement in PSNR (or up to 10% reduction in bitrate).    We propose its inclusion in the Test Model for the high complexity mode.    We also believe that the benefits of the proposed adaptive quantization method (documents Q15-A-49, Q15-B-42, Q15-C-34, and Q15-D-34) can be approached with the rate-distortion based quantization method.

| Comparing Results to Document Q15-D-34 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | (QP, Skip) | Original Quantizer Q15-D-34 |  | Adaptive Quantizer Q15-D-34 |  | TMN-3.1.2 |  | RD Quantizer |  |
|  |  | Y PSNR | Rate(I+P) | Y PSNR | Rate(I+P) | Y PSNR | Rate(I+P) | Y PSNR | Rate(I+P) |
| Coastguard | (16, 2) | 28.26 | 38.14 | 28.53 | 37.44 | 28.29 | 39.21 | 28.24 | 37.76 |
| Coastguard | (16, 1) | 28.36 | 46.95 | 28.56 | 45.65 | 28.34 | 49.23 | 28.26 | 46.58 |
| Coastguard | (16, 0) | 28.48 | 64.08 | 28.58 | 60.20 | 28.41 | 70.59 | 28.21 | 63.33 |
| Foreman | (16, 2) | 29.59 | 40.62 | 29.73 | 37.83 | 29.42 | 36.06 | 29.45 | 35.26 |
| Foreman | (16, 1) | 29.59 | 50.18 | 29.71 | 46.85 | 29.51 | 45.00 | 29.45 | 43.24 |
| Foreman | (16, 0) | 29.63 | 65.01 | 29.71 | 60.02 | 29.57 | 68.34 | 29.44 | 63.12 |
| Akiyo | (16, 2) | 31.87 | 6.29 | 31.81 | 5.84 | 31.83 | 7.09 | 31.70 | 6.24 |
| Akiyo | (16, 1) | 31.82 | 7.61 | 31.78 | 6.91 | 31.82 | 8.83 | 31.64 | 7.50 |
| Akiyo | (16, 0) | 31.88 | 11.06 | 31.76 | 9.79 | 31.84 | 13.08 | 31.59 | 11.59 |
| Carphone | (16, 2) | 30.21 | 25.90 | 30.41 | 24.62 | 30.15 | 28.62 | 30.16 | 28.21 |
| Carphone | (16, 1) | 30.22 | 35.20 | 30.36 | 33.12 | 30.17 | 36.58 | 30.09 | 36.64 |
| Carphone | (16, 0) | 30.23 | 57.61 | 30.36 | 54.22 | 30.19 | 58.89 | 30.07 | 56.96 |

| Results from Table 1 in Document Q15-C-47, QCIF resolution only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No options |  |  |  | RD Quantization |  |  |  |
| Sequence | QP | Y PSNR | Cb PSNR | Cr PSNR | Bit rate | Y PSNR | Cr PSNR | Cb PSNR | Bit rate |
| MaD QCIF | 4 | 37.47 | 40.60 | 40.44 | 121.39 | 38.11 | 40.92 | 40.78 | 130.83 |
|  | 5 | 36.31 | 39.76 | 39.67 | 93.69 | 36.27 | 39.67 | 39.59 | 90.69 |
|  | 7 | 34.09 | 38.31 | 38.27 | 56.47 | 33.93 | 38.13 | 38.05 | 52.39 |
|  | 10 | 31.87 | 36.85 | 36.67 | 30.76 | 31.81 | 36.62 | 36.53 | 28.64 |
|  | 15 | 29.97 | 35.41 | 35.36 | 16.06 | 29.72 | 35.09 | 35.11 | 13.73 |
|  | 25 | 28.01 | 33.64 | 33.72 | 7.67 | 27.78 | 33.33 | 33.22 | 6.78 |
| Hall QCIF | 4 | 38.81 | 40.67 | 42.41 | 69.12 | 39.05 | 40.60 | 42.38 | 68.93 |
|  | 5 | 37.60 | 39.91 | 41.75 | 55.32 | 37.37 | 39.66 | 41.58 | 50.84 |
|  | 7 | 35.17 | 38.58 | 40.78 | 35.23 | 34.91 | 38.34 | 40.54 | 33.59 |
|  | 10 | 32.66 | 37.34 | 39.87 | 21.31 | 32.53 | 37.13 | 39.79 | 21.82 |
|  | 15 | 30.62 | 36.30 | 39.56 | 12.57 | 30.62 | 36.08 | 39.61 | 12.81 |
|  | 25 | 29.12 | 35.65 | 39.34 | 6.72 | 29.15 | 35.49 | 39.17 | 6.63 |
| Container QCIF | 4 | 37.65 | 42.07 | 41.77 | 78.42 | 38.09 | 42.63 | 42.34 | 79.77 |
|  | 5 | 36.52 | 41.28 | 40.85 | 59.93 | 36.41 | 41.36 | 40.93 | 55.50 |
|  | 7 | 34.27 | 39.86 | 39.11 | 36.26 | 34.10 | 39.82 | 38.92 | 33.36 |
|  | 10 | 31.99 | 38.06 | 37.36 | 21.03 | 32.01 | 38.16 | 37.27 | 20.07 |
|  | 15 | 29.92 | 36.89 | 36.20 | 12.01 | 29.79 | 36.84 | 36.14 | 10.60 |
|  | 25 | 28.23 | 36.33 | 35.36 | 6.11 | 28.03 | 36.26 | 35.20 | 5.22 |
| Silent QCIF | 4 | 37.37 | 40.10 | 41.02 | 95.11 | 38.17 | 40.69 | 41.58 | 102.30 |
|  | 5 | 36.19 | 39.10 | 40.16 | 75.92 | 36.27 | 39.15 | 40.29 | 75.73 |
|  | 7 | 33.78 | 37.39 | 38.75 | 49.74 | 33.79 | 37.36 | 38.78 | 49.28 |
|  | 10 | 31.68 | 35.66 | 37.22 | 30.61 | 31.70 | 35.55 | 37.20 | 31.14 |
|  | 15 | 29.85 | 34.52 | 36.09 | 18.32 | 29.87 | 34.47 | 36.01 | 17.98 |
|  | 25 | 28.54 | 33.72 | 35.34 | 10.46 | 28.50 | 33.79 | 35.28 | 10.22 |
| News QCIF | 4 | 37.96 | 40.78 | 41.36 | 105.46 | 38.78 | 41.54 | 42.03 | 112.31 |
|  | 5 | 36.81 | 39.95 | 40.57 | 84.79 | 36.86 | 40.15 | 40.74 | 84.14 |
|  | 7 | 34.29 | 37.95 | 38.82 | 56.77 | 34.32 | 38.06 | 38.88 | 56.19 |
|  | 10 | 31.95 | 36.13 | 37.21 | 35.42 | 32.08 | 36.28 | 37.24 | 36.07 |
|  | 15 | 29.96 | 34.58 | 35.98 | 20.91 | 29.88 | 34.50 | 35.95 | 20.22 |
|  | 25 | 28.20 | 33.36 | 35.13 | 10.74 | 28.06 | 33.39 | 35.15 | 10.52 |
| Foreman QCIF | 4 | 37.44 | 41.35 | 41.98 | 198.49 | 38.30 | 41.96 | 42.65 | 215.75 |
|  | 5 | 36.37 | 40.65 | 41.20 | 157.79 | 36.53 | 40.70 | 41.35 | 158.44 |
|  | 7 | 34.22 | 39.17 | 39.61 | 104.20 | 34.30 | 39.14 | 39.68 | 103.16 |
|  | 10 | 32.01 | 37.68 | 37.91 | 65.42 | 32.25 | 37.78 | 38.12 | 66.38 |
|  | 15 | 30.00 | 36.36 | 36.38 | 40.59 | 29.99 | 36.33 | 36.44 | 39.42 |
|  | 25 | 27.75 | 34.95 | 34.49 | 23.79 | 27.75 | 34.85 | 34.50 | 23.42 |