| QUESTION(s): | Q.11/16 |  |
| --- | --- | --- |
| SOURCE: | ITU-T-SG11 Rapporteur Groups for Q.11 & Q.15/16 Tampere Finland |  |
| TITLE: | Liaison response to SG15/Q.4 regarding Multimedia on ADSL |  |
| LIAISON STATEMENT |  |  |
| TO: | ITU-SG15/Q.4 |  |
| APPROVAL: | Agreed to at the Rapporteurs Group Meeting Tampere Finland |  |
| FOR: | Information |  |
| DEADLINE: | N/A |  |
| CONTACT: | Tom Geary, Q.11 Rapporteur Rockwell International 4311 Jamboree Rd. P.O. Box C Newport Beach CA 92660 USA P +1 714 446-9641 F +1 714 446-9642 Email:tom.geary@rss.rockwell.com | Gary Sullivan, Q.15 Rapporteur PictureTel 100 Minuteman Rd., M/S 635, 3B28 Andover, MA 01810 USA P +1 978 623 4324 F +1 978 749 2804 Email:garys@pictel.com |

SG16/Q.11 and Q.15 were pleased to receive the liaison from SG15/Q.4 soliciting comment for the xDSL work in SG15.

Regarding the requirements of physical layer transports for the H.series Multimedia Terminals, we consider bitrate, bit error rate and latency delay as the key items which drive the acceptance of Multimedia terminals in the market place.    H.series terminals are defined for operation as point-to-point and multi-point configurations.    There are Recommendations requiring bit rates from >9.6kbit/s to > 9 Mbit/s and targeting bit error rates of from 10^-3 to 10^-10, but optimum performance is achieved with bit error rates of < 10^-6.    Algorithms utilized for audio and video vary in error robustness and some are designed typically to conceal bit and burst errors of 50-100 bits in duration.    Latency is of prime importance, especially on lower bit rate connections.    The target end-to-end one way delay is 150-200 mSec.    When transcoding is necessary via gateways or MCUs to connect differing types of terminals, the above latency and delay numbers are the maximum found to be acceptable by users.    Transport can be synchronous, frame based or packet based and utilizes the common protocols such as HDLC, H.223, ATM and IP.    Call startup is also of importance where times in excess of 2 seconds are found to be objectionable.

Regarding the aspects of ADSL, point-to-point configurations receive best acceptance when the bit rate is near symmetrical.    On the other hand, asymmetrical transport can result in optimum bandwidth utilization in multi-point configurations where each end terminal must receive (downstream) more video data than it will transmit (upstream) and it is expected ADSL will fulfill this application nicely.

For your convenience, a summary table of all the current Recommendations applicable to video conferencing is included.

Table 1

Audiovisual communication systems in various network environments

| Network | GSTN & Wireless | N-ISDN | N-ISDN | Guaranteed QoS LANs | Non-guaranteed QoS LANs | ATM (B-ISDN, ATM LANs) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel capacity | up to 33.6 kbit/s | up to 1536 or 1920 kbit/s | up to 1536 or 1920 kbit/s | up to 6/16 Mbit/s | up to 10/100 Mbit/s | up to 600 Mbit/s |
| Characteristics | • ubiquitous | • circuit based • existing | • circuit based • existing | • similar to N-ISDN | • Internet • packet loss prone | • future basic network |
| Total system (date of the first approval) | H.324 (96/03) | H.320 (90/12) | H.324 Annex D (98/02) | H.322 (96/03) | H.323 (96/11) | H.310 (96/11) H.321 (96/03) |
| Audio coding | G.723.1 G.729/A | G.711 G.722 G.723.1 G.728 G.729 | G.711 G.722 G.723.1 G.728 G.729 | G.711 G.722 G.723.1 G.728 G.729 | G.711 G.722 G.723.1 G.728 G.729 | G.711 G.722 G.723.1 G.728 G.729 ISO/IEC 11172-3 |
| Video coding | H.261 H.263 | H.261 H.262 H.263 | H.261 H.262 H.263 | H.261 H.262 H.263 | H.261, H.263 | H.261 H.262 H.263 |
| Data | T.120 etc. | T.120 etc. | T.120 etc. | T.120 etc. | T.120 etc. | T.120 etc. |
| System control | H.245 | H.242 | H.245 | H.242 | H.245 | H.242 (for H.321) H.245 (for native H.310) |
| Multimedia multiplex and synchronization | H.223 | H.221 | H.223 | H.221 | H.225.0, TCP/IP etc. | H.221 (for H.321) H.222.0 H.222.1 (for native H.310) |
| Call setup signaling | National standards | Q.931 | Q.931 & National standards | Q.931 | Q.931, H.225.0 | Q.2931 |

Notes

•	Audiovisual system H.324/M for mobile environments is under study.

•	H.321 is for adaptation of H.320 in B-ISDN.

•	Difference between B-ISDN and customer premises ATM networks (ATM LANs) is mostly in administrative aspects.

We thank you for your interest and welcome any further requests for information or clarifications.

END