| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-64 Filename: q15d64.doc Generated: 23 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Stephan Wenger TU Berlin, Sekr. FR6-3 Franklinstr. 28-29 D-10587 Berlin | Tel: Fax: Email: | +1 415 297-2063 +1 604 736-7853 stewe@cs.tu-berlin.de |
| Title: | Ideas for a H.263+ Test model and H.324M-Interfacing for mobile Video |  |  |
| Purpose: | Proposal and Discussion |  |  |

_____________________________

## 1. Document Structure

This document serves three purposes:

- it describes a ‘packetization’ scheme, similar to the one defined for the Internet/RTP environment (as defined in Q15-D20) for interfacing a H.263+ codec to H.223,

- it outlines a test model for H.263+ error resilience mechanisms for the mobile case (based on the ‘packetization’ scheme as defined in 1), and

- it briefly outlines some ideas on common test conditions for this envionments.

## 2. Interfacing H.263+ to the H.324 mobile world.

### 2.1. Description of the ‘as is’ interface

Currently, the H.223 transport protocol (which is used in H.324m) accepts coded video fed into the AL3 interface.    AL3 packetizes this data and optionally uses re-transmission mechanisms to improve the quality of the outgoing bitstream.    As far as the author understands, this re-transmission mechanism, however, is rarely used because of the induced delay.

The packetization process of AL3 is unaware of the payload (video-) data.    There is, in particular, no definition that, for example, pictures or GOBs start at a packet boundary.    It is, however, guarantied that outgoing packets are received unchanged, with the exception of possible transmission errors.    That is, neither the byte position of the packet start in the video stream, nor the length of the packet must be expected to be changed.    Both bit errors (often with a bursty characteristic) and packet losses can occur.

The packetization process in the Mux layer (hierarchically below the AL3 layer) is out of the scope of this discussion.

### 2.2. Proposed procedure

Translating the protocol structure of H.223 to the Internet world, the AL3 of H.223 roughly corresponds to the RTP service interface.    H.263+ is a true media codec in the RTP sense.    At the moment, there is no such mechanism like a ‘payload header’ (as described for the RTP case in Q15-D20) in H.223/H.263 interfacing.    The author feels, that this will lead to lower video quality and recommends to add such a ‘payload header’ to an appropriate document, probably as a new adaptation layer of H.223 (as suggested by Tom Geary).    Some ideas of the technical content are drafted below, but surely need further investigation and verification.

### 2.3. Packetization procedure

Since in mobile environments the error rates can be substantially, and some parts of a coded picture are more important than other parts, it is reasonable to protect those part more than other parts.    This is especially true for the picture header.    Additionally, it is helpful to have a redundant picture header in a packet not containing the ‘original’ picture header so that other pictures packets can be decoded even if the original picture header got lost.    This idea follows the outlines of Q15-D20, see this document and Q15-D17 for justifications.

We propose to send one or several consecutive GOBs or a Slice in one AL3 packet.    The 16 zero-bits of the picture and GOB/Slice header should not be included in the packet, to save bitrate.    This can be done, because by definition each packet starts with a picture/GOB/Slice header.

The last packet of each picture should start with a 8 bit fixed length number, which contains the number of bits of the picture header of this picture (excluding the first 16 zero-bits).    After this 8 length bits, a redundant copy of the original picture header follows.    After that, the byte-aligned payload follows.    The following figure shows an example of a picture divided into three picture Segments.

PictureHeader		Data of Segment 1 (variable length, filled with 0-bits)

SegmentHeader		Data of Segment 2 (variable length, filled with 0-bits)

LengthOf PictureHeader	PictureHeader		Segment Header	Data of Segment 3

A picture can, in contrast to the RTP case, easily be divided into several picture segments for better resiliency, because the amount of packetization information is substantially lower (typically only 5 bytes per packet including a 16 bit CRC over the payload).

### 2.4. Decoding procedure

The picture should be decoded as follows:

First, all packets of a picture are received from the network.    If packet 1 of a picture is received correctly and contains no bit errors (as can be figured out by checking the CRC of this packet), this packet can be fed into the decoder unchanged.    If it contains bit errors, the picture header is stored for future use and the rest of the packet is discarded (not fed into the decoder).

All other packet except the last one are fed into the decoder only, if they do not contain bit-errors.

The last packet contains a redundant copy of the picture header.    This redundant copy is used as follows: If the first packet was error free, the redundant copy is not used.    If the first packet was lost, the redundant picture header is used instead.    In all other cases it is unsure that a error-free picture header was received and therefore all packets of that picture are discarded, regardless of their correctness.

If at least one picture header was received correctly, this picture header can be used in conjunction with all correctly received packets for their decoding.

## 3. Test model description

This paragraph outlines a description of a test model.    The test model assumes an implementation the payload spec as described in section 2.    To simulate the erronous channel, the uncoded sequence is first coded as described below.    After, the packet loss is simulated by deleting the relevant GOBs out of the data stream.    Last, the remaining data is decoded by a compliant decoder employing error concealment techniques as described below.

- use 1 packet == 1 GOB,

- use error concealment techniques as described in Takamura et al AVSPN97 (similar to MPEG 2’s error recovery for motion vectors).

- Use decoding procedure as described above.

- Use Annex N with back channel as one sub-model

- For high error rates 10^-3 simulate NACK messages with 2 second delay and reliable transmission

- For medium error rates 10^-4 simulate NACK messages with 1 second delay and reliable transmission

- For low error rates 10^-5 simulate NACK messages with 0.5 seconds delay and reliable transmission

- Use Annex N VRC 2-3 as the other sub-model

## 4. Common conditions

- Error patterns @ various error rates

- Packet loss simulation environment (TU Berlins ploss, or software from UCLA?)

- Data streams like in Q15-D65, or only QCIF

- Constant Bit-rate?    Would be more realistic