| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-64 r1 Filename: q15d64.doc Generated: 23 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Stephan Wenger TU Berlin, Sekr. FR6-3 Franklinstr. 28-29 D-10587 Berlin | Tel: Fax: Email: | +1 415 297-2063 +1 604 736-7853 stewe@cs.tu-berlin.de |
| Title: | Ideas for a H.263+ Test model and H.324M-Interfacing for mobile Video |  |  |
| Purpose: | Proposal and Discussion |  |  |

_____________________________

## 1. Document Structure

This document serves four purposes:

- It introduces the video-oriented Q.15 experts to the error-prone environment of H.324 mobile applications.    More specific, it describes the possible situations, which can happen when a H.223 AL3 (video-) SDU is delivered to a receiver using a mobile channel.    Note, that this part is written by a non-H.324m expert based on the knowledge gained in an one hours introduction of a mobile expert during Q.15’th Tampere meeting.    Any corrections are very welcome.

- It describes a ‘packetization’ scheme, similar to the one defined for the Internet/RTP environment (as defined in Q15-D20) for interfacing a H.263+ codec to H.223.    The author feels, that it might be necessary to make this part normative for similar reasons as on the Internet payload specifications have normative status.

- It outlines a test model for H.263+ error resilience mechanisms for the mobile case (based on the ‘packetization’ scheme as defined in 2).    This includes some restrictions regarding the use of the packetization scheme, and information about the applicable optional modes of H.263+.

- It briefly outlines some ideas on common test conditions for this environment.

## 2. H.223’s AL3: background information

H.324 terminals rely on H.223 transport.    H.223 includes three “adaptation layers” (Alx) for various data types.

AL3 is intended to be used for video data, although it is unaware of any video specific data structures like pictures, GOBs, synchronization points.    It features an optional 16 bit CRC over its SDUs, and a re-transmission mechanism for damaged SDUs.    As far as the author knows, re-transmission is rarely used in current (wireline) H.324 systems because of the induced delay (and the author does not expect that this will change for future wireless systems).    It is not allowed to re-transmit SDU more than one time, leading to a well defined delivery time but also to a non-guarantied quality of delivery: SDUs can be lost or incorrectly transmitted even if the re-transmission algorithm is used.    The maximum length of a (variable length) AL3 SDU is 2048 bytes and it is always byte-oriented.    AL3 is part of the network-independent sections of H.223, thus used over both wireline and wireless networks.

Hierarchically “below” AL3’s mechanisms, AL3 PDUs are delivered to H.223’s multiplexing mechanism.    This layer achieves a low latency multiplexing of various ALx PDUs according to their importance.    ALx PDUs are broken up into smaller multiplexing entities, and the link is time-multiplexed between those smaller Mux PDUs.    This mechanism allows sending large ALx PDUs without hurting critical latency requirements of other data types, especially audio.

All the described mechanisms are common for all H.223 systems, including wireline and wireless.    Mechanisms below the multiplex layer are defined in both the main body of H.223 and its Annexes A, B, and C to provide a scalable error protection/mechanism for networks of various types.    H.223 “baseline” (without the use of any of the Annexes) is intended for wireline networks and has shown acceptable performance using today’s modem technology.    For the much more error-prone mobile channels, the three Annexes provide scalable error-protection at the cost of additional computational complexity and bit-rate overhead.    It is beyond the scope of this document to describe the mechanisms of those Annexes in detail, please see [H.223] and [Q11-E-xxx] for more information.

A receiver get the following set of information with each AL3 packet:

- The packet itself (payload),

- A Mux-layer error-indication (EI-) flag, which provides information about the integrity of the payload and the length field (described next).    A not-ok setting of the EI flag means an error condition in the mux layer.    It is, however, not guarantied, that an ok of the EI flag indicates an error-free transmission at the Mux layer.    Note, that error- free means in that case, that the packetization structure of the Mux-layer is unharmed and thus the length of the payload and its content are (most likely) correct; bit errors in the payload can still occur.

- A length field, which indicates the length of the payload.    This field might or might not be correct.    It is likely although not guarantied, that the length field is correct if the EI-flag does not report a Mux-error.

When sending AL3 SDUs over an error-prone mobile channel using Annex B of H.223, one of the following four situations might occur at the receiver:

| Situation | EI-Flag | Length | CRC | Situation |
| --- | --- | --- | --- | --- |
| 1 | X | 0 | X | Packet loss. No information to decode |
| 2 | Not Ok | Non 0 | X | Mux error. CRC most likely incorrect. Packet might contain other payload, length might be different from sent SDU length, whole Mux SDUs might be lost. Note, that almost always the CRC will be ‘Not ok’. |
| 3 | Ok | Non 0 | Not Ok | Likely no Mux error. Bit errors (or, unlikely, whole MUX packets with wrong information) in payload. |
| 4 | Ok | Non 0 | Ok | Save to decode (error condition very unlikely) |

Table 1: H.223 error conditions using AL3’s CRC, receiver’s view.

It seems to the author, that there is consent in Q.11’s mobile group that it is necessary to gain as much information as possible out of coded data.    Obviously, situation 4 received data can be decoded and situation 1 data cannot be decoded (because non-existent).    It seems to be accepted knowledge to Q.11 mobile experts, that as much as possible data out of situation 3 and (maybe) situation 2 SDUs should be decoded as well.

## 2. A packetization scheme for H.263+ into AL3 SDUs

This section describes a general packetization scheme for H.263+ coded video and H.223’s AL3 SDUs.    The author feels that it might be valuable to mandate some of the characteristics as normative content, similar to the H.263+ on RTP packetization RFC for H.323.    Procedural questions, however, are outside the scope of this document.

Based on the current H.223 and H.263 recommendations, it is possible (and implementation dependent) to split H.263 video data at any byte-boundary.    For this packetization scheme, however, more limited mechanisms are proposed to achieve a better re-synchronization capability without employing special VLC-resynchronization mechanisms in video decoders.    Note that the test-model description below set even more strict limits.    The purpose of this section is to provide ideas of normative content that could to be added to future versions of H.223, or to a H.324 implementers guide, along with a brief justification.

H.263 video coding works with variable length codes (VLCs), which make re-synchronization to the data stream difficult if the synchronization was lost due to bit errors or to packet losses.    To facilitate this re-synchronization, H.263 (as many other video coding standard including H.261 and H.262) provides synchronization points in the form of picture, GOB and slice headers, which can be easily found in case of lost synchronization.    H.263 is designed that information following a synchronization point can be decoded independently from previous information.    That is, none of the prediction mechanisms within a single coded picture (such as motion vector prediction or advanced INTRA prediction) is used across synchronization points.    It is therefore proposed to add the following phrase at an appropriate place, maybe within an H.324 implementers guide:

<Begin of proposed normative content>

Each synchronization point should be at the start of an AL3 SDU’s payload.        Think about that!

<End of proposed normative content>

Since AL3 SDUs can be up to 2048 bytes long, the bit-rate of the proposed mobile channels is comparably low, and a coded picture can contain a lot of synchronization point, it is unlikely that any application needs to send AL3 packets not starting with a synchronization point, although this is allowed by the specification (similar to follow-up packets in case of the H.263+ RTP payload spec).

### 2.1. Encoding and packetization

Due to the low overhead of the H.223 packetization, which is in the order of 5 bytes per AL3-SDU (including the 16 bit CRC), it is reasonable to use much smaller packet sizes than for example on the Internet.    It is therefore recommended to break up the coded picture into several packets.    For now, the author proposes to use the GOB mechanism, because scan-order slices are not really compatible with the Reference Picture Selection mode (as outlined elsewhere) and not too much experience with rectangular slices is available to the author.

Each GOB containing macroblock data (there is at least one macroblock in the GOB with COD=0) should be coded with a header.    If one ore more preceding GOBs do not contain any coded data, they should be included into this GOB (and the GOB address should be adjusted accordingly).    At the start of a picture, a picture header (which is the first GOB header) is coded.    After, all empty GOBs are coded and the first non-empty GOB is coded as well in the same packet.    The second packet contains the GOB-header of the first GOB after the previously coded non-empty GOB, all subsequent empty GOBs and the first non-empty GOB.    This scheme should be repeated until the end of the picture is reached.

The reason for not coding the headers of empty GOBs is the incurred overhead, which is in the order of 10 bytes (5 bytes packetization overhead, plus 40 bits for the GOB header; the 11 COD bitsdo not count as overhead because they have to be coded anyway).    A coded picture with GOBs 1, 5, 7 and 8 empty would be packetized as follows:

| Packet | Content |
| --- | --- |
| 1 | Picture header, 11 COD bits indicating empty GOB 1, GOB 2 without header |
| 2 | Header and data of GOB 3 |
| 3 | Header and data of GOB 4 |
| 4 | Header of GOB 5, 11 COD bits indicating empty GOB 5, GOB 6 |
| 5 | Header of GOB 7, 22 COD bits indicating empty GOB 7 and 8, GOB 9 |

Table xxx: example for packetizing a QCIF picture with empty GOBs

### 2.2. Decoding

Since the packetization scheme operates on GOB level, and each packet starts with GOB headers, those packets are essentially self-contained provided the picture header of the picture the GOB belongs to is available.    The mechanism of error control beyond the requirement of starting a packet at a synchronization point of H.263+ is outside the scope of this set of rules.    Section 3 provides one idea of a mechanism about which the author feels that it might produce good results.

Generally, all packets containing payload have to be collected and combined to one coded picture.    If complete packets are lost, error concealment techniques can be used to achieve a higher picture quality.    Packets containing bit errors might be discarded or (partially) decoded depending on the decoders capabilities.

## 3. Test model description

This section outlines a description of the Q15 test model for the mobile communication case.    The test model assumes an implementation of the payload specification as described in section 2, along with some additional restrictions.

The idea of the test-model is to send GOBs in AL3 packets, with AL3’s CRC mechanism used and AL3’s re-transmission mechanism not used.    All packets that are received along with an evidence of possible damage (by one of the various error-checking means as AL3-CRC, and MUX-EI flag, see section 1) are discarded.    That is, no situation 2, 3, and 4 packets in the terminology of section 1 are used for decoding, although they might contain useful information.    Note, that this simplified mechanism will possibly lead to a lower performance than to allow some form of a decoding attempt of a possible damaged AL3 SDU (to recover at least parts of the damaged data).    Future enhancements might be necessary.    Descriptions of possible mechanisms along with simulation results are solicited.

Since some wireless applications do allow the use of a feedback-channel and others do not, two cases are distinguished in the following:

- Uni-directional transmission is applicable for multicast or broadcast channels and protocol hierarchies without means to convey a back channel message, and

- Bi-directional transmission, applicable for all other applications.

Please note that H.324 mobile terminals likely belong to the second categrory, if the codecs implement Annex N and H.245 implements the back channel transport mechanisms (both are optional).

### 3.1. Uni-directional transmission

To simulate the erroneous channel, the sequence is first coded using the H.263+ specific coding tools.    After, the data is packetized as described in section 2.    The packet loss/damage is simulated by deleting or patching packets.    The mobile group of Q.11/16 will evaluate the possibility to provide a tool, which performs this task based on a simulation of the H.223 behavior and error patters as used by this group.    Document Q11-F-xx (identical with Q15-E-xx, to be provided) describes Q15’s requirement to such a tool.    Last, the remaining data is decoded by a compliant decoder employing error control and error concealment techniques.

### 3.1.1. Encoding

H.263 encoders should use Video Redundancy Coding (VRC) on individual GOBs to achieve error resilience.    To achieve the best possible error robustness, the Sync GOB of VRC should be evenly distributed over the GOBs.    That is, each picture should contain as few sync-GOBs as possible.    For picture sizes up to QCIF, a 2/5 VRC scheme makes it possible to have a maximum of one sync GOB per picture.    The following table shows the VRC scheme (expressed in number of threads, and picture per thread) to be used for a specific error characteristic (expressed in error probability and error distribution.

| Error Probability | Error Distribution | VRC scheme |
| --- | --- | --- |
| 0 | X | None |
| X | X | 2 threads, 5 pictures per thread |

Please note that this table needs further investigation as soon as a simulation environment becomes available.    It might be a valuable addition to add the sending of whole I-GOBs or to mandate a higher I-macroblock frequency to avoid error accumulation due to VRC.

### 3.1.2. Packetization

The packetization process should follow the guidelines described in section 2.

### 3.1.3. Erroneous channel simulation

To simulate the error prone channel, the sequence of packets resulting out of the packetization process should be processed through a damage simulator.    This simulator applies bit errors by patching individual bits in a packet and multiplex errors (which can have the several forms as described in section.    The output of the damage simulator is a sequence of packets, along with a sequence of EI-bits, CRCs and length information.

### 3.1.4. De-packetization, reconstruction of the coded data

The de-packetization process discards all damaged data packets and reconstructs a data stream by concatenating data packets identified as correct (by checking the CRC and the EI-flag).    Please note that the resulting bit-stream is not fully H.263 compliant, because GOBs might be missing.    Since each packet starts with a GOB header (as mandated by the packetization procedure of section 2), missing GOBs can be identified by the decoder and appropriate error-concealment action might be performed.

[It might also be possible to reconstruct a correct data stream be adding pseudo-GOBs (with uncoded blocks) and/or by adding an external signal to identify those GOBs (this can also be done using non-standardized supplementary information.    Suggestions are solicited.]

Please note that the described procedure might lead to non-optimal results because packets with single bit errors are completely discarded, resulting in the loss of partially correct information.    This part of the test model is likely to change in the future to incorporate more sophisticated mechanisms.

### 3.1.5. Decoding

The decoding process is straightforward, if no error concealment techniques are used.    There are, however, error concealment techniques known which greatly improve the quality of the reproduced image sequence [Akira97???].    The test model might include one or several of those error concealment techniques in the future.    Suggestions are solicited.

### 3.2. Bi-directional transmission

If the scope of the communication relationship is limited to the point-to-point case (as it is true for most H.324/M systems), then most system environments including H.324/M allow to employ the powerful mechanisms using a feedback channel.    The feedback channel informs the encoder about the decoder’s status regarding the successful decoding of pictures or picture segments (e. g GOBs).    This information can be used by the encoder to choose an earlier reference picture (or reference picture segment) to avoid erroneous or not-decoded reference material.

H.263+ features two different ways of conveying such back-channel information: the information can be multiplexed in the video bitstream of the opposite direction (VideoMux back channel) or it can be conveyed by other (external) means.    Since it would be very difficult to simulate the VideoMux back channel (because the characteristics of the video information of the opposite direction are not known and would have to be simulated simultaneously) the test model assumes an external back channel.    The only standardized way to convey such external back channel messages in H.324M terminals is through H.245.    H.245 information uses a guarantied quality transmission (which implies a variable and potentially indefinite delay) which makes a correct estimation of the simulation conditions difficult, too.    It seems, however, easier for both Q.11 Mobile experts and Q.15 experts to use a fixed delay for the back-channel (rather to simulate this delay).    Q.11 experts are asked in Q11-Fxx to provide a reasonable estimation for such a delay.

Encode a picture (possibly using RPS back channel information)

H.263+ coded picture

Packetization

Each packet contains one non-empty GOB

Back channel message delay simulation, provided by Q.11 experts

AL3 SDUs (packets)

Loss/Damage Simulation,

provided by Q.11 experts

Back channel message

(possibly damaged) AL3 SDUs, CRC, Mux EI-Flag

Back channel message generator, CRC check, de-packetizer

H.263+ picture (Containing per GOB RPS info)

RPS capable H.263+ decoder, w/ optional error concealment

### 3.2.1. Encoder

The encoder processes one picture at a given time, using the (simulated) back channel information regarding lost GOBs.    This is done by not coding an early temporal reference in Annex N’s forward channel information if the GOB was not reported to be lost.    Otherwise, an appropriate temporal reference is coded.

### 3.2.2. Packetization

The packetization process should follow the procedures described in section 2.

### 3.2.3. Loss/Damage simulation

See 3.1.3.

### 3.2.4. Back channel generation, De-packetization

See 3.1.4.    Additionally, all known to be lost GOBs are reported to the Back channel delay simulation.    Please note, that this might include empty GOBs, since it is impossible to identify empty/non-empty GOBs after a sequential loss of more than two packets.    If only one, or two packets are lost sequentially, only those GOBs known to contain information should be included in the back-channel message.

### 3.2.5. Decoder

See 3.1.5.

### 3.2.6. Back channel message delay

The back channel message delay module should delay any back-channel message in an appropriate way.    The delay should be expressed in intervals corresponding to the picture clock frequency.    For now, Q.15 experts recommend to use a fixed delay, but Q.11 experts are encouraged to investigate the possibility to develop a tool which provides a better simulation of a real-world’s system.    It should be assumed, that the back-channel message is conveyed over H.245, employing the codepoints available in H.245R3 for this purpose.

## 4. Common conditions

- Error patterns @ various error rates

- Data streams like in Q15-D65, or only QCIF?    QCIF and SQCIF?

- Constant bit-rate?    Would be more realistic.    What bit-rates?    Specify rate control?

- Reasonable delay for back channel message (possibly including distribution)

## 5. To do list

- Integration of Toshi’s ACK/NACK back channel model (VideoMux only???)

- Write Q.11 F-xx document with interface specification for damage/delay simulator.

- Error concealment.    Error recovery.