| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fifth Meeting: Whistler, British Columbia, CA, 21-24 July, 1998 | Document Q15-E-37 Filename: q15e37.doc Generated: 17 July ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Guy Côté Stephan Wenger Michael Gallant University of British Columbia 2356 Main Mall Vancouver, BC, Canada V6T 1Z4 | Tel: Fax: Email: | (604) 822-4988 (604) 822-5949 guyc,stewe,mikeg@ece.ubc.ca |
| Title: | Effects of standard-compliant macroblock intra refresh on rate-distortion performance |  |  |
| Purpose: | Information, proposal |  |  |

_____________________________

# 1.0	Introduction

This document describes performance of different Intra Macroclock (I-MB) refresh rates.    The H.263 standard specifies that the I-MB refresh rate be at least one in every 132 for MBs with coefficient information. However, this requirement has not been incorporated into reference software. To ensure that this requirement is met, a note should be added to the Test Model.

# 2.0	Intra-Macroblock refresh pattern

The minimum I-MB refresh rate is once every 132 macroclocks, and only for macroblocks with coefficient information.    An update pattern should be defined in the Test Model.    Using a complete I-frame is clearly not a good solution. We propose to use a random pattern to produce the initial update pattern, so the I-MBs are evenly distributed among frames over the I-MB refresh frequency.    Specifically, we propose to use the pseudo random generator of H.263 Annex A for easy cross verification.    While this leads to a somewhat lower performance in PSNR for short sequences (because some MBs are I-refreshed well before due-time), it seems to be the only way to avoid large I-MB bursts that increase the size of individual pictures substantially.

# 3.0	Simulation results

In order to assess the coding overhead of adding I-MBs, we include some simulation results in Table 1.    Each sequence was first coded with a fixed QP of 13 and no I-MB refresh.    After, the same QP was chosen for the same sequence with one I-MB after the spatially corresponding MB was transmitted 132 times with coefficient info.    The third simulation verified that result, using TMN8 rate control on the (higher) bit-rate yielded by I-MB update coding, without forcing I-MB update, to measure the resulting PSNR.

Table 1: Simulation Results for Default 132 Intra Mabroclock Refresh Rate

| Sequence | I-MB | QP | Bitrate(Kbps) | Y-PSNR |
| --- | --- | --- | --- | --- |
| Coastguard | 0 | 13 | 97.6 | 27.8 |
|  | 132 | 13 | 99.8 | 27.8 |
|  | 0 | TMN-8 RC | 99.8 | 27.8 |
| Paris | 0 | 13 | 53.4 | 27.5 |
|  | 132 | 13 | 55.0 | 27.5 |
|  | 0 | TMN-8 RC | 55.0 | 28.2 |
| Grandma | 0 | 13 | 7.5 | 32.0 |
|  | 132 | 13 | 7.8 | 32.0 |
|  | 0 | TMN-8 RC | 7.8 | 32.1 |

The default 132 Intra MB refresh rate has little effect on bitrate/PSNR for the sequences tested:    between 2 and 4% bitrate increase.    The PSNR difference for such a bitrate increase (when using it without I-MB update) was insignificant.

# 4.0	Proposal for test model

We propose to add the following text to the Test Model:

INTRA Macroblock Refresh:    As required by the standard, every MB must be coded as INTRA at least once every 132 times when coefficients are transmitted.      To avoid large bursts of INTRA macroblocks for short periods, a simple pattern for the macroblock update is to randomly initialize each macroblock’s update counter [0, 132). The pseudo random generator of H.263 Annex A shall be used.    The algorithm is described as follows:

MB_intra_update[xpos][ypos] = count incremented by one every time coefficient information is sent for the macroblock at position (xpos, ypos), namely MB[xpos][ypos].

We choose the INTRA mode for a given macroblock as:

Initialize after an I-frame: MB_intra_update[xpos][ypos] = rand(0, 132)

While (more non I-frames to encode)

If(MB_intra_update[xpos][ypos] == 132 AND MB[xpos][ypos] contains coefficient) {

Encode MB[xpos][ypos] in INTRA mode;

} else if (MB[xpos][ypos] contains coefficient) {

++MB_intra_update[xpos][ypos];

}

}