| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Sixth Meeting: Seoul, South Korea, 3-6 November, 1998 | Document Q15-F-18 Filename: q15f18.doc Generated: 24 Nov. ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | R.J.Fryer University of Strathclyde Department of Computer Science 26 Richmond Street Glasgow G1 1XH Scotland, UK | Tel: Fax: Email: | +44 (0) 141 548 3387 +44 (0) 141 552 5330 rjf@cs.strath.ac.uk |
| Title: | The Strathclyde Compression Transform : An Outline Technical Description |  |  |
| Purpose: | Information in support of a Proposal for H.26L |  |  |

_____________________________

# Abstract

The Strathclyde Compression Transform (SCT) is a video compression algorithm that has been targeted thus far on low bandwidth conversational applications that mandate minimal, constant frame transmission delays at high frame rate.    Based on variable dimension vector quantisation, the SCT encodes video frames in a manner which gives precise control over the number of bits assigned to each frame and hence over the transmission delay.    Though the real-time conversational mode is the preferred one in current applications using the SCT, other rate control schemes can be implemented equally easily, so enabling the algorithm to be applied to a wide range of other tasks.

# Introduction

Two key factors determine the usability of a video codec for conversational services: the frame capture to frame display time and the perceptual quality of the received sequences. To maintain a natural conversation it is necessary for the participants to see and hear each other's reactions to vocal and visual signals in as short a time as possible. The frame capture to frame display interval (which includes the frame encoding, transmission and decoding times) disrupts this interchange. Perceptual quality of the display affects the usability, both aesthetically and in terms of being able to interpret subtle “face and body language”.    These issues have been addressed in previous contributions

A key aspect of the SCT is its ability to deliver, within a very close approximation, the precise bit allocation per frame on a frame-by frame basis.    There are no global adjustments to be made [such as the value for Q in H.263] which have a content-dependent effect.    As an alternative to the minimum delay mode, bit allocation can be on some other criterion [such as achievement of a pre-determined quality measure], which can again be set on a frame-by frame basis.    This flexibility makes the development of rate control schemes straightforward.    Again this has been addressed in a previous contribution.

# The Basis of the SCT

The Strathclyde Compression Transform [SCT] is an extension of Variable Dimension Vector Quantisation.    It can be enhanced at need to include any number of block approximation and/or block improvement schemes including DCT.

## Variable Rate Image Compression

A typical video sequence, even one limited to a “talking head”, will have periods with high detail and/or motion separated by periods of little motion or detail.    In such circumstances it would seem, at first sight, reasonable to focus available bandwidth on sub-sequences with high detail and/or motion at the expense of the remaining frames.    In such a case the bit budget available to compress a single video frame is determined by the instantaneous frame rate and currently available bandwidth.    As both these factors in principle are variable, the compression ratio for each frame must be controllable.

### Variable Dimension Vector Quantisation

Vector quantisation (VQ) has been found to be an efficient technique for data compression [1,2].    Following Shannon's rate-distortion theory, a better compression is achievable by coding vectors rather than scalars.    VQ represents a mapping from a k-dimensional space Rk to a finite subset Y of Rk. This finite set Y is called a codebook. To code a data sequence, an encoder assigns to each data vector x in the set Rk an index corresponding to a vector in Y, that in turn is mapped to a codeword y in the set Y by a decoder.

In its simplest implementation for image compression, VQ requires that an image is divided into fixed sized non-overlapping sub-image blocks, each represented by a data vector x in the set Rk.    The pixels in the sub-image block correspond to the elements in the data vector x.    Each image data vector is compared with the entries of a suitable codebook Y and the index i of the codebook entry yi most similar to the source data vector transmitted to a receiver.    At the receiver, the index accesses the corresponding entry from an identical codebook, allowing reconstruction of an approximation to the original image.

While simple, this implementation can only compress at a single compression ratio determined by the size of the block used to partition the image and the number of entries in the codebook.    Changing either the block size or the codebook size will allow the compression ratio to vary, but involves the training and storage of many codebooks to take account of all block and codebook sizes used if the compression ratio is to be a dynamic variable.

An alternative is to use several distinct block sizes to encode an image with a codebook assigned to each block size.    Variable dimension vector quantisation operates on the principle that large blocks should be used to encode areas of low detail, while small blocks should encode areas of high detail [3].    It is a technique whereby an image can be compressed to a pre-determined compression ratio by defining, for instance, the number of blocks overall that may be used in the encoding of the image.

### Image Segmentation

The established implementation of image compression by variable dimension VQ is to divide an image using a quad-tree format [4].    This process, hereafter termed total block replacement, divides an image into a grid of p by p sub-image blocks.    Each of these blocks may be replaced by four p/2 by p/2 blocks, which in turn may each be replaced by four p/4 by p/4 blocks, etc. according to image detail and the required compression ratio.    The number of blocks used to encode the image is determined by the compression ratio which defines the total number of bits available (the bit budget).

While total block replacement does take account of the detail in a source image and can deliver a precise compression ratio, the encoding can include blocks that give little or no improvement [5].    For example, four quarter-sized blocks may replace a single block in order better to represent detail contained in just one quadrant of the original block.    In this case only one of the quarter sized blocks was required to improve compression quality, the other three being redundant.

Consider an alternative sub-division scheme.    Termed block occlusion, any size of block at any valid position may be used in the encoding of an image, provided it improves the match between the original and compressed representation.    Block occlusion typically gives a better compression than total block replacement [5] as it targets blocks according to where they will do the most good.

The process of block occlusion operates by generating a number of source image block approximations, hereafter termed patches.    The compression ratio is defined by the number of patches used in the encoding of an image.    The process starts by generating a patch for each of the largest blocks and adding these patches to the encoding of the image.    This provides an encoding of the picture at the highest compression ratio possible.

Thereafter, while the encoded version of the image has fewer bits than the frame bit budget, the block is chosen that a) has not been approximated by a patch and b) gives the greatest error between the current compressed representation and source image. This block is approximated by a patch, which is added to the encoding.    Any patch that is now occluded by the addition is removed from the encoding if it no longer improves quality (e.g. when totally occluded).

### The Best First Rule

Vector quantisation rarely matches a codebook vector to a data vector without error. Rather the codebook vector represents an approximation to the data vector.    The difference between a set of codebook vectors and the data vectors they represent determines the quality of the compression.

For image compression utilising sub-image blocks of variable size, for a given compression ratio many encodings of an image are possible. Each of these encodings gives a different figure for compression quality. As noted above, the usual way to select a good encoding of an image is to target those areas of the image to be compressed that have the greatest error between the current compressed representation and the original image.

However this approach does not take account of how well the codebook vectors chosen to approximate an image block actually match the original image.    The best first rule, rather than targeting the image blocks most in error, targets those image blocks that will give the biggest improvement. Improvement for a block is defined as the change in the difference between the compressed representation and original image prior to and following the approximation of the block. For best first block occlusion, the block encoded is the block, regardless of size, that when encoded and added to the compressed representation gives the greatest improvement of the compressed representation relative to the original image.

## Video Compression

The block occlusion and best first processes together provide a means of obtaining high quality, variable rate compression over a wide range of compression ratios.    With these processes, the compression of video is a simple extension of still image compression.

### The Strathclyde Compression Transform

Our video codec, which we have called the Strathclyde Compression Transform (SCT), uses block occlusion with block selection according to the best-first rule.    The SCT assumes that each frame may be encoded by block approximation. These blocks may be of any size and at any position, though in practice a limited number of distinct sizes are permitted at a limited number of positions to simplify block addressing and codebook construction.

The SCT assumes that the last encoded frame represents the starting state (initial compressed representation) for the compression of a new frame. Note that the addition of a patch to the compressed representation results in replacement of data over its own area within the encoding of the frame.

### The VQ Codebook

The SCT codec uses a codebook with four distinct codepages, one for each block size.    The codebook was created using a proprietary self-organising neural network [8,9] trained on a set of images of natural and man made scenes.    The codebook appears to be near-universal in the sense of being able to represent all types of image content with equal ease, at least within the spatial frequency bands that they are tuned to.    A section of the largest codepage is presented in Figure 1 for illustration.

Note that none of the test images and video sequences used to evaluate the SCT was included in the training set used to create the codebooks. The four codepages are for patches of size 32 by 32, 16 by 16, 8 by 8 and size 4 by 4.    There are 256 patches on each page.

To keep the size, search time and memory required by the codebooks low, the codebook vectors are normalised with respect to contrast and mean intensity [6,7].    A block approximation by a codebook is thus a triple consisting of the codebook index, a contrast adjustment and a mean-intensity offset. The quality of the compression is determined by the quantisation of the contrast and mean-intensity values, by the size and content of the codebook and by the overhead of encoding block position.

Figure 1	25 of the 256 image patches drawn from the 32x32 pixel codepage of the current SCT codebook.    Each patch, here normalised to the interval 0 to 255 with a mean of 127, internally is normalised to ±1 with a mean of zero.    The allowing of negative contrast effectively doubles the size of the codebook.

### Motion Compensation

To allow control of bit allocation, the SCT does not separate the encoding of motion from that of new detail.    Rather it treats the last encoded frame or frames as extensions to the VQ codebook.    If a block is better approximated by copying a region of equivalent area from a past frame rather than from the standard VQ codebook, it will be added to the compressed representation as a block encoded by block translation.

In the past only the preceding frame has been used with a finite search area of +/- 15 pixels.    In the latest version an arbitrary number of past frames can be used and the size and shape of the motion compensation search area may be adjusted.    It has also been found to be beneficial to use half-pixel motion compensation, though only in the immediate vicinity of the zero-motion position.

### Residual Encoding

The ultimate quality of an encoded frame is set by the codebook used.    Inevitably a point can be reached where there is no block in the image which, if approximated by a codebook patch or translated block of another frame, can lead to an improved approximation of the source image.    Except in the most exceptional of circumstances this ‘best possible’ representation will differ, perhaps markedly, from the source image.

To deal with the effect of this quality limitation, the SCT has an optional mode that adds encoding of residual signal to the codebook and motion-compensated block possibilities.    This residual option is based on approximation by means of Haar wavelets.    The size of block currently ‘improvable’ in this way is limited to 4x4 pixels.    The technique is relatively expensive in bandwidth terms meaning that the overheads mandated by the use of this mode can more than cancel the image improvement achieved by the extra mode.    Nevertheless, in suitable circumstances, the use of this mode can be beneficial.    By its use it can be ensured that the representation of a stationary scene becomes identical to the source available at the camera.

## Versions of the SCT

There are currently two versions of the SCT in regular use — the so-called ‘off-line’ and ‘real-time’ modes.

### The Off-Line SCT Codec

This is the version described above and used to generate the data and demonstration sequences presented at this meeting.    It is not real-time for most purposes since processing of a frame of video typically takes between 0.5 and 1.0 seconds on a P300 MMX Windows95 PC.    This duration is, of course, affected by frame size and channel bitrate.    Surveillance systems using frame-rates of 1fps or less have successfully been demonstrated using this codec.

The complexity of the compression algorithm is dominated by the search activity involved.    For each candidate patch the system must examine each entry in the corresponding codebook and each permitted position within the motion compensation search space.    When the option is enabled the algorithm must compute the wavelet residual encoding of the block also.    For each potential match the ‘improvability’ achievable by its use must be computed and the means with the highest improvability selected.    The candidate patches must then be ranked according to their improvability and added to the encoding sequentially.    Re-evaluation of brightness and contrast for partially occluded patches already added to the encoding is also performed as the occlusions take place.

A considerable proportion of the encoding time is thus taken up in tight simple search loops.    Work is under way to accelerate the performance of these loops and it is felt that there is still a gain to be made.    Alternatively, simple hardware assist could be provided which would immediately make the codec real-time in practice.

It should be noted that the quality of the codebook is such that it has not been found to be beneficial to enable the wavelet residual encoding option for channel bandwidths up to 128kbps.    Wavelet residual encoding has not been used in generating any of the demonstration material for this meeting.

### The Real-time Codec

This version resembles the off-line codec in all important respects except for the codebooks used.    To reduce the overhead of codebook search an independent codebook has been developed that has greatly improved indexing properties.    The quality deliverable by this codebook is inferior to that of the off-line codebook by up to a decibel, but frame-rates of 10 to 15 per second on images of approximately QCIF size are easily achievable using standard PC architectures.    Because of the reduction in codebook quality it is advantageous to enable the wavelet residual encoding option.    The quality of talking head sequences can be high in such circumstances.

## Summary and Advantages of the SCT

In summary, therefore

- The demonstrated SCT codec is based on variable-dimension vector quantisation supplemented by motion compensation and (optionally) by residual encoding

- The codec takes advantage of block occlusion to maximise the efficiency of image representation

- The codec adopts ‘best first’ criteria for patch selection to maximise achievable quality

- Additional block representation and/or residual encoding schemes could be added easily without need to modify the basic approach

- The codec uses large scale and very high quality codebooks created using a proprietary neural-network.

- The search structure of the codec makes hardware implementations both cheap and simple, though useful versions can be run in real-time on standard PC architectures.

- The achievable quality using this codec is already comparable with H.263 though not using compatible options and having been developed by a small team with limited time.    It is highly likely that the attention of an expert standardisation team, bringing other particular skills to bear, could achieve quality improvements comparable to those achieved for H.263 over its development cycle.

## References

- R.M. Gray. Vector quantization. IEEE ASSP Magazine, pp 4-29, April 1984.

- Nasrabadi and R.A. King. Image coding using vector quantization: A review. IEEE Transactions on Communications, Vol.36 pp 957-971, 1988.

- A. Gersho and R.M. Gray. Vector Quantization and Signal Compression. Kluwer Academic Publishers, 1992.

- H. Samet. The quad tree and related hierarchical data structures.    ACM Computer Surveys, Vol. 16 pp 188--260, June 1984.

- R.B. Lambert, R.J. Fryer, W.P. Cockshott, and D.R. McGregor. A comparison of variable dimension vector quantization techniques for image compression. Proc. European Conference on Multimedia Applications, Services and Techniques, pp 655--670, Louvain-la-Neuve, May 1996.

- R.L. Baker and R.M. Gray. Differential vector quantization of achromatic imagery. In Proceedings of the International Picture Coding Symposium, March 1983.

- T. Murakami, K. Asai, and E. Yamazaki. A design of vector quantizer for video signals.    In Proceedings of the International Picture Coding Symposium, March 1983.

- R.B. Lambert, W.P. Cockshott, and R.J. Fryer.    A scalable neural architecture combining unsupervised and suggestive learning.    Proc. Int. Conference on Neural Networks and Genetic Algorithms, pp 167--176, Innsbruck, April 1993.

- R.B. Lambert, R.J. Fryer, and W.P. Cockshott. Recognition.    Patent IPN WO 94/24636.

e.g. LBC-96-238

LBC-97-051

Though DCT  has not proved beneficial at channel bitrates up to 128kbps and higher

of any valid size at any valid position

typically the block with highest squared error

Except, of course, for the effects of camera noise.

Patent filed

Further work [scheduled but not yet commenced] promises further improvements to these codebooks in terms of representational power.