| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Sixth Meeting: Seoul, Korea, 3-6 November, 1998 | Document Q15-F-32 Filename: q15f32.doc Generated: 28. Oct ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wiegand, Niko Färber, and Bernd Girod Telecommunications Laboratory University Erlangen-Nuremberg Germany Barry Andrews 8x8, Inc Santa Clara, CA, USA | Tel: Fax: Email: Tel: Fax: Email: | (9131) 85-27117 (Germany) (9131) 85-28849 (Germany) wiegand@nt.e-technik.uni-erlangen.de faerber@nt.e-technik.uni-erlangen.de girod@nt.e-technik.uni-erlangen.de (408) 654-0905 (USA) (408) 980-0432 (USA) andrews@8x8.com |
| Title: | Draft for Annex on Enhanced Reference Picture Selection |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Summary

This document presents a draft for Annex N including the Enhanced Reference Picture Selection capabilities. The draft unifies the syntax of the error resilience capabilities of Annex N (Reference Picture Selection) and the coding efficiency capabilities of multi-frame prediction. The changes that are proposed to be made to Annex N of H.263 Version 2 are underlined.

The extensions to achieve Enhanced Reference Picture Selection mainly addresses the multi-frame buffering syntax at the picture, GOB, or slice layer and the extended motion vector syntax on the macroblock layer. The purpose of Enhanced Reference Picture Selection is the extension of motion compensation to multi-frame prediction. The use of multi-frame motion-compensated prediction can increase the coding efficiency of the video coding system. The proposed syntax is based on the core experiement description on Enhanced Reference Picture Selection, see Q15-E-52.

The coding efficiency capabilities of Enhanced Reference Picture Selection are combined with the error resilience capabilities of Reference Picture Selection. The combination of the buffering scheme with the error resilience capabilities enables the use of the NEWPRED scheme in all back-channel modes. Furthermore, a CRC check is specified to detect buffer misalignment between encoder and decoder.

The extension of motion compensation to multi-frame prediction is achieved by extending the motion vector by a picture reference parameter. This extended motion vector may be used to address a block for motion-compensated prediction in each of the multiple reference frames. The picture reference parameter is a variable length code specifying a buffer index. For that, the reference frames are assembled in a buffering scheme that can be controlled by the encoder.

Annex N
Reference Picture Selection mode

(This annex forms an integral part of this Recommendation.)

## N.1	Introduction

This annex describes the optional Reference Picture Selection mode of H.263, which operates using a modified interframe prediction method called “NEWPRED.” The capability of this mode of H.263 is signaled by external means (for example Recommendation H.245). The amount of additional picture memory accommodated in the decoder may also be signaled by external means to help the memory management at the encoder. This mode can use backward channel messages sent from a decoder to an encoder to inform the encoder which part of which pictures have been correctly decoded at the decoder.  The use of this mode is indicated in the PLUSPTYPE field of the picture header. This mode has two back-channel mode switches which define whether a backward channel is used and what kind of messages are returned on that backward channel from the decoder, and has another submode defined in terms of the channel for the backward channel messages.

The two back-channel mode switches of this mode determine the type of messages sent on the back-channel, specifying whether ACK (acknowledgment messages), or NACK (non-acknowledgment messages) are sent.  Together, the two switches define four basic methods of operation:

- NEITHER:  In which no back-channel data is returned from the decoder to the encoder,

- ACK: In which the decoder returns only acknowledgment messages,

- NACK: In which the decoder returns only non-acknowledgment messages, and

- ACK+NACK:  In which the decoder returns both acknowledgment and non-acknowledgment messages.

The specific type of messages to be sent as outlined above is indicated in the picture header.

There are also two methods of operation in terms of the channel for backward channel messages:

- Separate Logical Channel mode:  This method of operation delivers back-channel data through a separate logical channel in the multiplex layer of the system, and

- VideoMux mode: This method of operation delivers back-channel data for received video within the forward video data of a video stream of encoded data.

This annex specifies a syntax for the backward channel messages as well as for forward channel data.

If Enhanced Reference Picture Selection is indicated, motion compensation can be extended to multi-frame prediction. The use of multi-frame motion-compensated prediction can increase the coding efficiency of the video coding system.  The extension of motion compensation to multi-frame prediction is achieved by extending the motion vector by a picture reference parameter. This extended motion vector may be used to address a block for motion-compensated prediction in each of the multiple reference frames. The picture reference parameter is a variable length code specifying a buffer index. For that, the reference frames are assembled in a buffering scheme that can be controlled by the encoder.  The coding efficiency capabilities of Enhanced Reference Picture Selection are combined with the error resilience capabilities of Reference Picture Selection. The combination of the buffering scheme with the error resilience capabilities enables the use of the NEWPRED scheme in all back-channel modes. Furthermore, a CRC check is specified to detect buffer misalignment between encoder and decoder.  The specifications addressing the Enhanced Reference Picture Selection mode determine the extensions to the syntax for the frame buffering scheme and the motion vector.

## N.2	Video source coding algorithm

The source coder of this mode is shown in generalized form in Figure N.1. This figure shows a structure which uses a number of picture memories. The source coder may select one of the picture memories to suppress the temporal error propagation due to the inter-frame coding. The Independent Segment Decoding mode (see Annex R), which treats boundaries of GOBs with non-empty headers or slices as picture boundaries, can be used to avoid error propagation due to motion compensation across the boundaries of the GOBs or slices when this mode is applied to a smaller unit than a picture, such as a GOB or slice.  The information to signal which picture is selected for prediction is included in the encoded bitstream. The strategy used by the encoder to select the picture to be used for prediction is out of the scope of this Recommendation.

FIGURE N.1/H.263

Source coder for NEWPRED

If Enhanced Reference Picture Selection is utilized, the video source coding algorithm can be extended to multi-frame motion compensation. The error resilience capabilities of the Reference Picture Selection mode are preserved. However, enhanced coding efficiency is achieved by allowing reference picture selection on the macroblock level. A picture buffering scheme with relative indexing is employed for efficient addressing of pictures in the multi-frame buffer.

The multi-frame buffer control may work in several modes of operation. For example, a sliding window over time can be accommodated by the buffer control unit. Past decoded and reconstructed pictures starting with the immediately preceding one ending with the picture which is decoded M time instants before are collected in the picture memories P0… PM-1. If the number of pictures maximally accommodated by the multiple reference picture buffer corresponds to M, the motion estimation when coding picture m: 0£m£M-1can utilize m pictures. In case m³M, the maximum number of pictures M can be used. Alternatively, an adaptive buffer control can be utilized.

If multi-frame motion compensation is indicated, for each macroblock of type INTER or INTER+Q, the source coder selects a motion vector and a picture reference if the coded macroblock indication (COD) is set to "0". The extension of the motion vector by the picture reference is called extended motion vector. The information to signal the extended motion vector is multiplexed in the encoded bit-stream. If the coded macroblock indication (COD) is set to "1", a picture reference is transmitted.

If multi-frame motion compensation is indicated, for each macroblock of type INTER4V or INTER4V+Q, the source coder selects extended motion vectors.

## N.3	Channel for Back-Channel messages

This mode has two methods of operation in terms of the type of channel for back-channel messages. One is the separate logical channel mode and the other is the videomux mode. The separate logical channel mode is a preferred mode and delivers the back-channel message defined in N.4.2 through the dedicated logical channel. The videomux mode is prepared for the system which cannot set up the separate extra channel for the back-channel messages due to the restriction of the number of channels’ combinations. The videomux mode delivers the back-channel messages through the same logical channel with the forward video data in the opposite direction.

### N.3.1 Separate Logical Channel Mode

The separate logical channel mode delivers back-channel messages through a dedicated logical channel opened only for the purpose of the back-channel messages. The association mechanism with the forward channel which delivers video data is provided by external means (for example Recommendation H.245).  Separate logical channel operation requires an external framing mechanism for synchronization of the messages within the back channel, as the back-channel syntax defined herein contains no synchronization flag words.

### N.3.2 VideoMux Mode

The videomux mode delivers the back-channel messages through the same logical channel with the forward video data in the opposite direction. The syntax of the multiplexed bitstream is described in the section N.4.1.  The back-channel messages may be inserted by using the Back-Channel message Indication (BCI) in the GOB or slice header.

## N.4	Syntax

### N.4.1	Forward Channel

The syntax for the forward channel data which conveys the compressed video signal is altered only in the Group of Blocks (GOB) or slice layer.

The syntax of the GOB layer is illustrated in Figure N.2. The fields of  TRI, TR, TRPI, TRP, BCI, and BCM are added to Figure 9.

FIGURE N.2/H.263

Structure of GOB layer for NEWPRED

When the optional Slice Structured mode (see Annex K) is in use, the syntax of the slice layer is modified in the same way as the GOB layer. The syntax is illustrated Figure N.3.

FIGURE N.3/H.263

Structure of Slice layer for NEWPRED

If Enhanced Reference Picture Selection is indicated, the syntax for the forward channel data which conveys the compressed video signal is altered in the Picture, Group of Blocks (GOB) or slice, or macroblock layer. (Syntax diagrams tbd.)

N.4.1.1	TRI (Temporal Reference Indicator) (1 bit)

TRI indicates whether or not the following TR field is present.

0: TR field is not present.
1: TR field is present.

N.4.1.2	Temporal Reference (TR) (8/10 bits)

When present, TR is an eight bit number unless a custom picture clock frequency is in use, in which case it is a ten bit number consisting of the concatenation of ETR and TR of the picture header.

#### N.4.1.3	TRPI (Temporal Reference for Prediction Indicator) (1 bit)

TRPI indicates whether or not the following TRP field is present.

0: TRP field is not present.
1: TRP field is present.

TRPI shall be equal to zero whenever the picture is an I or EI picture.

#### N.4.1.4	Temporal Reference for Prediction (TRP) (10 bits)

When present (as indicated in TRPI), TRP indicates the Temporal Reference which is used for prediction of the encoding, except in the case of B pictures and the B-picture part of an Improved PB frame. For B pictures or the B-picture part of an Improved PB frame, the picture having the temporal reference TRP is used for the prediction in the forward direction.  (Prediction in the reverse-temporal direction always uses the immediately temporally-subsequent picture.)  TRP is a ten bit number.  If a custom picture clock frequency was not in use for the reference picture, the two MSBs of TRP are zero and the LSBs contain the eight bit TR found in the picture header of the reference picture.  If a custom picture clock frequency was in use for the reference picture, TRP is a ten bit number consisting of the concatenation of ETR and TR from the reference picture header.

When TRP is not present, the most recent temporally-previous anchor picture shall be used for prediction, as when not in the Reference Picture Selection mode.  TRP is valid until the next PSC, GSC, or SSC.

If Enhanced Reference Picture Selection is indicated, the meaning of TRP changes. TRP specifies that:

- all pictures in the multi-frame buffer having buffer-index less than the picture associated with TRP are not used for motion compensation,

- the picture in the multi-frame buffer assigned to TRP that has buffer index 0,

- the pictures in the multi-frame buffer having buffer-index greater than the picture associated with TRP are available for motion compensation.

For that, it is assumed that the decoder replicates the multi-frame buffer of the encoder according to the specified buffering mode. The TRs are attached to the buffered frames.

#### N.4.1.5	Back-Channel message Indication (BCI) (Variable Length)

This field contains one or two bits, when set to "1", it signals the presence of the following video Back-Channel Message (BCM) field.  Otherwise, the field value is "01", which indicates the absence or the end of the video back-channel message field. Combinations of BCM and BCI may not be present, and may be repeated when present. BCI shall always be set to "01" if the videomux mode is not in use.

#### N.4.1.6	Back-Channel Message (BCM) (Variable Length)

The Back-Channel Message having syntax as defined in the section N.4.2, which is present only if the preceding BCI field is set to "1".

#### N.4.1.7.1	Number of Reference Pictures Active (NRA) (Variable Length)

A variable length codeword that is present only if Enhanced Reference Picture Selection is indicated. NRA specifies the number of active reference frames for decoding the current picture. For that, the NRA value is transmitted using Table N.4.

#### N.4.1.8	Reference Picture Buffer Sub-sampling (RPBS) (Variable length)

A variable length code word that is present only if Enhanced Reference Picture Selection is indicated. RPBS indicates the multiple reference picture buffer sub-sampling mode. If RPBS is set to ”1”, all pictures in the multiple reference picture buffer starting from index 0 ending with index NRA-1 may be utilized for prediction. The picture indexing is kept the same. If RPBS is set to ”0”, the multiple reference picture buffer is sub-sampled. The sub-sampled NRA reference pictures are indexed as 0 through NRA-1 in the order of their selection.

### N.4.1.10	Reference Picture Selection (RPS) (Variable length)

A variable length code word that is present only if Enhanced Reference Picture Selection is indicated. The code table for the picture reference parameter (PR) as given in Table N.1 is used to indicate which picture is to be sampled out of the multiple reference picture buffer. The RPS code word is transmitted as many times as the value of the NRA symbol.

### N.4.1.11	TR Check Indicator (TRCI) (1 bit)

A fixed length code word that is present only if Enhanced Reference Picture Selection is indicated. TRCI indicates whether the following TRC field is present. A value of 0 indicates that TRC is not present.

### N.4.1.12	TR Check (TRC) (12 bit)

A fixed length code word that is present only if Enhanced Reference Picture Selection is indicated and TRCI has a value of 1. When present, TRC is a 12 bit field that is generated by calculating a cyclic redundancy check (CRC) over a set of TR fields. Its purpose is to assure that the encoder and decoder use the same reference frames for prediction when using the picture reference parameter (PR) at the macroblock layer. Though this is not necessary in error free environments, a mismatch between the content of the reference picture buffer at the encoder and decoder may result from transmission errors. In this case it is important to detect the mismatch at the decoder in order to initiate appropriate recovery mechanisms, e.g. requesting a fast INTRA update. Using TRC it is possible to detect the mismatch by comparing the received value with the locally generated CRC.

The CRC is calculated as follows. First the appropriate set of temporal references (TR) has to be determined. If the reference picture buffer is not sub-sampled using RPBS and RPS, then the TRs of the specified NRA frames are used. If the reference picture buffer is sub-sampled, however, the TRs of the NRA sub-sampled frames are used.

Second, a message block is built from the selected set of TRs. The TR values are represented by 10-bit numbers and concatenated to the message block in the order of their position in the reference picture buffer. Assuming that the reference picture buffer is filled completely with M frames and sub-sampling is not used, then the LSB of the TR that corresponds to the picture with buffer index 0 is the first bit of the message block while the MSB of the TR that corresponds to the picture with index M-1 is the last bit of the message block.

Finally, in the third step, the CRC is calculated from the message block. The used generator polynomial is g(x) = x12 + x11 + x3 + x2 + x + 1. The resulting 12-bit CRC is assigned to TRC. For example, assume that sub-sampling is used and that a picture with TR=14 is placed at index 0 in the reference picture buffer and that a second picture with TR=10 is placed at index 1. Then the resulting message block is 00000010100000001110 and TRC is 101001000101.

### N.4.1.13	Reference Picture Buffering Mode (MRPBM) (2 bits)

A fixed length code word that is present only if Enhanced Reference Picture Selectionis indicated. MRPBM specifies the buffering of the currently decoded picture. The multiple reference picture modes are signaled using a two bit code word. The two modes are specified in Table N.1.

Table 4.1.13. Modes of operation for multiple reference picture buffer management.

| Code word | Mode |
| --- | --- |
| ”00” | Sliding Window |
| ”01” | Adaptive Remove/Add Picture |
| ”10” | Reserved for future use |
| ”11” | Reserved for future use |

The Sliding Window mode stands for removing the last picture in case the multiple reference picture buffer is full, and adding the currently decoded picture at position 0 to the multiple reference picture buffer. In case the multiple reference picture buffer is not full, no frame is removed from the buffer. The Adaptive Remove/Add Picture mode stands for a more general version of the sliding window mode. Here any frame can be removed from the multiple reference picture buffer while the currently decoded frame can be inserted at any position into the buffer.

### N.4.1.14	Remove Picture Indication (RPI) (1 bit)

A fixed length code word that is present only if Enhanced Reference Picture Selectionis indicated and if the MRPBM code is set to ”01”. If RPI is set to ”0”, no picture is removed from the memory unless a picture is added and the multiple reference picture memory is full. In that case, the picture with index M-1 is removed from the multiple picture memory. If RPI is set to ”1”, a picture is removed from the multiple picture memory.

### N.4.1.15	Remove Picture Position (RPP) (Variable length)

A variable length code word that is present only if Enhanced Reference Picture Selection is indicated and if RPI is set to ”1”. The code table for the picture reference parameter (PR) as given in Table 3.2.1 is used to indicate which picture is to be removed from the multiple reference picture buffer. After removing the indexed picture from the m’th position in the multiple reference picture buffer, the indexes greater than m are decremented by one.

### N.4.1.16	Add Picture Indication (API) (1 bit)

A fixed length code word that is present only if Enhanced Reference Picture Selection is indicated and if the MRPBM code is ”01”. If API is set to ”0”, the currently decoded picture is not added to the multiple picture memory. If API is set to ”1”, the currently decoded picture is added to the multiple picture memory.

#### N.4.1.17	Macroblock Layer

The macroblock layer syntax is modified if Enhanced Reference Picturee Selection is indicated and if the number of selected reference pictures (NRA value) is greater than one.

If the COD bit is set to "0", indicating that the macroblock is coded, the syntax of the macroblock layer as depicted in Fig. N.4 is modified in that, the MVD, MVD2, MVD3, and MVD4 parts are extended by a picture reference (PR) parameter. Otherwise, the same syntax is used as described in section 5.3 of the H.263 recommendation.

| COD | MCBPC | MODB | CBPB | CBPY | DQUANT | MVD | MVD2 | MVD3 | MVD4 | MVDB | Block Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure N.4: H.263 macroblock syntax layer if COD="0".

| PR | MVDx | MVDy |
| --- | --- | --- |

Figure N.5: Extended motion vector syntax. PR: picture reference parameter, MVDx: x-component of spatial displacement, MVDy: y-component of spatial displacement

If the COD bit is set to "1", meaning that the macroblock is not coded, the syntax of the macroblock is modified in that the COD bit is followed by the picture reference (PR) parameter as depicted in Fig. N.6.

| COD | PR |
| --- | --- |

Figure N.6: H.263 macroblock syntax layer if COD="1".

Variable length codes for the picture reference parameter are given in Table N.1.

Table N.1 Variable length codes for the picture reference parameter.

| Absolute position indicated by picture reference parameter (PR) | number of bits | Codes |
| --- | --- | --- |
| 0 | 1 | 1 |
| “x0”+1 (1:2) | 3 | 0x00 |
| “x1x0”+3 (3:6) | 5 | 0x11x00 |
| “x2x1x0”+7 (7:14) | 7 | 0x21x11x00 |
| “x3x2x1x0”+15 (15:30) | 9 | 0x31x21x11x00 |
| “x4x3x2x1x0”+31 (31:62) | 11 | 0x41x31x21x11x00 |
| “x5x4x3x2x1x0”+63 (63:126) | 13 | 0x51x41x31x21x11x00 |
| “x6x5x4x3x2x1x0”+127 (127:254) | 15 | 0x61x51x41x31x21x11x00 |
| “x7x6x5x4x3x2x1x0”+255 (255:510) | 17 | 0x71x61x51x41x31x21x11x00 |
| “x8x7x6x5x4x3x2x1x0”+511 (511:1022) | 19 | 0x81x71x61x51x41x31x21x11x00 |
| “x9x8x7x6x5x4x3x2x1x0”+1023 (1023:2046) | 21 | 0x91x81x71x61x51x41x31x21x11x00 |
| “x10x9x8x7x6x5x4x3x2x1x0”+2047 (2047:4094) | 23 | 0x101x91x81x71x61x51x41x31x21x11x00 |

N.4.2	Back-Channel Message (BCM) Syntax

The syntax for the back-channel which conveys the acknowledgment/non-acknowledgment messages is illustrated in Figure N.4.  This message is returned from a decoder to an encoder in order to tell whether a forward channel data was correctly decoded or not.

| BT | URF | TR | ELNUMI | ELNUM | BCPM | BSBI | BEPB1 | GN/MBA | BEPB2 | RTR | BSTUF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FIGURE N.4/H.263

Structure of back-channel message (BCM) syntax for NEWPRED

#### N.4.2.1	Back-channel message Type (BT) (2 bits)

Back-channel message type indicates if the corresponding part of the encoded message is correctly decoded or not. Which type of message is required for the encoder is indicated in the picture header of the forward channel.

00: reserved for future use
01: reserved for future use
10: NACK. This indicates the erroneous decoding of the corresponding part of the forward channel data.
11: ACK.    This indicates the correct decoding of the corresponding part of the forward channel data.

#### N.4.2.2	Unreliable Flag (URF) (1 bit)

The Unreliable Flag is set to 1 when a reliable value for TR or GN/MBA is not available to the decoder. (When BT is NACK, a reliable TR may not be available at the decoder.)

0: reliable
1: unreliable

#### N.4.2.3	Temporal Reference (TR) (10 bits)

Temporal reference contains the TR information of the video picture segment for which the ACK/NACK is indicated in the back-channel message.  Note: The meaning of the term “video picture segment” as used herein is defined in Annex R.  If a custom picture clock frequency was not in use for the reference picture, the two MSBs of TR are zero and the LSBs contain the eight bit TR found in the picture header of the reference picture.  If a custom picture clock frequency was in use for the reference picture, TR is a ten bit number consisting of the concatenation of ETR and TR from the reference picture header.

#### N.4.2.4	Enhancement Layer Number Indication (ELNUMI) (1 bit)

The enhancement layer number indication is “0” unless the optional Temporal, SNR and Spatial Scalability mode (Annex O) is used in the forward channel data and some enhancement layers of the forward channel are combined on one logical channel and the back-channel message refers to an enhancement layer (rather than the base layer), in which case the enhancement layer number indication shall be “1”.

#### N.4.2.5	Enhancement Layer Number (ELNUM) (4 bits)

Enhancement layer number is present if and only if ELNUMI is “1”, in which case it contains the layer number of the enhancement layer referred to in the back-channel message.

#### N.4.2.6	BCPM (1 bit)

BCPM is “0” unless the CPM mode is used in the forward channel data, in which case it is “1”. If BCPM is “1”, this indicates that BSBI is present.

#### N.4.2.7	Back-channel Sub Bitstream Indicator (BSBI) (2 bits)

A fixed length codeword of 2 bits that is only present if BCPM is “1”. The BSBI is the natural binary representation of the appropriate Sub-Bitstream number in the forward channel data for which the ACK/NACK message is indicated in the back-channel message as described in section 5.2.4 and Annex C.

#### N.4.2.8	Back-channel Emulation Prevention Bit 1 (BEPB1) (1 bit)

A field which is present if and only if the videomux mode is in use. This field is always set to “1” to prevent a start code emulation.

#### N.4.2.9	GOB Number/Macroblock Address (GN/MBA) (5/6/7/9/11/12/13/14 bits):

A GOB number or Macroblock address is present in this field.  If the optional Slice Structured mode (see Annex K) is not in use, this field contains the GOB number of the beginning of the video picture segment for which the NACK/ACK message is indicated in the back-channel message. If the optional Slice Structured mode is in use, this field contains the macroblock address of the beginning of the slice for which the NACK/ACK message is indicated in the back-channel message.  The length of this field is the length specified elsewhere in this Recommendation for GN or MBA.

#### N.4.2.10	Back-channel Emulation Prevention Bit 2 (BEPB2) (1 bit)

A field which is present if and only if the videomux mode is in use. This field is always set to “1” to prevent a start code emulation.

#### N.4.2.11	Requested Temporal Reference (RTR) (10 bits)

Requested temporal reference is present only if BT is NACK. RTR indicates the requested temporal reference of the GOB or slice associated with the NACK. Typically it is the TR of the last correctly decoded video picture segment of the corresponding position at the decoder.  If a custom picture clock frequency was not in use for the requested reference picture, the two MSBs of RTR are zero and the LSBs contain the eight bit TR found in the picture header of the requested reference picture.  If a custom picture clock frequency was in use for the requested reference picture, RTR is a ten bit number consisting of the concatenation of ETR and TR from the requested reference picture header.

#### N.4.2.12	Stuffing (BSTUF) (Variable Length)

This field is present if and only if the separate logical channel mode is in use and the back channel message is the last in an external frame.  BSTUF consists of a codeword of variable length consisting of zero or more bits of value “0”.  This field is only present at the end of an external frame.

## N.5	Decoder Process

The decoder of this mode may need an additional number of picture memories to store the correctly decoded video signals and their Temporal Reference (TR) information. The decoder uses the stored picture for which the TR is TRP as the reference picture for inter-frame decoding instead of the last decoded picture, if the TRP field exists in the forward channel data. When the picture for which the TR is TRP is not available at the decoder, the decoder may send a forced INTRA update signal to the encoder by external means (for example Recommendation H.245).  Unless a different frame storage policy is negotiated by external means, correctly decoded video picture segments shall be stored into memory for use as later reference pictures on a first-in, first-out basis as shown in Figure N.1 (except for B pictures, which are not used as reference pictures), and video picture segments which are detected as having been incorrectly decoded should not replace correctly decoded ones in this memory area.

An acknowledgment message (ACK) and a non-acknowledgment message (NACK) are defined as back-channel messages. An ACK may be returned when the decoder decodes a video picture segment successfully. NACKs may be returned when the decoder fails to decode a video picture segment, and may continue to be returned until the decoder gets the expected forward channel data which includes the requested TRP or an INTRA update. Which types of message shall be sent is indicated in the RPSMF field of the picture header of the forward channel data.

In a usage scenario known as “Video Redundancy Coding”, the Reference Picture Selection mode may be used by some encoders in a manner in which more than one representation is sent for the pictured scene at the same temporal instant (usually using different reference pictures).  In such a case in which the Reference Picture Selection mode is in use and in which adjacent pictures in the bitstream have the same temporal reference, the decoder shall regard this occurrence as an indication that redundant copies have been sent of approximately the same pictured scene content, and shall decode and use the first such received picture while discarding the subsequent redundant picture(s).

If Enhanced Reference Picture Selection is indicated, the decoder stores the reference pictures for inter-frame decoding in a buffer-based scheme. The buffering may follow a first-in, first-out mode. Alternatively, the buffering may follow a customized adaptive scheme specified by the encoder in the forward channel. When any reference picture is not available at the decoder or the CRC check for the TRs in the multi-frame buffer shows an error, the decoder may send a forced INTRA update signal to the encoder by external means (for example Recommendation H.245). The decoder replicates the multi-frame buffer of the encoder according to the specified buffering mode. The TRs are attached to the buffered frames. The buffering scheme is operated also when partially erroneous frames are decoded according to the specified buffer mode information.