| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Seventh Meeting: Monterey, 16-19 February, 1999 | Document Q15-G-12 Filename: q15g12.doc Generated: 5 Feb ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Max Luttrell, John D. Villasenor Department of Electrical Engineering University of California Los Angeles, CA 90095, USA J.H. Park D.S. Park Samsung Electronics Company South Korea | Tel: Fax: Email: | 1 310 825-0228 1 310 825-7928 villa@icsl.ucla.edu |
| Title: | Data Partitioning Simulation Results |  |  |
| Purpose: | Information |  |  |

_____________________________

We present simulation results for the data partitioning based syntax described in document Q15-G-13.

We have run simulations for the foreman, silent, hall, container, glasgow, and news sequences according to the common conditions outlined in document Q15-E-50 and found performance gains from several tenths of a dB to several dB depending on the video sequence and error conditions.

We use the TCON based algorithm for error concealment .

From our anchor validation work with UBC, we have discovered that increasing the INTRA refresh frequency from once in 132 frames (the max specified in section 4.4/H.263) to once in 5 frames improves performance for both the anchor and the data partitioning mode, and narrows the gap between them.  However, even when an INTRA refresh of 5 is used, data partitioning still results in better performance. In order to ensure that the results we present here are conservative, all of the simulations used to generate the tables in this document used an INTRA refresh of 5.

Further, results from our contribution in Seoul (Q15-F-29) show that the data partitioning mode results in better performance in cases when a multiplexer, and the resulting CRC information, is not available.

One interesting result from the simulations is the contrast in number of MUX packets produced by our proposed method relative to the anchor mode.  The anchor (GOB) coder produces 9 packets per frame and therefore has 9*4000=36,000 packets for one run of 4000 coded frames.  The proposed method was producing longer packets, and therefore fewer packets, leading to less mux overhead.  In order to take advantage of this extra bandwidth, we have both reduced packet size and increased target video bitrate in order to reach the 64kbps available.

As can be seen from Tables 1 and 2, we have an average improvement of 1-2 dB in the DECT error pattern and an average improvement of several tenths of a dB in the WCDMA error pattern.  In addition, data partitioning results in significant improvements in the number of dropped frames, particularly for the WCDMA patterns.

Table 1.  The video bitstream and payload overhead bytes size information

| Seq. | DECT 32k Channel |  |  |  | WCDMA 64k Channel |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Video Packets | Total Video Bytes | Payload And Overhead Bytes For Video | difference | Total Video Packets | Total Video Bytes | Payload And Overhead Bytes For Video | difference |
| Container |  |  |  |  |  |  |  |  |
| Anchor | 36000 | 1,213,308 | 1465264 |  | 36000 | 2,400,753 | 2656393 |  |
| DP mode | 19647 | 1,226,724 | 1364245 |  | 29232 | 2,400,784 | 2605436 |  |
| Hall |  |  |  |  |  |  |  |  |
| Anchor | 36000 | 1,201,951 | 1453937 |  | 36000 | 2,401,701 | 2653701 |  |
| DP mode | 18819 | 1,204,038 | 1335741 |  | 27182 | 2,401,970 | 2592310 |  |
| Silent |  |  |  |  |  |  |  |  |
| Anchor | 36000 | 847,854 | 1099802 |  | 36000 | 1,601,423 | 1853323 |  |
| DP mode | 16514 | 870,600 | 986191 |  | 29623 | 1,601,796 | 1809150 |  |
| News |  |  |  |  |  |  |  |  |
| Anchor | 36000 | 1,227,030 | 1478988 |  | 36000 | 2,401,830 | 2653759 |  |
| DP mode | 20216 | 1,245,961 | 1387446 |  | 29803 | 2,402,185 | 2610783 |  |

Table 2.  Simulation results for DECT 32kbps channel, INTRA refresh=5

|  | 32kbps |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Error Free |  |  |  | DECT 1.4 km/h 10dB |  |  |  |  | DECT 1.4 km/h 20dB |  |  |  |
| Container | Y | Cb | Cr |  | Y | Cb | Cr | Drop |  | Y | Cb | Cr | Drop |
| UCLA Anchor | 29.13 | 38.42 | 37.55 |  | 24.46 | 36.83 | 35.81 | 620 |  | 28.05 | 38.02 | 37.14 | 19 |
| UBC Anchor | 29.15 | 38.44 | 37.57 |  | 24.52 | 36.85 | 35.90 | 584 |  | 28.06 | 37.88 | 37.04 | 10 |
| Slice Mode | 29.98 | 38.66 | 37.80 |  | 24.93 | 36.61 | 35.52 | 753 |  | 29.02 | 38.35 | 37.44 | 0 |
| DP mode | 29.84 | 38.64 | 37.78 |  | 25.59 | 37.60 | 36.69 | 190 |  | 29.03 | 38.39 | 37.56 | 0 |
| hall |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 29.94 | 37.43 | 39.69 |  | 24.00 | 33.50 | 35.20 | 624 |  | 28.94 | 36.87 | 39.07 | 16 |
| UBC Anchor | 29.70 | 37.33 | 39.61 |  | 24.78 | 34.30 | 36.14 | 594 |  | 28.82 | 36.78 | 38.97 | 15 |
| Slice Mode | 30.11 | 37.41 | 39.70 |  | 24.04 | 32.87 | 33.59 | 791 |  | 29.03 | 36.63 | 38.85 | 18 |
| DP mode | 30.07 | 37.46 | 39.74 |  | 25.74 | 36.36 | 39.08 | 299 |  | 29.36 | 37.30 | 39.61 | 15 |
| silent |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 28.04 | 35.80 | 37.36 |  | 23.81 | 33.78 | 35.07 | 689 |  | 27.09 | 35.63 | 37.23 | 17 |
| UBC Anchor | 27.73 | 35.54 | 37.20 |  | 23.60 | 32.89 | 34.09 | 694 |  | 26.76 | 34.52 | 36.00 | 15 |
| Slice Mode | 28.18 | 35.66 | 37.22 |  | 23.93 | 33.78 | 35.14 | 917 |  | 27.02 | 34.69 | 36.07 | 37 |
| DP mode | 28.02 | 35.63 | 37.27 |  | 24.21 | 33.98 | 35.50 | 225 |  | 27.52 | 35.51 | 37.15 | 14 |
| news |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 27.43 | 35.95 | 37.10 |  | 20.36 | 29.22 | 32.72 | 641 |  | 25.47 | 34.44 | 36.34 | 18 |
| UBC Anchor | 27.22 | 35.84 | 37.15 |  | 21.05 | 28.63 | 32.33 | 632 |  | 25.96 | 34.75 | 36.40 | 13 |
| Slice Mode | 27.51 | 35.89 | 37.03 |  | 21.44 | 28.23 | 31.79 | 675 |  | 26.61 | 34.95 | 36.40 | 22 |
| DP mode | 27.47 | 35.99 | 37.22 |  | 22.81 | 30.10 | 34.16 | 192 |  | 26.68 | 35.13 | 36.85 | 0 |

Table 3.  The simulation result for WCDMA 64kbps channel, INTRA refresh=5

|  | 64kbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Error Free |  |  |  |  | WCDMA-64kb-211hz-3 |  |  |  |  |  |  | WCDMA-64kb-211hz-4 |  |  |  |  |  |  |
| Container | Y | Cb | Cr |  |  | Y |  | Cb |  | Cr | Drop |  | Y | Cb |  |  | Cr |  | Drop |
| UCLA Anchor | 32.80 | 39.86 |  | 39.32 |  | 30.74 | 39.46 |  | 38.78 |  | 9 |  | 32.46 |  | 39.81 | 39.24 |  | 0 |  |
| UBC Anchor | 32.34 | 39.73 |  | 39.15 |  | 30.98 | 39.38 |  | 38.73 |  | 0 |  | 32.13 |  | 39.70 | 39.10 |  | 0 |  |
| Slice Mode | 32.97 | 40.08 |  | 39.50 |  | 31.52 | 39.69 |  | 39.02 |  | 0 |  | 32.72 |  | 40.03 | 39.44 |  | 0 |  |
| DP mode | 32.76 | 39.88 |  | 39.29 |  | 31.85 | 39.75 |  | 39.13 |  | 0 |  | 32.65 |  | 39.86 | 39.27 |  | 0 |  |
| Hall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 32.40 | 38.64 |  | 40.70 |  | 31.08 | 38.42 |  | 40.58 |  | 21 |  | 32.22 |  | 38.62 | 40.69 |  | 2 |  |
| UBC Anchor | 32.33 | 38.60 |  | 40.68 |  | 31.16 | 38.23 |  | 40.30 |  | 0 |  | 32.18 |  | 38.58 | 40.67 |  | 0 |  |
| Slice Mode | 32.50 | 38.64 |  | 40.71 |  | 31.19 | 38.12 |  | 40.10 |  | 0 |  | 32.27 |  | 38.58 | 40.65 |  | 0 |  |
| DP mode | 32.42 | 38.64 |  | 40.73 |  | 31.40 | 38.28 |  | 40.36 |  | 0 |  | 32.29 |  | 38.62 | 40.71 |  | 0 |  |
| Silent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 30.18 | 36.78 |  | 38.23 |  | 29.55 | 36.67 |  | 38.14 |  | 0 |  | 30.08 |  | 36.76 | 38.23 |  | 0 |  |
| UBC Anchor | 29.89 | 36.63 |  | 38.09 |  | 29.17 | 36.34 |  | 37.76 |  | 0 |  | 29.79 |  | 36.61 | 38.07 |  | 0 |  |
| Slice Mode | 30.31 | 36.82 |  | 38.22 |  | 29.23 | 36.29 |  | 37.61 |  | 0 |  | 30.15 |  | 36.80 | 38.19 |  | 0 |  |
| DP mode | 30.07 | 36.73 |  | 38.16 |  | 29.65 | 36.65 |  | 38.07 |  | 0 |  | 29.89 |  | 36.71 | 38.15 |  | 0 |  |
| News |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 29.72 | 37.29 |  | 38.26 |  | 27.96 | 35.50 |  | 37.57 |  | 3 |  | 29.40 |  | 36.97 | 38.15 |  | 0 |  |
| UBC Anchor | 29.51 | 37.14 |  | 38.16 |  | 27.91 | 35.68 |  | 37.23 |  | 0 |  | 29.31 |  | 37.00 | 38.10 |  | 0 |  |
| Slice Mode | 29.78 | 37.23 |  | 38.15 |  | 27.50 | 35.21 |  | 36.67 |  | 0 |  | 29.10 |  | 36.81 | 37.91 |  | 0 |  |
| DP mode | 29.68 | 37.26 |  | 38.21 |  | 28.53 | 36.35 |  | 37.80 |  | 0 |  | 29.42 |  | 37.16 | 38.17 |  | 0 |  |
| Foreman |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UCLA Anchor | 25.40 | 36.16 |  | 36.61 |  | 23.28 | 35.60 |  | 35.82 |  | 46 |  | 25.02 |  | 36.08 | 36.50 |  | 4 |  |
| UBC Anchor | 25.36 | 36.96 |  | 36.96 |  | 23.76 | 36.48 |  | 36.37 |  | 0 |  | 25.12 |  | 36.90 | 36.89 |  | 0 |  |
| Slice Mode | 25.37 | 36.06 |  | 36.51 |  | 23.42 | 35.42 |  | 35.66 |  | 0 |  | 25.05 |  | 35.99 | 36.41 |  | 0 |  |
| DP mode | 25.32 | 36.11 |  | 36.55 |  | 23.84 | 35.64 |  | 35.95 |  | 0 |  | 25.17 |  | 36.04 | 36.47 |  | 0 |  |