| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Seventh Meeting: Monterey, 16-19 February, 1999 | Document Q15-G-16 rev 1 Filename: q15g16r1.doc Generated: 11 Feb ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |  |
| --- | --- | --- | --- | --- |
| Source: | Test model editors Stephan Wenger TU Berlin, Sekr. FR 6-3 Franklinstr. 28-29 D-10587 Berlin, Germany Signal Processing and Multimedia Group Guy Côté, Michael Gallant, Faouzi Kossentini Department of ECE University of British Columbia 2356 Main Mall Vancouver BC V6T 1Z4 Canada | Tel: Fax: Email: | +49-172-300 0813 +49-30-314-25156 stewe@cs.tu-berlin.de {guyc,mikeg,faouzi}@ece.ubc.ca |  |
| Title: | Test model 11 |  |  |  |
| Purpose: | Information |  |  |  |

_____________________________

Contents

1	Introduction	1

2	Structure	2

3	Application scenarios	3

3.1	Variable bit rate, error-free environment	3

3.1.1	Low complexity model	3

3.1.2	High complexity model	3

3.2	Fixed bit rate, error-free environment (H.320 or H.324)	4

3.2.1	Low complexity model	4

3.2.2	High complexity model	4

3.3	Fixed bit rate, packet-lossy environment (H.323)	4

3.3.1	Packetization and depacketization	5

3.3.2	Low complexity model	7

3.3.3	High complexity model	7

3.4	Fixed bit rate, highly bit-error prone environment (H.324/M)	7

3.4.1	Low complexity model	8

4	Common algorithms	9

4.1	Motion estimation	9

4.1.1	Sum of absolute difference distortion measure	9

4.1.2	Low complexity, fast-search	10

4.1.3	Medium complexity, full search	11

4.1.4.	High complexity, rate-distortion optimized full search	11

4.1.4	Other motion vector search related issues	12

4.2	Quantization	12

4.2.1	Quantization for INTER Coefficients:	13

4.2.2	Quantization for INTRA non-DC coefficients when not in Advanced Intra Coding mode	13

4.2.3	Quantization for INTRA DC coefficients when not in Advanced Intra Coding mode	13

4.2.4	Quantization for INTRA coefficients when in Advanced Intra Coding mode	14

4.3	Alternative INTER VLC mode (Annex S)	14

4.4	Advanced INTRA Coding mode (Annex I)	14

4.5	Modified Quantization mode (Annex T)	16

5	Algorithms used for individual Application Scenarios	17

5.1	Mode decision	17

5.1.1	INTRA macroblock refresh and update pattern	17

5.1.2	Low complexity mode decision for error free environments	18

5.1.3	High complexity mode decision for error free environments	20

5.1.4	Low complexity mode decision for error-prone environments	20

5.1.5	High complexity mode decision for error-prone environments	21

5.2	Rate control	21

5.2.1	Frame level rate control	21

5.2.2	Macroblock level rate control	22

5.2.3	High complexity models and rate control	23

6	Encoder Pre-Processing	25

7	Decoder Post-Processing	26

7.1	De-ringing post-filter	26

7.2	De-blocking post-filter	27

7.3	Error detection	29

7.4	Error concealment	30

8	Information capture section	31

8.1	PB-frames mode (Annex G and M)	31

8.1.1	Improved PB-frames motion estimation and mode decision	31

8.2	Scalabilitymode (Annex O)	31

8.2.1	True B-frame motion estimation and mode decision	31

8.2.2	EI/EP frame motion estimation and mode decision	32

8.2.3	Rate Control for P and B Frames	32

8.3	Reduced-Resolution Update mode (Annex Q)	34

8.3.1	Motion estimation and mode selection	34

8.3.2	Motion estimation in baseline mode (no options)	34

8.3.3	Motion estimation in advanced prediction mode (Annex F)	35

8.3.4	Motion estimation in the unrestricted motion vector mode (Annex D)	36

8.3.5	Down-sampling of the prediction error	36

8.3.6	Transform and Quantization	37

8.3.7	Switching	37

8.3.8	Restriction of DCT coefficients in switching from reduced-resolution to normal-resolution	38

8.4	Fast Search Using Mathematical Inequalities	39

8.4.1	Search Order	39

8.4.2	Multiple Triangle Inequalities	40

8.5	Control of Encoding Frame Rate	40

8.5.1	Adaptive control of the Encoding Frame Rate	41

9	Future extensions	45

9.1	RD-optimized mode decision and quantization working under bit rate constraints	45

9.2	Frame rate calculation	45

9.3	Bi-directional communication in error prone environments	45

10	References	46

11	Appendix: Common Simulation Conditions	48

List of Contributors

Barry Andrews

8x8 Inc.

Santa Clara, CA, USA

andrews@8x8.com

Gisle Bjontegaard

Telenor International

Telenor Satellite Services AS

P.O. Box 6914 St. Olavs plass

N-0130 Oslo, Norway

Tel: +47 23 13 83 81

Fax: +47 22 77 79 80

gisle.bjontegaard@oslo.satellite.telenor.no

Guy Côté

Department of ECE

University of BC

2356 Main Mall

Vancouver BC V6T 1Z4

Canada

guyc@ece.ubc.ca

Michael Gallant

Department of ECE

University of BC

2356 Main Mall

Vancouver BC V6T 1Z4

Canada

mikeg@ece.ubc.ca

Thomas Gardos

Intel Corporation

5200 NE Elam Young Parkway

M/S JF2-78

Hillsboro, OR 97214 USA

Thomas.R.Gardos@intel.com

Karl Lillevold

Intel Corporation

5200 NE Elam Young Parkway

M/S JF2-78

Hillsboro, OR 97214 USA

Karl.Lillevold@intel.com

Peter List

DeTeBerkom

Tel.: +49 61 5183 3878

Fax.: +49 61 5138 4842

p.list@berkom.de

Akira Nakagawa

Fujitsu Lab. Ltd.

Tel.: +81 447 5426

Fax.: +81 447 5423

anaka@flab.fujitsu.co.jp

Toshihisa Nakai

OKI Electric Ind. Co., Ltd.

Tel: +81 6 949 5105

Fax: +81 6 949 5108

nakai@kansai.oki.co.jp

Jordi Ribas

Sharp Laboratories of America, Inc.

5750 NW Pacific Rim Blvd.

Camas, Washington 98607 USA

Tel: +1 360 817 8487

Fax: +1 360 817 8436

jordi@sharplabs.com

Gary Sullivan

Microsoft Corporation

One Microsoft Way

Redmond, WA 98053 USA

Tel: +1 425 703 5308

Fax: +1 425 936 7329

garysull@microsoft.com

Stephan Wenger

Technische Universitaet Berlin

Sekr. FR 6-3

Franklinstr. 28-29

D-10587 Berlin

Germany

stewe@cs.tu-berlin.de

Thomas Wiegand

University of Erlangen-Nürnberg

Germany

wiegand@nt.e-technik.uni-erlangen.de

# Introduction

This document describes the Test Model Near-Term (TMN) for the ITU-T Recommendation H.263, Version 2. The purpose of the Test Model is to help manufacturers understand how to use the syntax and tools of the Recommendation, and to show implementation examples.

Recommendation H.263, Version 2 (H.263 from hereon), was ratified in January 1998 by SG16, based on the working draft version 20 submitted in September 1997 by the video experts group.  The official document can be obtained directly from ITU-T.

H.263 defines a specific bitstream syntax and a corresponding video decoder such that video terminals from different manufacturers can inter-operate.   Encoder design is beyond the normative scope of the Recommendation and up to the manufacturer.  However, in developing the Recommendation, preferred encoding methods emerged. These methods produce good results in terms of video quality and compression efficiency, at complexity levels suitable for operation on general and special purpose processors, for different environments (e.g. circuit-switched or packet-switch networks). Moreover, the performance levels obtained by these methods serves as a benchmark for research and development of future ITU-T video coding Recommendations.  This document specifies encoder operations, as well as decoder operations beyond the normative text of H.263, for example in case of error-prone transmission.

The coding method employed in H.263 is a hybrid DPCM/transform algorithm, consisting of motion estimation and compensation, DCT, quantization, run-length coding, VLC and FLC coding.  Additionally, there are numerous optional modes of operation permitted by H.263, as defined by annexes in the Recommendation. This document assumes some familiarity with H.263, its optional modes, and video coding in general.  A tutorial on H.263 and its optional modes is available in [Q15-D-58].

All documents referenced in this Test Model fall into one of the following categories:

- ITU Recommendations: These will be referenced by their well-known shortcuts, e.g. H.323 or BT.601.   In certain cases the version of the recommendation is significant and will be included in an appropriate form, such as publication year or version number.  ITU-T recommendations can be obtained directly from the ITU-T. See http://www.itu.int for details.

- ITU-T SG16 contributions: These will be referenced by their shortcut name, e.g. [Q15-D-58].  For most of the contributions referenced in this document, hyperlinks are used to point to the documents on the PictureTel ftp-site at ftp://standard.pictel.com.

- Other academic publications: These will be referenced in IEEE style.  Drafts of such publications were often provided by proponents in the form of ITU-T SG16 contributions. In such cases, they are referenced as ITU-T SG16 contributions.

# Structure

This Test Model document describes encoding methods through functional units typically employed by an encoder. In addition, this document provides information on the application of the various methods for specific scenarios, referred to as Application Scenarios.  These are:

- Variable bit rate with fixed quantizer in an error free environment (commonly used for video coding research purposes),

- Fixed bit rate in a practically error free environment (H.320/H.324),

- Fixed bit rate in a packet lossy environment (H.323), and

- Fixed bit rate in a highly bit error prone environment (H.324/M).

The Application Scenarios are discussed in Section 3, which makes reference to mechanisms described in later sections. Section 3 also defines the simulation environments assumed in this Test Model.

Mechanisms common to all Application Scenarios are described in Section 4.  These include motion vector (MV) search, quantization, and optional modes of H.263 that are common to all Application Scenarios.  Mechanisms relevant to specific Application Scenarios are discussed in Section 5.  Combined with the Recommendation itself, these sections define a framework for an H.263 codec that performs reasonably well for the various Applications Scenarios.

Section 6 discusses pre-processing of video data. Section 7 discusses decoder post-processing. Section 8 captures all information that was deemed valuable and adopted for inclusion into the Test Model, but does not yet fit within any of the Application Scenarios. Some of the adopted proposals are only summarized and referenced, whereas others are explicitly detailed, depending on the decisions made by Q.15 experts. Section 9 contains an overview of possible research directions or research in progress deemed to be valuable to future versions of the Test Model.

# Application scenarios

This section discusses relevant Application Scenarios. Its purpose is to provide an overview of useful mechanisms for each scenario and to outline the corresponding simulation environment where appropriate.    Description of the environment outside the video data path is kept to a minimum, although references are provided for convenience.

## Variable bit rate, error-free environment

The variable bit rate scenario employs a constant quantizer value for all pictures and picture regions to produce a constant level of quality.  This scenario is useful for video coding research and related standardization work. For example, it provides a framework for assessing the objective and subjective quality of bitstreams generated by new, coding efficiency-oriented proposals to anchor bitstreams (See Appendix: Common Simulation Conditions).  There are two different models for this scenario: the low complexity model and the high complexity model. These models are described in the next two sections.

### Low complexity model

The low complexity model employs the optional modes defined in level 1 of the preferred mode combinations of Appendix 2 of H.263, which are advanced INTRA coding mode (Annex I), deblocking filter mode (Annex J), modified quantization mode (Annex T), and supplementary enhancement information mode (Annex L).  None of the enhanced capabilities of the supplementary enhancement information mode are currently employed in the Test Model, as they address features that might be important for certain product designs, but are of limited importance for video coding research. Motion estimation and mode decision are performed using the low complexity motion vector search and mode decision methods.

Details can be found in Section  4.1.2 (low complexity motion vector search), 5.1.2 (low complexity mode decision), 4.4 (advanced INTRA coding mode), and 4.5 (modified quantization mode).

### High complexity model

The high complexity model is designed to provide the best possible reconstructed picture quality, at the expense of increased computational complexity.  It employs a subset of the optional modes defined in level 3 of the preferred mode combinations of Appendix 2 of the H.263 Recommendation.  In particular, all the H.263 optional modes of the low complexity model described above, plus unrestricted motion vectors (Annex D, with limited extended range), advanced prediction mode (Annex F) and alternative INTER VLC mode (Annex S), are employed.  Motion estimation and mode decision are performed using the high complexity motion vector search and mode decision methods.

Details can be found in the Sections 4.1.4 (high complexity motion vector search), 5.1.3 (high complexity mode decision), 4.4 (advanced INTRA coding mode), 4.3 (alternative INTER VLC mode) and 4.5 (modified quantization mode).

## Fixed bit rate, error-free environment (H.320 or H.324)

This application is characterized by the need to achieve a fixed target bit rate with reasonably low delay.  The transport mechanism is bit-oriented, and provides an environment that can be considered error-free for all practical purposes.  This scenario is closer to a practical application, thus the complexity versus quality tradeoff is an important issue. In order to achieve a target bit rate, the quantizer step size is no longer fixed, but determined by the rate control algorithm.  Furthermore, although a target frame-rate is generally specified, the rate control algorithm is free to drop individual source frames if the bit-rate budget is exceeded.

### Low complexity model

The low complexity model includes all the mechanism described in Section 3.1.1, with the addition of rate control to achieve the target bit rate..  In order to achieve a target bit rate, the quantizer step size is no longer fixed, but determined by the rate control algorithm.

Details can be found in Sections 4.1.2 (low complexity motion vector search), 5.1.2 (low complexity mode decision), 4.4 (advanced INTRA coding mode), 4.5 (modified quantization mode), and 5.2 (rate control).

### High complexity model

The high complexity model includes all the mechanisms described in Section 3.1.2, with the addition of rate control to achieve the target bit rate. The use of rate control with the high complexity motion estimation and mode decision algorithms requires some simplifications, as described in Section 5.2.3.

## Fixed bit rate, packet-lossy environment (H.323)

This Application Scenario is characterized by the need to achieve a fixed target bit rate, and a packetization method for transport.  In H.323 systems, an RTP-based packet oriented transport is used. The main characteristics of such a transport are as follows:

- variable packet sizes as determined by the sender, in the neighborhood of 1 Kbyte, to reduce packetization overhead and to match the Internet maximum transfer unit (MTU) size, and

- significant packet loss rates.

Note that long-term averages of packet loss rates are available to the encoder from mandatory RTCP receiver reports, which are part of RTP.  Also, since RTP and the associated underlying protocol layers ensure that packets are delivered in the correct sequence and that they are bit error free, the only source of error is packet loss.

It is well-known that bi-directional communication employing back-channel messages can greatly improve the reproduced picture quality (see Section 9.3). However, bi-directional communication is often not feasible due to the possible multicast nature of the transport and application, and due to the possible transmission delay constraints of the application.  Also the complexity of simulating a truly bi-directional environment is non-trivial. For these reasons, the Test Model currently supports uni-directional communication only.

For practical reasons, the Test Model requires simplifications to model transport mechanisms such that they can be implemented in network simulation software.  First, a fixed bit-rate rate control algorithm is employed, to simplify the interaction between source rate control and transport mechanisms..  In real systems, while RTP’s buffer management may smooth short-term variations in transmission rates, the target bit rate and the receiver buffer size are adjusted periodically based on factors such as the average packet loss rate and billing constraints. Second, constant short-term average packet loss rates are assumed, to simplify the interaction of error resilience and transport mechanisms.  In real systems, error resilience support should be adaptive, e.g. based on windowed average packet loss rates.

The remainder of this section discusses packetization and depacketization issues and the application of video coding tools for this scenario. There are two different models, corresponding to low and high complexity. This provides a framework for assessing the objective and subjective quality of bitstreams generated by new, error resilience-oriented proposals to known, anchor, bitstreams (See Appendix: Common Simulation Conditions).

### Packetization and depacketization

This section describes a packetization/depacketization scheme using RFC2429 as the payload format for RTP, that has been demonstrated to perform well for the environment assumed in this scenario.

The use of an encoding mechanism to reduce temporal error propagation, which is inevitable in a packet-lossy environment, is assumed. One such mechanism, described in Section 5.1.1, is judicious use of INTRA coded macroblocks. The use of slice structured mode (Annex K), where the slices are of the same size and shape as a GOB, is also assumed.  In this scenario, slices are used instead of GOBs because Annex K permits a bitstream structure whereby the  slices need not appear in the regular scan-order.  The packetization scheme depends on such an arrangement.  In this simple case, the use of slices instead of GOBs has minor implications beyond the syntactic level. These implications are related to prediction restrictions across slices for the remote motion vectors in the Advanced Prediction mode (Annex F).

The packetization scheme is based on interleaving even and odd numbered GOB-shaped slices, and arises from two design considerations. First, since the packetization overhead for the IP/UDP/RTP headers on the Internet is approximately 40 bytes per packet, reasonably big packets have to be used.  Second, to allow for effective error concealment, as described in Section 7.4, consecutive slices should not be placed in the same packet. This results in two packets per picture and allows for reasonable concealment of missing macroblocks if only one of the two packets is lost.  This method may be extended to more than two packets per picture if the coded picture size is larger than 2800 bytes, where a maximum payload size of 1400 bytes per packet and an MTU size of 1500 bytes are assumed.  Note that while the loss of the contents of the picture header can seriously degrade decoded picture quality, this can be concealed by assuming that the contents of the picture header are unchanged, except for the temporal reference field, which can be recovered from the time reference included in the RTP header.  When the video encoder changes picture header content, an RFC2429 mechanism whereby redundant copies of the picture header can be included into the payload header of each packet is employed to allow for partial decoding of a picture (and subsequent concealment) on receipt of a single packet.

A typical example of this packetization scheme for a video bitstream coded at 50 kbps and 10 frames per second at QCIF resolution is presented in Figure 1.  The constant packetization overhead (consisting of the IP/UDP/RTP headers, 40 bytes per packet in total) is 80 bytes per packetized picture.

Figure 1. An example of two packets of a picture using the interleaved packetization scheme

The 2 bytes minimum header defined in RFC2429 does not contribute to the overhead, because this header replaces the 16 bit Picture Start Code or Slice Start Code that proceeds each picture/slice in H.263.

- For the above packetization scheme, in a packet-lossy environment, four different situations can occur during depacketization.  In the case that both packets are received, slices are reordered and decoding is straightforward. In the case that only the first packet is received, the available data is decoded and the missing slices are detected and concealed as described in Section 7.4. In the case that only the second packet is the available data, including the redundant picture header, is decoded and missing slices are detected and concealed as described in Section 7.4. If both packets are lost, no data is available for the decoder and the entire frame must be concealed by re-displaying the previous frame..

### Low complexity model

The low complexity model uses the same encoder mechanisms as described in Section 3.2.1 with two additions that improve error resilience.  GOB-shaped slices are employed, with slice headers inserted at the begin of each slice. These serve as synchronization markers and  to reset spatial predictive coding, such as motion vector coding and inter-coefficient coding, as defined in Annex K of H.263.  Also, the rate at which macroblocks are forced to be coded in INTRA mode (refreshed) is varied with the inverse of the average packet loss rate.  For example, if the average packet loss rate is 10%, every coded macroblock should be coded in INTRA mode every 10 coded frames. This is implemented via the INTRA_MB_Refresh_Rate of the INTRA macroblock refresh mechanism, as described in Section 5.1.1.

### High complexity model

The high complexity uses the packetization, error concealment and GOB-shaped slices described above and an extension of the high complexity mode described in Section 3.1.2, whereby the high complexity mode decision of Section 5.1.3 is replaced by the high complexity mode decision of Section 5.1.5. This mode decision considers the error-prone nature of the network, the packetization process, the packet loss rate, and the error concealment employed by the decoder to select the macroblock coding mode. Details can be found in Section 5.1.5 (high complexity mode decision for error-prone environments).

## Fixed bit rate, highly bit-error prone environment (H.324/M)

This Application Scenario is characterized by the need to achieve a fixed target bit rate in a highly bit-error prone environment. In such environments H.223, the transport protocol employed by the corresponding H.324/M system, is primarily optimized for low delay operation. H.223 does not provide error correction mechanisms for the payload, with the exception of optional re-transmission algorithms. Therefore, the video decoder must be able to detect and handle bit errors.

For practical reasons, several assumptions and simplifications regarding the transport simulation are necessary:

- Framed mode of H.223 is employed, with AL3 SDUs for H.263 data (allowing for the 16 bit CRC to detect errors).

- H.223 Annex B is used.

- Uni-directional communication is assumed, i.e. no re-transmission algorithms or back-channel mechanisms are used, due to the stringent delay constraints..

Currently, there is only a low-complexity model defined for this scenario.  A high complexity model, similar to the one described in Section 3.3.3, would be possible. However as there is no RTCP feedback mechanisms (or a similar tool) to transmit the bit error rate to the encoder, such a parameter would have to be assumed, thereby rendering the high complexity mode much less effective.

### Low complexity model

The low complexity model uses the same video coding tools as the fixed bit rate, error-free environment low complexity model, described in Section 3.2.1.  To improve error resilience, a packetization method and a forced INTRA coding method are also included. Each GOB is coded with a GOB header. This header serves as synchronization markers and resets spatial predictive coding, such as motion vector coding and inter-coefficient coding, as defined in the H.263 Recommendation. Each coded GOB is packetized into one AL3 SDU. Any received AL3 SDUs that fails the CRC test are not processed by the decoder, but concealed. This test model currently makes no attempt to decode partially corrupted GOBs due to the difficulty to exactly define a decoder’s operation in such a case.  The only syntax violation that is allowed, and used to detect missing GOBs at the decoder, is the out-of-sequence GOB numbering.   Also, a forced INTRA macroblock refresh rate of once every 5 coded picture is employed at the encoder, and thus an average of about 20% of coded macroblocks per picture will be forced to the INTRA mode. This is implemented via the INTRA_MB_Refresh_Rate of the INTRA macroblock refresh mechanism, as described in Section 5.1.1.

# Common algorithms

Section 4 discusses algorithms common to all Application Scenarios.

## Motion estimation

The test model defines three motion estimation algorithms.

- A low complexity, fast-search algorithm based on a reduced number of search locations.

- A medium complexity, full-search algorithm.

- A high complexity, full-search algorithm that considers the resulting motion vector bit rate in addition to the quality of the match.

Of these, the low and high complexity are the most frequently used, for the corresponding low and high complexity models of the Application Scenarios.  The medium complexity algorithm can replace the low complexity algorithm and provides slightly better rate-distortion performance at much higher computational complexity. Note that any block-based search algorithm can be employed. However, the three described in the Test Model have been widely used and are known to perform well for the various Application Scenario models.

H.263 can use one or four motion vectors per macroblock depending of the optional modes that are enabled.  Refer to Annex D, F, and J of the H.263 Recommendation for a description of the various limitations of the motion vectors.  Also, the permissible extent of the search range depends on the sub-mode of Annex D being employed.

### Sum of absolute difference distortion measure

Both integer pel and half pel motion vector search algorithms use the sum of absolute difference (SAD) as a distortion measure.  The SAD is calculated between all luminance pels of the candidate and target macroblocks.  In some cases, the mode decision algorithm will favor the SAD for the (0,0) vector. The SAD for a candidate motion vector is calculated as follows:

SAD(u,v)=j=1Mi=1MXt(i,j)−Xt−1(i−u,j−v) whereM=8, for 8x8 motion vectors

The SAD calculation employs the partial distortion technique to increase efficiency.  The partial distortion technique compares the accumulated SAD after each row of M pixels to the minimum SAD found to date within the search window.  If the accumulated SAD exceeds the minimum SAD found to date, the calculation for the candidate motion vector is terminated, as the candidate motion vector will not produce a better match, in terms of a lower SAD value, than the best match found so far.

### Low complexity, fast-search

The low complexity search was approved after extensive comparison to the full-search algorithm showed that little or no performance degradation was incurred for QCIF and CIF resolution pictures. Details are available in [Q15-C-49] and [Q15-A-43].

#### 16x16 integer pel search

The search center is the median predicted motion vector as defined in Section 6.1.1 and Annex F.2 of H.263.  The (0,0) vector, if different than the predicted motion vector, is also searched.  The (0,0) vector is favored by subtracting 100 from the calculated SAD. The allowable search range is determined by the sub-mode of Annex D being employed.

The algorithm proceeds by sequentially searching diamond-shaped layers, each of which contains the four immediate neighbors of the current search center. Layer i+1 is then centered at the point of minimum SAD of layer i. Thus successive layers have different centers and contain at most three untested candidate motion vectors, except for the first layer around the predicted motion vector, which contains four untested candidate motion vectors.

The search is stopped only after

- all candidate motion vectors in the current layer have been considered and the minimum SAD value of the current layer is larger than that of the previous layer or

- after the search reaches the boundary of the allowable search region and attempts to go beyond this boundary.

The resulting 16x16 integer pel MV is then half-pel refined as described next.

#### 16x16 half pel refinement

The half pel search area is ±1 half pel around the best 16x16 candidate.  The search is performed by calculating half pel values as described in Section 6.1.2 of H.263, and then calculating the SAD for each possible half pel vector.  Thus, eight additional candidate vectors are tested.The vector resulting in the best match during the half pel refinement is retained.

#### 8x8 integer pel search

The 8x8 integer motion vectors are set to the best 16x16 integer pel motion vector.  There is no additional 8x8 integer-pel search. Because of the differential encoding of motion vectors, this restriction ensures that the 8x8 motion vectors selected for the target macroblock are highly correlated.

#### 8x8 half pel refinement

The half pel refinement, is performed for each of the blocks around the 8x8 integer pel vector.  The search is essentially the same as that described in Section 4.1.2.2, with  block sizes equal to 8.  Note that the (0,0) vectors are not favored here.

### Medium complexity, full search

#### 16x16 integer pel search

The search center is again the median predicted motion vector as defined in Section 6.1.1 and Annex F.2 of H.263.  The (0,0) vector, if different than the predicted motion vector, is also searched.  The (0,0) vector is favored by subtracting 100 from the calculated SAD. The allowable search range is determined by the sub-mode of Annex D being employed.

The algorithm proceeds by sequentially searching layers in an outward spiral pattern, with respect to the predicted motion vector, to the full extent of the permitted search range, for both the 16x16 and the 8x8 integer pel search.  Thus each layer, except for the layer consisting of only the search origin, adds 8*layer_number candidate motion vectors. For each layer, the search begins at the upper left corner of the spiral and proceeds in the clockwise direction.

This spiral search is more efficient if it originates from the median predicted motion vector, as this will usually produce a good match early in the search which, when combined with the partial distortion technique, can significantly reduce the number of row SADs that are calculated. Half pel refinement is identical to the low complexity algorithm, as described in Section 4.1.2.2.

#### 8x8 integer pel search

The 8x8 integer per search exhaustively searches a reduced search window, in the neighborhood the best 16x16 integer pel motion vector, of size +/-2 integer pel units. This permits a slightly greater variation in the 8x8 motion vectors for the target macroblock, but these vectors can now provide higher quality matches. Half pel refinement is identical to the low complexity algorithm, as described in Section 4.1.2.4.

### High complexity, rate-distortion optimized full search

In order to regularize the motion estimation problem, a Lagrangian formulation is used wherein distortion is weighted against rate using a Lagrange multiplier. If any mode which supports 8x8 motion vectors is enabled, e.g. Annex F and/or Annex J, a full search is employed for both the 16x16 and 8x8 integer pel motion vectors.   Otherwise, a full search is employed only for the 16x16 integer pel motion vectors.

For each 16x16 macroblock or 8x8 block, the integer pel motion vector search employs spiral search pattern, as described in Section 4.1.3.1.  the half pel refinement employs the pattern described in Section 4.1.2.2.  The half pel motion vector for 8x8 block i is obtained before proceeding to obtain the integer or half pel motion vectors for 8x8 block i+1, etc. This allows for accurate calculation of the rate terms for the integer and half pel motion vectors for the 8x8 blocks, as the predicted motion vector will be completely known.

The RD optimized integer pel search selects the motion vector that minimizes the Lagrangian cost defined as

The distortion, D, is defined as the SAD between the luminance component of the target macroblock (or block) and the macroblock (or block) in the reference picture displaced by the candidate motion vector. The SAD calculations use the partial distortion matching criteria using the minimum SAD found to date.  The rate, R, is defined as the sum of the rate for the vertical and horizontal macroblock (or block) motion vector candidates, taking into account the predicted motion vector as defined in Section 6.1.1 of the Recommendation. The parameter  is selected as described below. The search is centered at the predicted motion vector.  The (0,0) vector is searched but not favored, as the Lagrangian minimization already accounts for the motion vector rate. The best 16x16 and 8x8 candidates are then half pel refined using a +/- 1 half pel search. The half pel search also employs the same Lagrangian formulation as above, to select the half pel motion vector that minimzes the Lagrangian cost. The mode decision algorithm then uses the best 16x16 and 8x8 vectors.

The choice of λMOTION has a rather small impact on the result of the 16x16 block motion estimation. But the search result for 8x8 blocks is strongly affected by λMOTION, which is chosen as

λMOTION=0.92⋅QP,

where QP is the macroblock quantization parameter.  If the application is for a fixed bit rate, the relationship is modified as described in Section 5.2.3.

### Other motion vector search related issues

Although not used in any Application Scenarios, the test model includes motion vector search algorithms for PB-frames (when Annex G or M are enabled) and for the various picture types permitted with scalability mode defined in Annex O.  These are discussed in more detail in Sections 8.1.1 and 8.2.2 respectively.

## Quantization

The quantization parameter QUANT may take integer values from 1 to 31.  The quantization reconstruction spacing for non-zero coefficients is 2  QP, where:

QP = 4	for Intra DC coefficients when not in Advanced Intra Coding mode, and

QP = QUANT	otherwise.

Define the following:

COF	A transform coefficient (or coefficient difference) to be quantized,

LEVEL	The quantized version of the transform coefficient,

REC	Reconstructed coefficient value,

“/”	Division by truncation.

The basic inverse quantization reconstruction rule for all non-zero quantized coefficients can be expressed as:

|REC| = QP · (2 · |LEVEL| + p)    		if QP = “odd”, and

|REC| = QP · (2 · |LEVEL| + p) - p		if QP = “even”,

where

p = 1	for INTER coefficients, and

p = 1	for INTRA non-DC coefficients when not in Advanced Intra Coding mode, and

p = 0	for INTRA DC coefficients when not in Advanced Intra Coding mode, and

p = 0	for INTRA coefficients (DC and non-DC) when in Advanced Intra Coding mode.

The parameter p is unity when the reconstruction value spacing is non-uniform (i.e., when there is an expansion of the reconstruction spacing around zero), and p is zero otherwise.  The encoder quantization rule to be applied is compensated for the effect that p has on the reconstruction spacing.  In order for the quantization to be MSE-optimal, the quantizing decision thresholds should be spaced so that the reconstruction values form an expected-value centroid for each region.  If the pdf of the coefficients is modeled by the Laplacian distribution, a simple offset that is the same for each quantization interval can achieve this optimal spacing.  The coefficients are quantized according to such a rule, i.e., they use an “integerized” form of

|LEVEL| = [|COF| + (f  p)  QP] / (2  QP)

where f  {12,34, 1} is a parameter that is used to locate the quantizer decision thresholds such that each reconstruction value lies somewhere between an upward-rounding nearest-integer operation (f = 1) and a left-edge reconstruction operation (f = 0), and f is chosen to match the average (exponential) rate of decay of the pdf of the source over each non-zero step.

### Quantization for INTER Coefficients:

Inter coefficients (whether DC or not) are quantized according to:

|LEVEL| = (|COF|  QUANT / 2) / (2  QUANT)

This corresponds to f =12with p = 1.

### Quantization for INTRA non-DC coefficients when not in Advanced Intra Coding mode

Intra non-DC coefficients when not in Advanced Intra Coding mode are quantized according to:

|LEVEL| = |COF| / (2  QUANT)

This corresponds to f = 1 with p = 1.

### Quantization for INTRA DC coefficients when not in Advanced Intra Coding mode

The DC coefficient of an INTRA block when not in Advanced Intra Coding mode is quantized according to:

LEVEL = (COF + 4) / (2  4)

This corresponds to f = 1 with p = 0.  Note that COF and LEVEL are always non-negative and that QP is always 4 in this case.

### Quantization for INTRA coefficients when in Advanced Intra Coding mode

Intra coefficients when in Advanced Intra Coding mode (DC and non-DC) are quantized according to:

|LEVEL| = (|COF| + 3  QUANT / 4) / (2  QUANT)

This corresponds to f =34with p = 0.

## Alternative INTER VLC mode (Annex S)

During the entropy coding the encoder will use the Annex I INTRA VLC table for coding an INTER block if the following three criteria are satisfied:

- Annex S, alternative INTER VLC mode is used and signaled in the picture header.

- Coding the coefficients of an INTER block using the Annex I INTRA VLC table results in fewer bits than using the INTER VLC table.

- The use of Annex I INTRA VLC table shall be detectable by the decoder.  The decoder assumes that the coefficients are coded with the INTER VLC table.  The decoder detects the use of Annex I INTRA VLC table when coefficients outside the range of 64 coefficients of an 8x8 block are addressed.

With many large coefficients, this will easily happen due to the way the INTRA VLC was designed, and a significant number of bits can be saved at high bit rates.

## Advanced INTRA Coding mode (Annex I)

Advanced INTRA coding is a method to improve intra-block coding by using inter-block prediction. The application of this technique is describe in Annex I of the H.263 Recommendation. The procedure is essentially intra-block prediction followed by quantization as described in Section 4.2.4, and the use of different scanning orders and a different VLC table for entropy coding of the quantized coefficients.

Coding for intra-blocks is implemented by choosing one among the three predictive modes described in the H.263 Recommendation. Note that the H.263 Recommendation employs the reconstructed DCT coefficients to perform the inter-block prediction, whereas the original DCT coefficients are employed in the encoder to decide on the prediction mode.  This Test Model describes the mode decision performed at the encoder based on the original DCT coefficients.

The blocks of DCT coefficients employed during the prediction are labeled A(u,v), B(u,v) and C(u,v), where u and v are row and column indices, respectively. C(u,v) denotes the DCT coefficients of the block to be coded, A(u,v) denotes the block of DCT coefficients immediately above C(u,v) and B(u,v) denotes the block of DCT coefficients immediately to the left of C(u,v). The ability to use the reconstructed coefficient values for blocks A and B in the prediction of the coefficient values for block C depends on whether blocks A and B are in the same video picture segment as block C.  A picture segment is defined in Annex I of the H.263 Recommendation.  We define Ei(u,v) to be the prediction error for intra mode i=0,1,2. The prediction errors are computed for all three coding modes as follows:

mode 0: DC prediction only.

If (block A and block B are both intra coded and are both in the same picture segment as block C)

{

E0(0,0) = C(0,0) - ( A(0,0) + B(0,0) )//2

}

else

{

If (block A is intra coded and is in the same picture segment as block C)

{

E0(0,0) = C(0,0) - A(0,0)

}

else

{

If (block B is intra coded and is in the same picture segment as block C)

{

E0(0,0) = C(0,0) - B(0,0)

}

else

{

E0(0,0) =  C(0,0) – 1024

}

}

}

E0(u,v) = C(u,v)        u!=0, v!=0, u = 0..7, v = 0..7.

mode 1: DC and AC prediction from the block above.

If (block A is intra coded and is in the same picture segment as block C)

{

E1(0,v) = C(0,v) - A(0,v)          v = 0..7, and

E1(u,v) = C(u,v)                   u = 1..7, v = 0..7.

}

else

{

E1(0,0) = C(0,0) - 1024

E1(u,v) = C(u,v)             (u,v) != (0,0), u = 0,_,7, v = 0,_,7

}

mode 2: DC and AC prediction from the block to the left.

If (block B is intra coded and is in the same picture segment as block C)

{

E2(0,v) = C(u,0) - A(u,0)        u = 0..7, and

E2(u,v) = C(u,v)                 v = 1..7, u = 0..7.

}

else

{

E2(0,0) = C(0,0) - 1024

E2(u,v) = C(u,v)            (u,v) != (0,0), u = 0,_,7, v = 0,_,7

}

The prediction mode selection for Advanced INTRA Coding is done by evaluating the absolute sum of the prediction error, SADmode i, for the four luminance blocks in the macroblock and selecting the mode with the minimum value.

SADmode,i=bEi(0,0)+32uEi(u,0)+32vEi(0,v),

i = 0..3, b = 0 .. 3, u,v = 1..7.

Once the appropriate mode is selected, quantization is performed. The blocks are quantized as described in Section 4.2.4.

## Modified Quantization mode (Annex T)

Annex T, Modified Quantization, greatly reduces certain color artifacts (particularly at low bit rates) and increases the range of luminance coefficients. Moreover, Annex T allows the encoder to set the quantizer step size to any value at the macroblock granularity, which may improve the performance of rate control algorithms.  Annex T is mandated for all Application Scenarios and is strongly encouraged for low-bit rate product designs.

# Algorithms used for individual Application Scenarios

Section 5 discusses algorithms used for specific Application Scenarios.

## Mode decision

H.263 allows for several types of macroblock coding schemes, such as INTRA mode (coding non-predicted DCT coefficients), INTER mode (predictive coding using 1 motion vector) and INTER4V (predictive coding using 4 motion vectors).  The choice of the appropriate mode is one of the key functionalities of an encoder and the quality of the decision influences greatly the performance of the encoder. A high and a low complexity algorithm for both error-free and error-prone environments have been developed and are described in the following sections.  First, a mechanism for performing the mandatory INTRA update is described.

### INTRA macroblock refresh and update pattern

As required by the standard, every macroblock must be coded in INTRA mode at least once every 132 times when coefficients are transmitted.   To avoid large bursts of INTRA macroblocks for short periods, a simple pattern for the macroblock update is used to randomly initialize each macroblock’s update counter to a value in the range [0, 132]. The pseudo random generator of H.263 Annex A shall be used to intialize the random pattern.  This mode decision process overrides the coding mode after any other mode decision process.  The algorithm is described as follows:

MB_intra_update[xpos][ypos]: counter incremented by one every time coefficient information is sent for the macroblock at position (xpos, ypos), namely MB[xpos][ypos].

INTRA_MB_Refresh_Rate: in error free environments a constant (132).  In error prone environments, INTRA_MB_Refresh_Rate might be a integer variable (adapted to the error rate), with a value in (1, 132).

The INTRA mode for a given macroblock is chosen as:

Initialize after an I-picture: MB_intra_update[xpos][ypos] = random_of_Annex_A (0, INTRA_MB_Refresh_Rate)

While (more non I-picture to encode)

If ((MB_intra_update[xpos][ypos] == INTRA_MB_REFRESH_RATE) &&

(MB[xpos][ypos] contains coefficient))

{

Encode MB[xpos][ypos] in INTRA mode;

}

else if (MB[xpos][ypos] contains coefficient)

{

++MB_intra_update[xpos][ypos];

}

Further details are available in [Q15-E-15] and [Q15-E-37].

### Low complexity mode decision for error free environments

Figure 2 outlines the low complexity mode decision algorithm, which interacts with the steps of the low and medium complexity motion vector search, described in Section 4.1.2 and 4.1.3, respectively.

After performing integer-pel motion estimation, the encoder selects the prediction mode, INTRA or INTER.  The following parameters are calculated to make the INTRA/INTER decision:

μMB=1256i=116j=116X(i,j)

A=i=116j=116X(i,j)−μMB

INTRA mode is chosen if

A<(SAD(u,v)−500)

If INTRA mode is chosen, no further mode decision or motion estimation steps are necessary.  If rate control is in use, the DCT coefficients of the blocks are quantized using quantization parameter determined by rate control. Advanced INTRA coding mode operations are performed as defined in Section 4.4. The coded block pattern (CBP) is calculated and the bitstream for this macroblock is generated.

Figure 2. Block diagram of the low complexity mode decision process

If INTER mode is chosen the motion search continues with a half pel refinement around the 16x16 integer pel MV.

If 8x8 motion vectors are permitted, i.e. either Annex F or Annex J, or both are enabled, the motion search also continues with half pel search around the 8x8 integer pel MVs. The mode decision then proceeds to determine whether to use INTER or INTER4V prediction.

The SAD for the half pel accuracy 16x16 vector (including subtraction of 100 if the vector is (0,0)) is

SAD16(u,v)

The aggregate SAD for the 4 half pel accuracy 4 half 8x8 vectors is

SAD8×8=k=14SAD8,k(uk,vk)

INTER4V mode is chosen if

SAD8×8<SAD16−200,

otherwise INTER mode is chosen.

### High complexity mode decision for error free environments

The problem of optimum bit allocation to the motion vectors and the residual coding in any hybrid video coder is a non-separable problem requiring a high amount of computation. To circumvent this joint optimization, the problem is split into two parts: motion estimation and mode decision. High complexity motion estimation for the INTER and INTER4V mode is conducted first, as described in Section 4.1.4. Given these motion vectors, the overall rate-distortion costs for all considered coding modes are computed.

Independently, the encoder selects the macroblock coding mode that minimizes

The distortion, D, is defined as the sum of squared difference (SSD) between the luminance and chrominance component coefficients of the target macroblock and the quantized luminance and chrominance component coefficients for the given coding mode. The rate R is defined as the rate to encode the target macroblock including all control, motion, and texture information, for the given coding mode.

The parameter  is selected as

λMODE=0.85⋅QP2

where QP is the macroblock quantization parameter.

The coding modes explicitly tested are SKIPPED, INTRA, INTER, and INTER4V. The INTER mode uses the best 16x16 motion vector and the INTER4V mode uses the best 8x8 motion vectors as selected by high complexity motion estimation algorithm. If the application is for a fixed bit rate, the relationship is modified as described in Section 5.2.3.

If Annex F is in use, the overlapped block motion compensation window is not considered for making coding mode decisions. It is applied only after all motion vectors have been determined and mode decisions have been made for the frame.

### Low complexity mode decision for error-prone environments

The low complexity mode decision algorithm for error prone environments uses the same mechanism as described in Section 5.1.2, with one addition. The INTRA macroblock update frequency is increased, by using a value less than 132 for the INTRA_MB_Refresh_Rate parameter of Section 5.1.1.  Guidelines for choosing a value for this variable can be found in Sections 3.3.2 and 3.4.1.

### High complexity mode decision for error-prone environments

The high complexity mode decision algorithm for error prone environment is similar to the one described in Section 5.1.3. Independently for every macroblock, the encoder selects the coding mode that minimizes:

J=(1−p)D1+pD2+λMODER,

where λmodeis the same as Section 5.1.3.  To compute this cost, the encoder simulate the error concealment method employed by the decoder and must obtain the probability that a given macroblock is lost with probability p during transmission. The distortion thus arises from two sources, where D1 is the distortion attributed to a received macroblock, and D2 is the distortion attributed to a lost and concealed macroblock.

For a given MB, two distortions are computed for all coding modes considered: the coding distortion D1 and the concealment distortion D2. D1 is weighted by the probability (1-p) that this MB is received without error, and D2 is weighted by the probability p that the same MB is lost and concealed. D2 depends on the error concealment method employed and is constant for all coding mode considered. D1  will depend on the coding mode considered.  For the INTRA mode, the distortion is the same as the error free case.  For the SKIP and INTER modes, D1 is the quantization distortion plus the concealment distortion of the macroblock from which it is predicted, more precisely,

D1=Dq+pD2(v,n−1)

where D2(v,n-1) represents the concealment distortion of the macroblock in the previous frame, (n-1), at the current spatial location displaced by the motion vector v, where v is set to zero for the SKIP mode and set to the motion vector of the considered macroblock for INTER mode.

Dq is defined as the SSD between the reconstructed pixels values of the coded macroblock and the original pixel values, and D2 is defined as the SSD between the reconstructed pixels values of the coded macroblock and the concealed pixel values.  The rate R is defined as the rate to encode the target macroblock including all control, motion, and texture information, for the given coding mode.

## Rate control

This rate control algorithm applies for all fixed bit rate Application Scenarios.  It consists of a frame-layer, in which a target bit rate for the current frame is selected, and a macroblock-layer, in which the quantization parameter (QP) is adapted to achieve that target.  The development of this algorithm is described in [Q15-A-20].  This rate control algorithm, often referred to as TMN8 rate control, is the only rate control algorithm used for the Application Scenarios.

Initially, the number of bits in the buffer W is set to zero, W=0, and the parameters Kprev and Cprev are initialized to Kprev=0.5 and Cprev=0.  The first frame is intra-coded using a fixed value of QP for all macroblocks..

### Frame level rate control

The following definitions are used  in describing the algorithm:

B’ - Number of bits occupied by the previous encoded frame.

R  - Target bit rate in bits per second (e.g., 10000 bps, 24000 fps, etc.).

G  - Frame rate of the original video sequence in frames per second (e.g. 30 fps).

F  - Target frame rate in frames per second (e.g., 7.5 fps, 10 fps, etc.). G/F must be an integer.

M - Threshold for frame skipping. By default, set M = R/F. (M/R is the maximum buffer delay.)

A -  Target buffer delay is AM sec. By default, set A = 0.1.

The number of bits in the encoder buffer is W = max (W + B’ - R/F, 0). The skip parameter is set to 1, skip = 1.

While W > M

{

W= max (W - R/F, 0)

skip++

}

Skip encoding the next

frames of the original video sequence.

The target number of bits per frame is

where

For fixed frame rate applications, skip is always equal to 1, the frame skip is constant and equal to G/F –1.  Also, when computing the target number of bits per frame, A should be set to 0.5.

### Macroblock level rate control

Step 1.      Initialization.

It is assumed that the motion vector estimation has already been completed.

is defined as the variance of the luminance and chrominance values in the kth  macroblock.

If the kth  macroblock is of type I (intra), set

Let	  and   j = 0.

,  the target number of bits as defined in A.1.

, the number of macroblocks in a frame.

K= K1= Kprev, and C= C1= Cprev, the initial value of the model parameters.

,  where

Step 2.  Compute Optimized Q for ith  macroblock

If    (running out of bits),  set .

Otherwise, compute:

.

Step 3.  Find QP and Encode Macroblock

QP= round  to nearest integer in set 1,2, …, 31.

DQUANT = QP - QP_prev.

If  DQUANT > 2, set  DQUANT = 2.  If  DQUANT < -2,  set  DQUANT = -2;

Set  QP = QP_prev + DQUANT.

DCT encode macroblock with quantization parameter QP, and set QP_prev = QP.

Step 4.  Update Counters

Let  be the number of bits used to encode the ith  macroblock, compute:

and  .

Step 5.  Update Model Parameters K  and C

The model parameters measured for the i-th macroblock are :

,  and  ,

where   is the number of bits spent for the luminance and chrominance of the macroblock.

Next, measure the average of the ’s and ’s computed so far in the frame.

If  ( and ),  set j= j+1 and compute  .

Compute .

Finally, the updates are a weighted average of the initial estimates, K1, C1, and their current average:

Step  6.

If  i = N,  stop  (all macroblocks are encoded).

Set  Kprev= K and  Cprev= C.

Otherwise,  let  i = i+1, and go to Step 2.

### High complexity models and rate control

For Lagrangian-based minimizations, such as those described in the high complexity motion estimation and mode decision algorithms in Sections 4.1.4 and 5.1.3 respectively, the Lagrangian parameter must remain constant over an entire frame. This is because at optimality, all picture regions must operate at a constant slope point on their rate-distortion curves [6]. In theory, this optimal operating point is found via a search through all possible values of . However, in this Test Model, a simple relationship between  and the quantization parameter is defined which produces a good approximation to the optimal value of .

Because the rate control algorithm described above can update the quantization parameter at each coded macroblock, the relationships between  and the quantization parameter must be modified slightly. It has been shown that, to employ the high complexity models for fixed bit rate scenarios, the Lagrangian parameters should calculated as

λMOTION=0.92⋅QPprev, and

λMODE=0.85⋅QPprev2

respectively, where QPprev is the average value, over all macroblocks, of the quantization parameter from the previous coded frame of the same type as the current frame.

The high complexity motion estimation and mode decision algorithms then proceed to determine appropriate motion vectors and macroblock coding modes, as in the variable bit rate scenarios, i.e. as if no rate control algorithm was enabled. Once the optimal motion vectors and modes have been determined, the rate control algorithm  can then be employed to encode the frame based on the already determined motion vectors and coding modes. In the special case that the mode decision algorithm has selected SKIPPED mode for the target macroblock, the rate control algorithm is not permitted to update the quantization parameter.

# Encoder Pre-Processing

Section 6 discusses video pre-processing tools may be used at the video encoder before video frames are processed.  This section is currently empty, contributions from the H.263 Video Experts Group are encouraged.

# Decoder Post-Processing

This section defines the operation of the video decoder in error-prone environments,  as well as post processing methods such as error concealment and  post-filtering algorithms. While post-filtering is useful in error-free and error-prone environments, all other mechanisms described here apply only to error prone environments.  These should be employed packet lossy and bit error prone Applications Scenarios.

## De-ringing post-filter

The de-ringing post-filter described in this section should be used whenever Annex J is in use.  Otherwise, the de-blocking post-filter described in Section 7.2 should be employed. For the definition of the functions and notation used in this section, see the definition of the loop filter in Annex J.

The one-dimensional version of the filter will be described.  To obtain a two-dimensional effect, the filter is first used in the horizontal direction and then in the vertical direction.  The pels A,B,C,D,E,F,G(,H) are aligned horizontally or vertically.  A new value -D1 - for D will be produced by the filter:

D1 = D + Filter((A+B+C+E+F+G-6D)/8,Strength1)	when filtering in the first direction.

D1 = D + Filter((A+B+C+E+F+G-6D)/8, Strength2)	when filtering in the second direction.

As opposed to Annex J filter, the post filter is applied to all pels within the frame.  Edge pixels should be repeated when the filter is applied at picture boundaries.  Strength1 and Strength2 may be different to better adapt the total filter strength to QUANT.  The relation between Strength1, 2 and QUANT is given in the table below.  Strength1, 2 may be related to QUANT for the macroblock where D belongs or to some average value of QUANT over parts of the frame or over the whole frame.

A sliding window technique may be used to obtain the sum of 7 pels (A+B+C+D+E+F+G).  In this way the number of operations to implement the filter may be reduced.

Table 1/TMN

| QUANT | Strength | Strength1 | Strength2 | QUANT | Strength | Strength1 | Strength2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 17 | 8 | 3 | 3 |
| 2 | 1 | 1 | 1 | 18 | 8 | 3 | 3 |
| 3 | 2 | 1 | 1 | 19 | 8 | 3 | 3 |
| 4 | 2 | 1 | 1 | 20 | 9 | 3 | 3 |
| 5 | 3 | 1 | 1 | 21 | 9 | 3 | 3 |
| 6 | 3 | 2 | 1 | 22 | 9 | 3 | 3 |
| 7 | 4 | 2 | 1 | 23 | 10 | 3 | 3 |
| 8 | 4 | 2 | 2 | 24 | 10 | 4 | 3 |
| 9 | 4 | 2 | 2 | 25 | 10 | 4 | 3 |
| 10 | 5 | 2 | 2 | 26 | 11 | 4 | 3 |
| 11 | 5 | 3 | 2 | 27 | 11 | 4 | 3 |
| 12 | 6 | 3 | 2 | 28 | 11 | 4 | 3 |
| 13 | 6 | 3 | 2 | 29 | 12 | 4 | 3 |
| 14 | 7 | 3 | 2 | 30 | 12 | 4 | 3 |
| 15 | 7 | 3 | 3 | 31 | 12 | 4 | 3 |
| 16 | 7 | 3 | 3 |  |  |  |  |

## De-blocking post-filter

In this section, a two-dimensional regularized post filter to simultaneously reduce the blocking and ringing artifacts is described.  This filter is recommended when Annex J is not used, so that over-blurred results can be avoided.  The degradation model due to the quantization process can be written as

g(i,j)=f(i,j)+n(i,j)

where g, f, n denote the recontructed image, the original image, and quantization noise in spatial domain, and (i,j) represents the vertical and horizontal coordinates in the video frame..  A four dimensional  smoothness metric, M, is incorporated in order to minimize the quantization noise.  M is defined as:

M(f(i,j))=ML(f(i,j))+MR(f(i,j))+MU(f(i,j))+MD(f(i,j))ML(f(i,j))=(1−αL(f(i,j)))[f(i,j)−g(i,j)]2+αL(f(i,j))[f(i,j)−f(i,j−1)]2MR(f(i,j))=(1−αR(f(i,j)))[f(i,j)−g(i,j)]2+αR(f(i,j))[f(i,j)−f(i,j+1)]2MU(f(i,j))=(1−αU(f(i,j)))[f(i,j)−g(i,j)]2+αU(f(i,j))[f(i,j)−f(i−1,j)]2MD(f(i,j))=(1−αD(f(i,j)))[f(i,j)−g(i,j)]2+αD(f(i,j))[f(i,j)−f(i+1,j)]2

where αi(i=L,R,U,D) represents the regularization parameters to control the relative contribution between fidelity and directional smoothness. Figure 3 shows the example for the position of these pixels.

Figure 3. Example of pixels' positions

The COD bit is employed to estimate the correlation of a macroblock between a current frame and the previous codedframe.  Therefore, different filters are used depending on COD, resulting in complexity reduction . The following describes the filters when COD is 0 (coded) or 1 (not coded).

1.  condition: block1 belongs to a coded macroblock (COD = 0)

If block1 shown in Figure1 (grey region) belongs to a coded macroblock, the following filter is applied to all pixels of block1.  When pel g(i,j) belongs to block1,  f(i,j) is obtained by minimizing the  following function:

f(i,j)=(4−αTOT(f(i,j)))g(i,j)+A4αTOT(f(i,j))=αL(f(i,j))+αR(f(i,j))+αU(f(i,j))+αD(f(i,j))A=αL(f(i,j))g(i,j−1)+αR(f(i,j))g(i,j+1)+αU(f(i,j))g(i−1,j)+αD(f(i,j))g(i+1,j)

The regularization parameters are chosen as:

αL(f(i,j))=KLQPl2[g(i,j)−g(i,j−1)]2+KLQPl2αR(f(i,j))=KRQPl2[g(i,j)−g(i,j+1)]2+KRQPl2αU(f(i,j))=KUQPl2[g(i,j)−g(i−1,j)]2+KUQPl2αD(f(i,j))=KDQPl2[g(i,j)−g(i+1,j)]2+KDQPl2

where QPl denotes the value of quantization parameter (quantization step size) of the last coded macroblock.  Since larger smoothness is necessary at block boundaries and relatively smaller smoothness is required within a block, the constants, Ki (i=L,R,U,D) are determined depending on the position of g(i,j) such as                                                               KL,KU=9, if j mod8=0

2. condition: block1 belongs to an uncoded macroblock (COD  = 1)

If block1 shown in Figure1 (grey region) belongs to a uncoded macroblock, the following rule is applied to all pixels of block1.

f(i,j)=fp(i,j)

where fp represents the corresponding pixels in the recovered previous frame.

The following pseudo-code fragment can be used to ease the implementation of the described algorithm

f       /* two dimensional array,  recovered frame */

g       /* two dimensional array, reconstructed frame */

QP      /* quantization parameter */

K_L     /* left side constant for regularization parameter */

K_R     /* right side constant for regularization parameter */

K_U     /* up side constant for regularization parameter */

K_D     /* down side constant for regularization parameter */

alpha_L /* regularization parameter to left side */

alpha_R /* regularization parameter to right side */

alpha_U /* regularization parameter to up side */

alpha_D /* regularization parameter to down side */

QP=(QP*QP);

if(COD == 0)

{

diff_L=(g(i,j)-g(i,j-1));

diff_R=(g(i,j)-g(i,j+1));

diff_U=(g(i,j)-g(i-1,j));

diff_D=(g(i,j)-g(i+1,j));

if(i mod 8 ==0){

K_U=9;

K_D=1;

} else if(i mod 8 == 7){

K_U=1;

K_D=9;

} else{

K_U=1;

K_D=1;

}

if(j mod 8 ==0){

K_L=9;

K_R=1;

} else if (j mod 8 == 7){

K_L=1;

K_R=9;

} else{

K_L=1;

K_R=1;

}

alpha_L=((K_L*QP)/(diff_L*diff_L+K_L*QP));

alpha_R=((K_R*QP)/(diff_R*diff_R+K_R*QP));

alpha_U=((K_U*QP)/(diff_U*diff_U+K_U*QP));

alpha_D=((K_D*QP)/(diff_D*diff_D+K_D*QP));

f(i,j)=(((4-alpha_L-alpha_R-alpha_U-  alpha_D)*g(i,j)+alpha_L*g(i,j-)+alpha_R*g(i,j+1)+alpha_U*g(i-1,j)+alpha_D*g(i+1,j))/4);

}

## Error detection

In error prone environments, it is necessary that the decoder be tolerant to erroneously received  bitstreams. In practice, there is usually some mechanisms in the transport protocol hierarchy that provides information about damaged or lost parts of the bitstream (e.g. RTP sequence number or H.223 CRC check).  By dividing coded pictures into (more or less) independently decodable segments (using GOB headers or slices), a reasonable performance can be achieved, even without basic mechanisms for error recovery in the video decoder itself.

For the error-prone Application Scenarios described in Sections 3.3 and 3.4, bit errors are never present in the bitstream and the only possible syntactic error is missing information between synchronization markers (picture, GOB or slice headers). This error must be detected during decoding and error concealment as described in Section 7.4 must be applied to the missing macroblocks. If a picture header is lost, it is replaced by the picture header of the previous picture, or by a redundant picture header if available (e.g. in the RTP payload header).

In the case where bit errors are present in the received bitstream, errors must be detected by the video decoder using syntax or syntactic violations. These include:

- An illegal codeword is found.  This is the most frequent event to stop decoding.

- A synchronization marker does not follow after reconstruction of a macroblock line. (If it is not known that GBSCs are used on every row of macroblocks, this condition can not be tested).

- Motion vectors point outside of the allowable range.

- Position of reconstructed DCT coefficient points are outside position 63, when Annex S is not in use.

- Chroma DC values are out of the normal range.

The first two bullets are the most important.  The list of other checkpoints could be increased considerably.  When such an error is detected, the video decoder looks for the next synchronization point in the bitstream.  All data between synchronization markers where the error was detected is discarded and concealed as described in Section 7.4.

## Error concealment

Missing macroblocks are concealed using of a very simple, but effective algorithm.  In parts of the picture where data is lost, data from the previously decoded picture is used for concealment. No error concealment is applied of part of the first received frame are corrupted. The motion vector of the missing macroblock is set to the median value of the motion vectors of the macroblocks to the left, above and above right of the missing macroblock.  If no surrounding motion vectors are available, the motion vector is set to zero.  Using the appropriate motion vector, the macroblock from the previous frame at the spatial location specified by this motion vector is copied to the location of the missing macroblock in the current frame.

# Information capture section

Section 8 contains all information that was officially approved for adoption into the test model by Q.15 experts, but do not fit into the Application Scenarios defined in Section 3.  Most of this information applies only to situations when certain optional modes are used, such as the scalability mode (Annex O), PB-frames (Annex G, M), or the Reduced Resolution Update mode (Annex Q).  Not all of the information here is thoroughly checked to be compatible with the ‘mainstream’ algorithms used for the Application Scenarios.  Additional information is solicited.

## PB-frames mode (Annex G and M)

Annex M provides better prediction options than Annex G, and should always be employed instead of Annex G whenever H.263 Version 2 is employed.

### Improved PB-frames motion estimation and mode decision

The candidate forward and backward motion vectors for each of the blocks in the B-macroblock is obtained by scaling the best motion vector from the P-macroblock, MV, as specified in H.263. To find the SADbidir, these vectors are used to perform a bi-directional prediction, as described in Annex M, improved PB-frames, in the H.263 standard, but with MVD set to zero.

Then, for the 16x16 B-macroblock, a normal integer and half pel motion estimation is performed, relative to the previous reconstructed P-picture. The best SADforw for this motion estimation is compared with the SADbidir for the bi-directional prediction. If (SADforw < SADbidir- 100), forward prediction is chosen for this macroblock. In this case, the forward motion vector found in the motion estimation above, is transmitted directly in MVDB, with no motion vector prediction. If the bi-directional prediction is found to be the best, no MVDB is transmitted.

## Scalabilitymode (Annex O)

### True B-frame motion estimation and mode decision

B-frames have several permissible prediciton options, therefore coding modes. Forward motion estimation is performed relative to the previous reconstructed I/P-frame, and backward motion estimation is performed relative to the future reconstructed I/P-frame.

In addition, B-frames allow for direct mode prediction, which does not require motion vector data to be transmitted, and bi-directional mode prediction, which uses the resulting motion vectors of the forward and backward motion estimation. For direct mode prediction, the motion vector between the surrounding I/.P or P/P frame pair is scaled.

The preferred coding modes, in order, are direct, forward, backward and bi-directional. To reflect the preferences, the SAD is computed for each possible prediction mode then adjusted. The direct mode is favored by subtracting 100 from its SAD. The forward mode is favored by subtracting 50 from its SAD. The backward mode SAD is unchanged. The bi-directional mode SAD is increased by 75. The mode with the lowest SAD after these modifications is chosen as the coding mode.

### EI/EP frame motion estimation and mode decision

EI- and EP-frames also have several permissible prediciton options, therefore coding modes. Forward motion estimation is performed relative to the previous reconstructed EI/EP-frame.

In addition, EI- and EP-frames allow for upward mode prediction, which does not require motion vector data to be transmitted, and bi-directional mode prediction, which uses the resulting motion vectors of the forward motion estimation. For upward mode prediction, the prediction is made from the same spatial location of the reconstructed, and possibly upsampled, frame the reference layer.

The preferred coding modes, in order, are upward, forward and bi-directional. To reflect the preferences, the SAD is computed for each possible prediction mode then adjusted. The upward mode is favored by subtracting 50 from its SAD. The forward mode SAD is unchanged. The bi-directional mode SAD is increased by 100. The mode with the lowest SAD after these modifications is chosen as the coding mode.

### Rate Control for P and B Frames

#### Macroblock Level

The macroblock level rate control described in Section 5.2.2 can be used directly for B frames. The only difference is that since the statistics of B frames are different from those of P frames, the rate control parameters K and C (which are updated at each macroblock) take values in different ranges. Consequently, when using P and B frames, different parameters {KP, CP} and {KB, CB} for the P and B frames, respectively, are to be used.

#### Frame Level

The frame-level rate control in Section 5.2.1 assigns a near constant target number of bits per P frame (after the first I frame), which is an effective strategy for low-delay video communications. But in scenarios where one or several B frames are inserted between the P’s, since the B frames are easier to encode, some technique is needed to assign fewer bits to the B frames.

In this section, an appropriate technique for assigning target number of bits to P and B frames is described. The derivation of this method is discussed in [Q15-C-19] [Q15-D-22] [2]. The derivation is based on the typical case where the pattern of frame types is:

I,B,…,B,P,B,…,B,P, B,…,B,P,B,…,B,P, …

The set of frames “B,…,B,P” is repeated periodically after the first I frame and is referred to as a group of pictures or GOP. Let MB be the number of B frames in a GOP. The target number of bits for the P picture in that GOP, TP, and the target for each of the B frames, TB, can be computed as follows:

TP=T−MBTB

TB=T−162N (CP−βCB)β+MB

β=0.9βPREV+0.1FEPEC

where the parameters in the above equations are defined as

- T, M, and N are, respectively, the number of bits for the GOP, the number of frames for the GOP, and the number of macroblocks in a frame.

- The value of β determines how many bits are assigned to the P frame and how many are assigned to the B’s. β increases with F and EP/EB, which is described below.

- F determines how large the PSNR of the P frames is in comparison to that of the B’s. For example, if F is equal to 1, the PSNR of both types of frames will be similar and if F is larger than 1 the PSNR of the P’s increases with respect to the that of the B’s. The following formula is used to determine the value of F:

- F=maxmin1.4Bpp−0.3,5,1

- where Bpp is the rate in bits per pel for the video sequence. Using this relationship, the PSNR of the P frames is on average about 1 dB higher than that of the B frames, which appears to be a reasonable tradeoff.

- EP is the energy for the P frame in the previous GOP, where energy is defined as the sum of the variances of the macroblock prediction errors, i.e.,

- EP=i=1Nsi2,

- where σi2 is the variance of the ith macroblock in the (previous) P frame, as defined in Section 5.2.2. On the other hand, EB is the mean of the energies for the B frames in the previous GOP, i.e.,

- EB=1MBm=1MBEB,m,

- where EB,m is the energy of the mth  B frame in the previous GOP.

- βPREVis set to F for the first GOP and to the previous value of β for the next GOPs.

- CP and CB are the motion and syntax rate (in bits per pel) for the P and B frames, respectively, and their values are obtained from the rate control at the respective macroblock levels as described in Section 5.2.2.

The previous frame-level rate control described in Section 5.2.1, was designed for GOP’s of the type “P…P”,  and corresponds to the special case where EP = EB, F = 1, (or, equivalently, β = 1) and CP = CB  in the above equations.

Finally, before a given frame is encoded (with a target of either TP or TB bits), the value D is subtracted from the target bit rate, as defined in Section 5.2.1, which provides feedback from the fullness of the encoder buffer and the frame skipping threshold. The latter was set to the channel bit rate (in bits per second) divided by the encoding frame rate, which is a good choice for low-delay scenarios. But when B frames are inserted between the P’s, and hence delay is not as important, a larger frame skipping threshold (and larger value of A in Section 5.2.1) would be more appropriate.

#### SNR and Spatial Enhancement Layer Rate Control

Typically the bit rate available for each of the enhancement layers is determined by the specific application. At each layer, the rate control can be used equivalently as in the base layer with the bit rate, frame rate, and GOP pattern for the given layer. The only difference is that, since different frames have different statistics at different layers, there must be different variables for K and C at each layer. Specifically, there should be different parameters K and C for different layers and, within a layer, different K and C’s for each frame type.

## Reduced-Resolution Update mode (Annex Q)

This section describes how to use the Reduced-Resolution Update mode, defined in Annex Q.  The information here has not been verified.

### Motion estimation and mode selection

In Reduced-Resolution Update mode, motion estimation is performed on the luminance 32x32 macroblock instead of 16x16 macroblock. SAD (Sum of Absolute Difference) is used as an error measure.  In this mode, each component of the macroblock motion vector or four motion vectors is restricted to be half-integer or zero value in order to widen the search range with the same MVD table.

### Motion estimation in baseline mode (no options)

#### Integer pel motion estimation

The search is made with integer pel displacement in the Y component.  The comparisons are made between the incoming macroblock and the displaced macroblock in the previous reconstructed  picture.  A full search is used, and the search area is up to ±30 pels in the horizontal and vertical direction around the original macro block position.

SAD(x,y)=i=1,j=132,32original−decodedprevious, x,y=
          up +- 30

For the zero vector, SAD(0,0) is reduced by 400 to favor the zero vector when there is no significant difference.

SAD(0,0)=SAD(0,0)−400

The (x,y) pair resulting in the lowest SAD is chosen as the integer pel motion vector, MV0.  The corresponding SAD is SAD(x,y).

#### INTRA/INTER mode decision

After the integer pel motion estimation the coder makes a decision on whether to use INTRA or INTER prediction in the coding.  The following parameters are calculated to make the INTRA/INTER decision:

MBmean=(i=1,j=132,32original)1024

A=i=1,j=132,32original−MBmean

INTRA mode is chosen if:   A<(SAD(x,y)−2000)

Notice that if SAD(0,0) is used, this is the value that is already reduced by 400 above.

If INTRA mode is chosen, no further operations are necessary for the motion search. If INTER mode is chosen the motion search continues with half pel search around the MV0 position.

#### Half pel search

The half pel search is done using the previous reconstructed frame. The search is performed on the Y-component of the macroblock.  The search area is ±1 half pel around the 32x32 target matrix pointed to by MV0, complying with the condition that each component of the candidate vector for the half pel search is half-integer or zero value.  For the zero vector (0,0), SAD(0,0) is reduced by 400 as for the integer search.

The half pel values are calculated as described in ITU-T Recommendation H.263, Section 6.1.2.

The vector resulting in the best match during the half pel search is named MV. MV consists of horizontal and vertical components (MVx, MVy), both measured in half pel units.

### Motion estimation in advanced prediction mode (Annex F)

This section applies only if advanced prediction mode is selected.

#### Integer pel motion estimation

2 integer pel search within [-31, 30] is performed for the 16x16 blocks around 32x32 integer vector.

#### Half pel search

The half pel search is performed for each of the blocks around the 16x16 integer vector.  The search area is ±0.5 pel around the 16x16 integer vector of the corresponding block, complying the condition that each component of the candidate vector for the half pel search is half-integer or zero value and within [-31.5, 30.5].

#### One vs. Four MV Decision in AP

This section applies only if advanced prediction mode is selected.

SAD for the best half pel 32x32 MB vector (including subtraction of 400 if the vector is (0,0)):

SAD32(x,y)

SAD for the whole macroblock for the best half pel 16x16 block vectors:

SAD4x16=14SAD16(x,y)

The following rule applies:

If:	SAD4x16<SAD32−800,	choose 16x16 block prediction

otherwise:				choose 32x32 MB prediction

### Motion estimation in the unrestricted motion vector mode (Annex D)

This section applies only if the extended motion vector range in the UMV mode is selected.

#### Search window limitation

Since the window with legal motion vectors in this mode is centered around the motion vector predictor for the current macroblock, some restrictions on the integer motion vector search is applied, to make sure the motion vectors found will be transmittable.

With these restrictions, both the 32x32 MB vector and the 16x16 block vectors found with the procedure described below, will be transmittable, no matter what the actual half pel accuracy motion vector predictor for the macroblock, or each of the four blocks, turns out to be.

#### Integer pel search

First, the motion vector predictor for the 32x32 MB vector based on integer motion vectors only, is found. The 32x32 MB search is then centered around the truncated predictor, with a somewhat limited search window. If four vectors, the 32x32 MB search window is limited to the range 29 - (2*16x16_block_search_window+1).   Since in this model the 16x16_block search window is 2.5, the default search window of 32x32 MB in the UMV mode turns out to be 23 integer positions.  Then the 16x16_block searches are centered around the best 32x32 MB vector, and 2 pel search is performed in each 16x16_block.

#### Half pel search

Half pel searches are performed as in the other modes.  The search area is ±0.5 pel around the best integer vector of the corresponding macroblock / block, complying the condition that each component of the candidate vector for the half pel search is half-integer or zero value.

### Down-sampling of the prediction error

After motion compensation on 16*16 block basis, the 16*16 prediction error block is down-sampled to the 8*8 reduced-resolution prediction error block.  In order to realize a simple implementation, filtering is constrained to a block which enables up-sampling on an individual block basis.  Fig. 1 shows the positioning of samples.  The down-sampling procedure for the luminance and chrominance pels is defined Fig.2.  Filtering is performed regardless of the block boundary.   “/” in Fig.2 indicates division by truncation.

Fig.  1

Positioning of samples in 8*8 reduced-resolution prediction error block and

16*16 prediction error block

Fig.  2

Creation of reduced-resolution prediction error

for pels inside block

### Transform and Quantization

A separable 2-dimensional Discrete Cosine Transform (DCT) is applied to the 8*8 reduced-resolution prediction error block in the same way as the default mode.  Then Quantization is performed in the same way as described in the default mode.

### Switching

In this section, a simple switching algorithm for Annex Q is described.

Note: This algorithm might be applicable to “Factor of 4” part of Annex P with a small modification.

#### Resolution Decision Algorithm

In order to decide the resolution for Annex Q, a simple decision algorithm is used based on QPi−1 and Bi−1.

QPi−1 :  the mean QP of the previous encoded frame

Bi−1    :  the number of bits used in the previous encoded frame.

Assuming that the relation between QPi−1and Bi−1 is close to inverse proportion, the product of QPi−1 and Bi−1 can be regarded as an index of the approximate complexity of the coded frame.

In the case the current frame is default mode, switching to reduced-resolution update mode is done if the product of QPi−1 and Bi−1 is larger than the threshold TH1.

In the case the current frame is reduced-resolution update, switching to default mode is done if this product of QPi−1 and Bi−1 is smaller than the threshold TH2.

from default mode to reduced-resolution update mode

if(QPi−1* Bi−1 > TH1){

Switch to

reduced-resolution update mode;

}

from reduced-resolution update mode to default mode

if(QPi−1* Bi−1 < TH2){

Switch to default mode;

}

TH1 is determined in the following equation, where QP1 and FR1 represents  the lowest subjective quality which is permitted to be encoded in default mode.

TH1 = QP1 * (Target_Bit rate / FR1)

In the same way, TH2 is determined in the following equation,  where QP2 and FR2 represents the highest subjective quality which is permitted to be encoded in reduced-resolution update mode.

TH2 = QP2 * (Target_Bit rate / FR2)

The values of QP1, FR1, QP2, and FR2 may depend on the source format, target frame rate, and target bit rate.  If Source Format indicates CIF, the target frame rate is 10 fps, and the target bit rate is 48kbps, then QP1 = 16, FR1 = 7,  QP2 = 7, FR2 = 9, respectively.

### Restriction of DCT coefficients in switching from reduced-resolution to normal-resolution

Once the reduced-resolution mode is selected, the detail of the image is likely to be lost.  If the mode goes back to the default mode again, the detail of image must be reproduced, which consumes a large amount of bits. This sudden increase of coding bits often causes an unintentional frame skips.  Furthermore,  because the resolution-decision algorithm described above uses the product of mean QP and the amounts of bits, this sudden increase of the bits cause to switch back to reduced-resolution update mode, and the oscillation between both modes often occurs.  In order to avoid this degradation, the restriction of DCT coefficients to be sent is introduced to the several frames after switching from reduced-resolution update mode to default mode.  In the first frame after switching to the default mode, the coefficients only within 4x4 low frequency can be sent,  then in the same way, 5x5  in the second, 6x6 in the third, and 7x7 in the forth.  This “smooth-landing” algorithm can suppress the unintentional frame skip and the oscillation of the modes effectively.

#### Rate Control

The rate control is identical to the default mode, except the quarter number of Macroblocks.

## Fast Search Using Mathematical Inequalities

Several methods are known to speed-up motion search that are based on mathematical inequalities [1][3].  These inequalities, e.g., the triangle inequality, give a lower bound on the norm of the difference between vectors. In block matching, the search criteria very often used for the distortion are the sum of the absolute differences (SAD) or the sum of the squared differences (SSD) between the motion-compensated prediction c[x,y] and the original signal s[x,y]. Incorporating the triangle inequality into the sums for SAD and SSD yields:

D(s,c)=x,yÎBsx,y−cx,yp³

D(s,c)=x,yÎBsx,yp1p−x,yÎBcx,yp1pp	(1)

by varying the parameter p=1 for SAD and p=2 for SSD. Note that for p=2, the inequality used in [3] differs from (1). Empirically, little difference has been found between those inequalities. For some blocks, the inequality used in [3] provides a more accurate bound whereas for other blocks the triangle inequality performs better.The set B comprises the sampling positions of the blocks considered, e.g., a block of 16x16 samples.

Assume Dmin to be the smallest distortion value previously computed in the block motion search. Then, the distortion D(s,c) of another block c in our search range is guaranteed to exceed Dmin if the lower bound of D(s,c) exceeds Dmin. More precisely, reject block c if

D(s,c)≥Dmin				(2)

The special structure of the motion estimation problem permits a fast method to compute the norm values of all blocks c[x,y] in the previously decoded frames [1]. The extension to a rate-constrained motion estimation criterion is straightforward [5].

### Search Order

It is obvious that a small value for Dmin determined in the beginning of the search leads to the rejection of many other blocks later and thus reduces computation. Hence, the order in which the blocks in the search range are checked has a high impact on the computation time. For example, given the Huffman code tables for the motion vectors as prior information about our search space, the search ordering should follow increasing bit-rate for the motion vectors. This increases the probability of finding a good match early in the search process. A good approximation of these probabilities is the spiral search,described in Section 4.1.3.1.

### Multiple Triangle Inequalities

Following [2], multiple triangle inequalities can be employed. Assume a partition of B into subsets Bn so that

B=nBn, and intersectnBn=∅				(3)

The triangle inequality (1) holds for all possible subsets Bn. Rewriting the formula for D(s,c) yields:

x,yÎBsx,y−cx,yp=nx,yÎBnsx,y−cx,yp			(4)

and applying the triangle inequality for all Bn yields

D(s,c)=x,yÎBsx,y−cx,yp³

nx,y∈Bsx,yp1p−x,y∈Bcx,yp1pp		 	(5)

Note that (5) is a tighter lower bound than (1), however, requires more computation. Hence, this allows for the trade-off of sharpness of the lower bound against computational complexity.

An important issue within this context remains to be the choice of the partitions Bn. Of course, (5) works for all possible subsets that satisfy (4). However, since the norm values of all blocks in the search space have to be pre-computed, the fast method described in [1] can be applied. Therefore, a random sub-division of B into n arbitary subsets may not be the appropriate choice. Instead, for sake of computation, a symmetric sub-division of B may be more desirable. In [3], it is proposed to divide a square 16x16 block into two different partitions. The first partitioning produces 16 subsets Bn each being one of 16 lines containing 16 samples. The second partition consists of 16 subsets Bn each being one of 16 columns containing 16 samples.

Since H.263 permits 8x8 blocks. the approach proposed in [4] is applied, where a 16x16 block is decomposed into sub-blocks. The 16x16 block is partitioned into 1 set of 16x16 samples, into 4 subsets of 8x8 samples. The various (subset) triangle inequalities are successively applied in the order of the computation time to evaluate them, i.e., first the 16x16 triangle inequality is checked, then the inequalities relating to blocks of size 8x8. On the 8x8 block level, the 8x8 triangle inequality is checked only.

## Control of Encoding Frame Rate

This section contains two algorithms that can be used instead of the frame layer rate control algorithm defined in Section 5.2.1.  While the fixed frame rate algorithm is useful only in environments where a completely fixed frame rate is more important than perceptual quality and delay, the adaptive control algorithm increases the perceptual quality while maintaining reasonable delay characteristics by adjusting the frame rate according to the scenes content.

### Adaptive control of the Encoding Frame Rate

The objective of this control of encoding frame rate algorithm is to keep the quality of P-frames in the tolerable range under sudden motion change and time-varying communication channel environments without obvious degradation in the perceptual motion smoothness.  The algorithm adjusts the encoding frame rate adaptively based on the motion information in the underlying video to keep the image quality of each P-frame in a tolerable range.  Since it is difficult to support good quality in both spatial and temporal resolution (in terms of motion smoothness) at very low bit rates, an control of encoding frame rate is adopted for a tradeoff of spatial/temporal quality based on the motion in video and the available channel bandwidth.  It is observed that human eyes are sensitive to the abrupt encoding frame rate (or interval) change.  This scheme aims at the reduction of temporal degradation in terms of motion jerkiness perceived by human beings.  At the same time, low encoding time-delay is imposed for real-time processing.  The control of encoding frame rate under time-varying CBR and the relation with TMN8 rate control algorithm is explained in the followings.

The following definitions are used.

i: Encoded frame index.

a,b,a' and b': Frame layer R-D model coefficients.

fi−1: Reconstructed reference frame at the previous time instant.

fi : Uncompressed image at the current time instant.

QPi: Average QP of all macroblocks in a frame.

B(Q) and  {D(Q): Rate and distortion models of a frame respectively.

MAD(fi−1,fi) : Mean of absolute difference between fi−1 and fi.

R: Current channel bandwidth.

Fi: Current encoding frame interval under the assumption the camera captures frames at a rate of G fps.

Step 1: Set the initial parameters.

Initial encoding frame rate F0.

Encoding frame interval adjustment threshold c=0.04.

(When c≥2, control of encoding frame rate is turned off and only macroblock layer rate control (TMN8) works.)

Frame layer R-D table size Tmax= 30.

i=0

Current frame layer R-D table index Ti=1

Step 2: Shift and add the R-D database table.

Step 2-1: For k=1:Ti−1

Bk=Bk+1,Dk=Dk+1,QPk=QPk+1

Step 2-2:

BTi=Bi,DTi=Di,QPTi=QPi

Step 3: Derive rate and distortion models w.r.t. the average QP of P frames.

Step 3-1: Calculate the rate and distortion model coefficients by using the above   database table.

B(Q)=aQ+bQ2⋅MAD(fi−1,fi),D(Q)=a'Q+b'where a=j=1TiBj⋅QPj−1j=1TiQPj−4−j=1TiBiQPj−2j=1TiQPj−32j=1TiQPj−2j=1TiQPj−4−j=1TiQPj−1j=1TiQPj−3,          b=j=1TiBjQPj−2−2a⋅j=1TiQPj−22j=1TiQPj−3,          a'=Tij=1TiDjQPj2Tij=1TiQPj−2−j=1TiQPj2   and          b'=j=1TiDj−a'⋅j=1TiQPjTi.

Step 3-2: Remove outlier

For k=1:Ti

If B−Bk>σB or D−Dk>σD then disable this datum temporarily.

Then repeat the Step 3-1 again with the refined data.

Step 4: Calculate the estimated distortion.

Step 5: Determine the encoding frame interval.

Step 5-1: Calculate the threshold values.

THd1=(1+c)⋅Davg,THd2=(1−c2)⋅Davg,

where Davg is the average distortion of previous 5 encoded frames.

Step 5-2: Adjust the encoding frame interval

If  {D>THd1, then increase the encoding frame interval by ΔFi(Fi).If  {D<THd2, then decrease the encoding frame interval by ΔFi(Fi).Otherwise keep the current encoding frame interval,

Step 6: Calculate the target frame rate for the current frame.

Bi=(Fi+ΔF(Fi))⋅RG   if  {DTHd1,(Fi−ΔF(Fi))⋅RG   if  {DTHd2,

Step 7: Call TMN8 (5.2.2 Macroblock level rate control) with the target bit rate Bi.

Returned data are QPi,Bi and Di.

Step 8: If all frames are encoded, then stop.

Otherwise go to Step 2 with Ti=minTi+1,Tmax and i=i+1.

# Future extensions

Section 9 captures ideas that are not yet fully studied by Q.15 experts, but deemed to be most likely beneficial for a video communication system.

## RD-optimized mode decision and quantization working under bit rate constraints

It was reported that the current high complexity variable bit rate algorithm (using RD-optimized mode decision) is not fully interoperable with the rate control mechanism , defined in this test model.  It should, however, be feasible to define an RD-optimized mode decision and quantization process, that takes rate constraints into account.

## Frame rate calculation

In real world systems the selection of a appropriate frame rate depending on target bit-rate, network conditions and the characteristics of the input video is one of the most important parts of an encoder.  Currently, Q.15 uses expert judgement to select a appropriate input video sequence / bit rate combination for all rate-contraint tests.  To automatize this decision process is a valuable goal.

## Bi-directional communication in error prone environments

H.263+ and all relevant ITU-T transport environments (H.320, H.323, H.324) do support feedback channel based error resilience support.  This support can be handled on the transport layer (such as using TCP as transport protocol, or using H.223’s AL3 retransmission mechanism), using ARQ (see Appendix 2 of H.263+ for details) or using Annex N of H.263+.  Although academic research showed the superiority of such mechanisms over any uni-directional mechanism currently used in the test model none of those mechanisms are currently used due to their complexity of simulation.

# References

- W. Li and E. Salari, “Successive Elimination Algorithm for Motion Estimation,” IEEE Trans. Image Proc., pp.105-107, Jan. 1995.

- T. Wiegand, X. Zhang, and B. Girod, “Long-Term Memory Motion-Compensated Prediction“,in IEEE Trans. on Circuits and Systems for Video Technology, Sep. 1998.

- Y.-C. Lin and S.-C. Tai, ``Fast Full-Search Block-Matching Algorithm for Motion-Compensated Video Compression'', in IEEE TR-COM, vol. 45, no. 5, pp. 527--531, May 1997.

- C.-H. Lee and L.-H. Chen, ``A Fast Search Algorithm for Vector Quantization Using Mean Pyramids of Codewords'', in IEEE TR-COM, vol. 43, no. 2/3/4, pp. 604--612, Feb./Mar./Apr. 1995.

- M.Coban and R.M. Mersereau, ``Computationally Efficient Exhaustive Search Algorithm for Rate-Constrained Motion Estimation'', in Proc. ICIP, Santa Barbara, USA, Oct. 1997.

- Y. Shoham and A. Gersho “Efficient Bit Allocation for an Arbitrary Set of Quantizers”, in IEEE Trans. On Acoustics, Speech, and Signal Processing, vol. 36, no. 9, pp. 1445—1453, Sep. 1988.

# Appendix: Common Simulation Conditions

This appendix references the appropriate common condition documents for the Application Scenarios described in Section 3.  References to anchor sequences, if available, are provided. Additional software is also indicated.

| Application Scenario | Reference Documents |
| --- | --- |
| Variable bit rate, error free (Section 3.1) | Conditions: Q15-D-62R1 Anchors: Q15-D-50 |
| Fixed bit rate, error free (Section 3.2) | Conditions: Q15-G-45 |
| Fixed bit rate, packet lossy (Section 3.3) | Conditions: |
| Fixed bit rate, highly bit error prone (Section 3.4) | Conditions: Q15-F-45 Anchors: Q15-G-31 Simulator: Q15-G-38 |

Versions 20 and 21 of the H.263 (Version 2) working draft are technically identical to the Recommendation.  Minor editorial revisions incorporated at the Jan ’97 approval meeting were included in Draft 21.