| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Seventh Meeting: Monterey, 16-19 February, 1999 | Document Q15-G-24 Filename: q15g24.doc Generated: 05 Feb ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Surin Kittitornkun, Yu Hen Hu Department of Electrical and Computer Engineering University of Wisconsin - Madison Madison, WI 53706-1691, USA Adam H. Li Department of Physics University of Wisconsin - Madison Madison, WI 53706, USA | Tel: Fax: Email: | 1-608-262-6274 1-608-262-1267 hu@engr.wisc.edu |
| Title: | Results of cross-validation experiments for data partitioning |  |  |
| Purpose: | To report results of cross validate the error resilience capability of the data partitioning scheme by performing simulations as specified by the test model |  |  |

_____________________________

# Summary

For error resilience capability of H.263+ bitstream, a data partitioning structure based on reversible variable length codes (RVLCs) was proposed by University of Los Angeles (UCLA) and Samsung Electronics [1,2]. Model implementation of such structure was made [3], and qualitative measurement of the performance of such data partitioning structure was carried out [4,5]. At the same time, there are also discussions on simulation environment, common condition and other issues for performing the qualitative measurement of the performance [6,7].

Here, we present the results of a simulation experiments for this data partitioning structure with RVLCs. To cross-validate the simulation result presented in previous meeting, we had an independent implementation on the encoder side at module level. Our simulation tests comply with all the current suggested simulation condition. We also did additional simulation under different forced update rate.

The results of the simulation show a significant and consistent improvement of the performance of the Data Partitioning with RVLC structure compare to the Anchor structure at different conditions we tested.

Validation and Simulation Method

The goal of the work is to

- Quantitatively measure the performance of  the Data Partitioning structure with RVLCs and compare it with those of the Anchor mode

- Cross validate the simulation result of UCLA and Samsung from previous documents

We based our code on the code of UCLA and Samsung simulation. We re-implement the encoder side of the codec on a module by module base. The following modules of the code are rewritten and replaced: MB_buffer_full, flush_MB_buffer, put_cbpcm_intra, put_cbpcm_inter, FindPMV_ER, CountBitsMB, CountBitsVectors,and put_rvlc.

We use the same decoder as the UCLA and Samsung simulation without modification (except for some debuging). In general, the result is greatly decoder dependent. By using the same decoder as the UCLA and Samsung simulation while re-implementing the encoder, it makes it possible to verify the error resilience performance of the new packet structure, and eliminate the ambiguity associated with the decoder.

During simulation, 4000 frames of each video sequence are encoded. Annex D, F, I, J and T are invoked, and an initial quantizer QP of 20 is set with the encoder. The encoded bit stream is multiplexed with AL-3 multiplexer which allocates 75% of transmission bit rate to video packets and 25% to other packets. Particular error pattern is then applied to the CRC checked transmitted binary digits. On the decoder side, the multiplexer processes the data, and passes the bitstream to the decoder while discarding the CRC check results. The decoder (with TCON error concealment option invoked) then decodes the bitstream accordingly, and PSNR is calculated according to specified in previous documents on the simulation environment [6,7].

The following sequences are used in the simulation:

For 64 kbit/s	Foreman	7.5 fps

Hall		10 fps

Container	10 fps

News		10 fps

Silent Voice	15 fps

Glasgow Tour	15 fps

For 32 kbit/s	Hall		10 fps

Container	10 fps

News		10 fps

Silent Voice	15 fps

In additional to the conditions mentioned above, the simulation is done for all sequences encoded with Forced Updating rate of 132 (least frequent INTRA update required by the standard) and Forced Update rate of 5 (more frequent INTRA update).

Six WCDMA error patterns are applied in simulation for the 64 kbit/s bitstreams. And for the 32 kbit/s bitstreams, each of the 10 dB and 20 dB Rayleigh patterns for DECT systems with two different Doppler speeds, 1.4 and 14 km/h is applied.

# Results and Analysis

The simulation results are displayed in the Appendix I, II and III of this document. The main conclusion from analysis of these simulation results can be summarized as follow.

- The Data Partitioning structure with RVLCs has significant and consistent improvement compare to the Anchor mode performance under Forced Update rate of 132.

The full result can be found in the Appendix I of this document. For the purpose of illustrating how much improvement we have for each error pattern, we average the luminance PSNR gain for all the bitstreams tested for that particular error pattern, and show it in the following table, Table 1.

| Error Pattern | WCDMA |  |  |  |  |  | DECT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 10 dB |  | 20 dB |  |
|  |  |  |  |  |  |  | 14 km/h | 1.4 km/h | 14 km/h | 1.4 km/h |
| Improvement of PSNR – Y (dB) | +3.55 | +3.79 | +3.51 | +1.80 | +1.95 | +1.35 | +3.66 | +1.03 | +3.46 | -0.13 |

Table 1. Improvement of PSNR – Y under Forced Update Rate of 132 (averaged over 6 sequences).

- The Data Partitioning structure with RVLCs has significant and consistent improvement compare to the Anchor mode performance under Forced Update Rate of  5.

Again, for illustration, the result under Forced Update Rate of 5 is included in a similar manner in Table 2 below. The full result can be found in Appendix II of this document.

By direct comparing the result under Forced Update Rate of 5 to the result under Forced Update Rate of 132 above, it is found that the improvement is consistent. We see the improvements of the same range or may be a little bit lower improvement for the WCDMA error, and for the DECT error patterns, the improvement is actually much larger under the Forced Update Rate of 5 condition.

| Error Pattern | WCDMA |  |  |  |  |  | DECT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 10 dB |  | 20 dB |  |
|  |  |  |  |  |  |  | 14 km/h | 1.4 km/h | 14 km/h | 1.4 km/h |
| Improvement of PSNR – Y (dB) | +3.94 | +2.73 | +3.28 | +0.79 | +0.76 | +1.17 | +5.49 | +3.26 | +4.28 | +1.71 |

Table 2. Improvement of PSNR – Y under Forced Update Rate of 5 (averaged over 4 sequences).

# Conclusion

In conclusion, by simulations under the common conditions recommended by the ITU documentation, we show that the Data Partitioning structure with RVLCs brings a significant and consistent improvement of PSNR over the Anchor mode for H.263+ bitstreams sent through simulated error-prone channels. And the improvement holds also at different Forced Update Rate.

# References

[1] J. Wen, J. Villasenor, and H.D. Shin. “Proposal for Single-Thread Motion Vector Prediction in H.263L Error Resilient Mode”, ITU-T Documentation Q15-C-35 (Eibsee, Germany 1997).

[2] J. Wen, J. Villasenor, and H.D. Shin. “Proposal for Error Resilience in H.263L”, ITU-T Documentation Q15-C-36 (Eibsee, Germany 1997).

[3] A. Li, J. Wen, J. Villasenor, and J.H. Park. “Model Implementation of Error Resilience Mode in H.263+ codec”, ITU-T Documantition Q15-E-19 (Whistler, Canada 1998)

[4] A. Li, M. Fong, J. Wen, J. Villasenor, and J.H. Park. “Test Results of Error Resilience with Modified Error Resilience Syntax with Data Partitioning and RVLC”, ITU-T Documentation Q15-C-20 (Whistler, Canada 1998)

[5] M. Luttrell, A. Li, M. Fong, J. Villasenor, and J.H. Park. “Simulation Results for Modified Error Resilience Syntax with Data Partitioning and RVLC”, ITU-T Documentation Q15-F-29 (Seoul, Korea 1998)

[6] S. Wenger. “Common Conditions for Video Performance Evluation in H.324/M Errror-Prone Systems”, ITU-T Documentation Q15-E-50 (Whistler, Canada 1998)

[7] S. Wenger. “Proposed Changes for Mobile Test Model”, ITU-T Documentation Q15-F-38 (Seoul, Korea 1998)

Appendix I – Test result under Forced Update Rate of 132

Table I Comparison of PSNR of luminance (Y) component between ER and baseline mode (a) WCDMA pattern, (b) Rayleigh pattern

(a)

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 20.31 | 20.51 | 21.48 | 24.81 | 24.74 | 25.03 |
|  | Anchor | 16.71 | 17.71 | 17.70 | 23.39 | 22.71 | 23.79 |
|  | Diff | 3.60 | 2.80 | 3.78 | 1.42 | 2.03 | 1.24 |
| hall | ER | 25.95 | 27.08 | 26.36 | 31.44 | 31.04 | 30.89 |
|  | Anchor | 21.43 | 22.59 | 22.15 | 29.26 | 29.33 | 29.66 |
|  | Diff | 4.52 | 4.49 | 4.21 | 2.18 | 1.71 | 1.23 |
| container | ER | 26.03 | 27.12 | 27.27 | 32.50 | 32.31 | 32.01 |
|  | Anchor | 21.19 | 21.88 | 21.91 | 30.51 | 30.15 | 30.33 |
|  | Diff | 4.84 | 5.24 | 5.36 | 1.99 | 2.16 | 1.68 |
| news | ER | 23.02 | 24.73 | 23.88 | 29.45 | 29.13 | 29.11 |
|  | Anchor | 18.51 | 18.92 | 19.23 | 26.56 | 25.20 | 27.10 |
|  | Diff | 4.51 | 5.81 | 4.65 | 2.89 | 3.93 | 2.01 |
| silent | ER | 25.61 | 26.12 | 25.46 | 30.50 | 30.48 | 30.72 |
|  | Anchor | 23.58 | 23.71 | 24.39 | 28.81 | 29.05 | 29.23 |
|  | Diff | 2.03 | 2.41 | 1.07 | 1.69 | 1.43 | 1.49 |
| glasgow | ER | 19.68 | 20.29 | 20.64 | 23.15 | 23.02 | 23.15 |
|  | Anchor | 17.91 | 18.29 | 18.63 | 22.52 | 22.60 | 22.72 |
|  | Diff | 1.77 | 2.00 | 2.01 | 0.63 | 0.42 | 0.43 |

(b)

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 16.02 | 16.27 | 23.84 | 21.95 |
|  | Anchor | 12.71 | 17.61 | 21.09 | 24.36 |
|  | Diff | 3.31 | -1.34 | 2.75 | -2.41 |
| silent | ER | 17.76 | 17.06 | 23.80 | 20.58 |
|  | Anchor | 14.24 | 16.15 | 22.78 | 25.29 |
|  | Diff | 3.52 | 0.91 | 1.02 | -4.71 |
| news | ER | 17.14 | 16.30 | 22.68 | 23.57 |
|  | Anchor | 13.50 | 14.56 | 17.19 | 19.59 |
|  | Diff | 3.64 | 1.74 | 5.49 | 3.98 |
| container | ER | 17.27 | 18.85 | 24.72 | 25.33 |
|  | Anchor | 13.10 | 16.05 | 20.14 | 22.71 |
|  | Diff | 4.17 | 2.80 | 4.58 | 2.62 |

Table II Comparison of PSNR of chrominance (Cb) component between ER and baseline mode (a) WCDMA pattern, (b) Rayleigh pattern

(a)

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 34.00 | 33.92 | 34.03 | 36.09 | 36.02 | 36.14 |
|  | Anchor | 31.22 | 31.59 | 31.92 | 35.43 | 35.37 | 35.60 |
|  | Diff | 2.78 | 2.33 | 2.11 | 0.66 | 0.65 | 0.54 |
| hall | ER | 36.91 | 39.07 | 38.78 | 40.77 | 40.82 | 40.70 |
|  | Anchor | 32.77 | 34.92 | 33.89 | 39.00 | 39.50 | 39.59 |
|  | Diff | 4.14 | 4.15 | 4.89 | 1.77 | 1.32 | 1.11 |
| container | ER | 39.07 | 39.17 | 39.01 | 40.74 | 40.65 | 40.67 |
|  | Anchor | 34.53 | 34.98 | 34.45 | 39.47 | 39.86 | 39.64 |
|  | Diff | 4.54 | 4.19 | 4.56 | 1.27 | 0.79 | 1.03 |
| news | ER | 31.41 | 33.14 | 30.65 | 36.90 | 37.00 | 36.36 |
|  | Anchor | 28.53 | 28.01 | 28.97 | 35.96 | 35.29 | 35.72 |
|  | Diff | 2.88 | 5.13 | 1.68 | 0.94 | 1.71 | 0.64 |
| silent | ER | 36.20 | 36.81 | 35.96 | 37.70 | 37.70 | 37.75 |
|  | Anchor | 33.53 | 34.64 | 33.99 | 36.63 | 36.61 | 36.83 |
|  | Diff | 2.67 | 2.17 | 1.97 | 1.07 | 1.09 | 0.92 |
| glasgow | ER | 32.07 | 32.59 | 32.78 | 34.38 | 34.35 | 34.39 |
|  | Anchor | 29.76 | 30.60 | 30.54 | 33.79 | 34.07 | 34.01 |
|  | Diff | 2.31 | 1.99 | 2.24 | 0.59 | 0.28 | 0.38 |

(b)

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 30.22 | 27.74 | 38.09 | 36.90 |
|  | Anchor | 22.42 | 28.76 | 32.62 | 36.02 |
|  | Diff | 7.80 | -1.02 | 5.47 | 0.88 |
| silent | ER | 30.79 | 31.46 | 34.72 | 34.00 |
|  | Anchor | 29.67 | 23.19 | 32.83 | 34.39 |
|  | Diff | 1.12 | 8.27 | 1.89 | -0.39 |
| news | ER | 22.29 | 23.60 | 26.77 | 28.71 |
|  | Anchor | 19.58 | 23.09 | 28.72 | 31.03 |
|  | Diff | 2.71 | 0.51 | -1.95 | -2.32 |
| container | ER | 31.55 | 34.61 | 37.63 | 37.68 |
|  | Anchor | 23.16 | 26.27 | 34.20 | 35.77 |
|  | Diff | 8.39 | 8.34 | 3.43 | 1.91 |

Table III Comparison of PSNR of chrominance (Cr) component between ER and baseline mode (a) WCDMA pattern, (b) Rayleigh pattern

(a)

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 33.67 | 33.75 | 34.22 | 36.34 | 36.39 | 36.46 |
|  | Anchor | 30.26 | 31.27 | 31.45 | 35.51 | 35.31 | 35.84 |
|  | Diff | 3.41 | 2.48 | 2.77 | 0.83 | 1.08 | 0.62 |
| hall | ER | 41.76 | 42.68 | 42.57 | 42.76 | 43.37 | 43.23 |
|  | Anchor | 36.70 | 37.74 | 38.14 | 42.23 | 42.02 | 42.36 |
|  | Diff | 5.06 | 4.94 | 4.43 | 0.53 | 1.35 | 0.87 |
| container | ER | 38.20 | 38.61 | 38.10 | 40.05 | 40.03 | 39.85 |
|  | Anchor | 34.18 | 35.34 | 34.31 | 38.72 | 39.29 | 38.98 |
|  | Diff | 4.02 | 3.27 | 3.79 | 1.33 | 0.74 | 0.87 |
| news | ER | 35.50 | 36.58 | 35.52 | 38.48 | 38.13 | 38.19 |
|  | Anchor | 32.52 | 32.35 | 33.03 | 37.57 | 37.24 | 37.62 |
|  | Diff | 2.98 | 4.23 | 2.49 | 0.91 | 0.89 | 0.57 |
| silent | ER | 37.79 | 38.01 | 38.15 | 38.85 | 38.82 | 38.87 |
|  | Anchor | 34.82 | 36.47 | 36.30 | 37.85 | 38.04 | 38.24 |
|  | Diff | 2.97 | 1.54 | 1.85 | 1.00 | 0.78 | 0.63 |
| glasgow | ER | 35.65 | 35.92 | 36.00 | 37.03 | 36.93 | 36.99 |
|  | Anchor | 33.85 | 34.55 | 34.51 | 36.52 | 36.76 | 36.74 |
|  | Diff | 1.80 | 1.37 | 1.49 | 0.51 | 0.17 | 0.25 |

(b)

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 36.00 | 30.37 | 40.89 | 40.42 |
|  | Anchor | 20.85 | 31.86 | 36.90 | 39.28 |
|  | Diff | 15.15 | -1.49 | 3.99 | 1.14 |
| silent | ER | 32.11 | 34.40 | 36.61 | 36.44 |
|  | Anchor | 31.55 | 23.25 | 35.17 | 36.22 |
|  | Diff | 0.56 | 11.15 | 1.44 | 0.22 |
| news | ER | 28.37 | 29.13 | 31.93 | 33.27 |
|  | Anchor | 22.83 | 26.65 | 31.96 | 33.51 |
|  | Diff | 5.54 | 2.48 | -0.03 | -0.24 |
| container | ER | 32.26 | 33.97 | 36.38 | 36.67 |
|  | Anchor | 23.86 | 25.66 | 33.36 | 35.38 |
|  | Diff | 8.40 | 8.31 | 3.02 | 1.29 |

Number of dropped frames

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 45 | 42 | 33 | 2 | 5 | 3 |
|  | Diff | 45 | 42 | 33 | 2 | 5 | 3 |
| hall | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 63 | 53 | 61 | 5 | 11 | 4 |
|  | Diff | 63 | 53 | 61 | 5 | 11 | 4 |
| container | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 53 | 53 | 53 | 53 | 53 | 53 |
|  | Diff | 53 | 53 | 53 | 53 | 53 | 53 |
| news | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 47 | 47 | 47 | 8 | 3 | 9 |
|  | Diff | 47 | 47 | 47 | 8 | 3 | 9 |
| silent | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 62 | 58 | 58 | 4 | 4 | 5 |
|  | Diff | 62 | 58 | 58 | 4 | 4 | 5 |
| glasgow | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 68 | 50 | 43 | 6 | 8 | 3 |
|  | Diff | 68 | 50 | 43 | 6 | 8 | 3 |

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 0 | 229 | 0 | 7 |
|  | Anchor | 634 | 455 | 101 | 67 |
|  | Diff | 634 | 226 | 101 | 60 |
| silent | ER | 27 | 381 | 0 | 9 |
|  | Anchor | 623 | 535 | 111 | 57 |
|  | Diff | 596 | 154 | 111 | 48 |
| news | ER | 0 | 239 | 0 | 4 |
|  | Anchor | 621 | 501 | 93 | 62 |
|  | Diff | 621 | 262 | 93 | 58 |
| container | ER | 0 | 195 | 0 | 4 |
|  | Anchor | 653 | 507 | 107 | 70 |
|  | Diff | 653 | 312 | 107 | 66 |

Appendix II – Test result under Forced Update Rate of 5

Table I Comparison of PSNR of luminance (Y) component between ER and baseline mode (a) WCDMA pattern, (b) Rayleigh pattern

(a)

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 23.15 | 23.52 | 23.78 | 24.98 | 24.99 | 25.03 |
|  | Anchor | 17.72 | 21.36 | 21.73 | 24.61 | 24.55 | 24.67 |
|  | Diff | 5.43 | 2.16 | 2.05 | 0.37 | 0.44 | 0.36 |
| hall | ER | 29.59 | 29.83 | 30.16 | 31.22 | 31.27 | 31.33 |
|  | Anchor | 25.74 | 26.77 | 26.81 | 30.72 | 30.72 | 30.76 |
|  | Diff | 3.85 | 3.06 | 3.35 | 0.50 | 0.55 | 0.57 |
| container | ER | 30.99 | 31.05 | 31.46 | 32.52 | 32.48 | 32.47 |
|  | Anchor | 27.46 | 28.22 | 25.66 | 32.02 | 31.90 | 31.99 |
|  | Diff | 3.53 | 2.83 | 5.80 | 0.50 | 0.58 | 0.48 |
| news | ER | 26.99 | 25.97 | 28.32 | 29.59 | 29.49 | 29.65 |
|  | Anchor | 22.17 | 22.97 | 23.62 | 28.19 | 28.12 | 28.53 |
|  | Diff | 4.82 | 3.00 | 4.70 | 1.40 | 1.37 | 1.12 |
| silent | ER | 28.84 | 29.27 | 29.00 | 30.29 | 30.26 | 30.28 |
|  | Anchor | 24.70 | 25.15 | 26.07 | 29.72 | 28.85 | 25.98 |
|  | Diff | 4.14 | 4.12 | 2.93 | 0.57 | 1.41 | 4.30 |
| glasgow | ER | 20.67 | 21.19 | 21.30 | 22.82 | 22.80 | 22.94 |
|  | Anchor | 18.78 | 19.96 | 20.43 | 21.45 | 22.61 | 22.76 |
|  | Diff | 1.89 | 1.23 | 0.87 | 1.37 | 0.19 | 0.18 |

(b)

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 19.95 | 20.61 | 27.15 | 26.03 |
|  | Anchor | 14.07 | 17.25 | 21.64 | 24.50 |
|  | Diff | 5.88 | 3.36 | 5.51 | 1.53 |
| silent | ER | 20.62 | 19.87 | 26.09 | 22.37 |
|  | Anchor | 15.32 | 17.05 | 23.96 | 25.26 |
|  | Diff | 5.30 | 2.82 | 2.13 | -2.89 |
| news | ER | 18.45 | 17.72 | 24.84 | 26.26 |
|  | Anchor | 13.74 | 15.19 | 19.13 | 20.36 |
|  | Diff | 4.71 | 2.53 | 5.71 | 5.90 |
| container | ER | 22.11 | 24.83 | 28.10 | 28.71 |
|  | Anchor | 16.06 | 20.52 | 24.35 | 26.43 |
|  | Diff | 6.05 | 4.31 | 3.75 | 2.28 |

Table II Comparison of PSNR of chrominance (Cb) component between ER and baseline mode (a) WCDMA pattern, (b) Rayleigh pattern

(a)

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 35.41 | 35.49 | 35.59 | 36.00 | 35.98 | 35.96 |
|  | Anchor | 31.71 | 34.19 | 34.37 | 35.82 | 35.80 | 35.84 |
|  | Diff | 3.70 | 1.30 | 1.22 | 0.18 | 0.18 | 0.12 |
| hall | ER | 39.32 | 39.52 | 39.52 | 40.06 | 40.05 | 40.14 |
|  | Anchor | 35.91 | 36.82 | 35.84 | 39.80 | 39.86 | 39.87 |
|  | Diff | 3.41 | 2.70 | 3.68 | 0.26 | 0.19 | 0.27 |
| container | ER | 39.34 | 39.44 | 39.51 | 39.74 | 39.78 | 39.78 |
|  | Anchor | 37.42 | 37.64 | 36.68 | 39.57 | 39.39 | 39.47 |
|  | Diff | 1.92 | 1.80 | 2.83 | 0.17 | 0.39 | 0.31 |
| news | ER | 35.54 | 33.26 | 35.99 | 37.19 | 37.16 | 37.20 |
|  | Anchor | 28.43 | 30.18 | 30.33 | 36.42 | 35.32 | 35.64 |
|  | Diff | 7.11 | 3.08 | 5.66 | 0.77 | 1.84 | 1.56 |
| silent | ER | 36.55 | 36.61 | 36.69 | 36.97 | 36.98 | 36.97 |
|  | Anchor | 34.68 | 34.34 | 35.69 | 36.82 | 36.74 | 36.61 |
|  | Diff | 1.87 | 2.27 | 1.00 | 0.15 | 0.24 | 0.36 |
| glasgow | ER | 32.83 | 33.28 | 33.39 | 34.40 | 34.45 | 34.50 |
|  | Anchor | 31.06 | 32.10 | 32.38 | 33.34 | 34.28 | 34.34 |
|  | Diff | 1.77 | 1.18 | 1.01 | 1.06 | 0.17 | 0.16 |

(b)

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 33.41 | 33.26 | 38.45 | 38.46 |
|  | Anchor | 20.76 | 27.07 | 33.74 | 35.50 |
|  | Diff | 12.65 | 6.19 | 4.71 | 2.96 |
| silent | ER | 32.37 | 33.27 | 35.42 | 34.57 |
|  | Anchor | 26.21 | 27.72 | 34.25 | 32.26 |
|  | Diff | 6.16 | 5.55 | 1.17 | 2.31 |
| news | ER | 23.71 | 28.57 | 29.75 | 34.95 |
|  | Anchor | 21.00 | 21.01 | 25.28 | 23.67 |
|  | Diff | 2.71 | 7.56 | 4.47 | 11.28 |
| container | ER | 35.69 | 37.32 | 38.13 | 38.31 |
|  | Anchor | 26.90 | 32.97 | 36.13 | 37.33 |
|  | Diff | 8.79 | 4.35 | 2.00 | 0.98 |

Table III Comparison of PSNR of chrominance (Cr) component between ER and baseline mode (a) WCDMA pattern, (b) Rayleigh pattern

(a)

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 35.72 | 35.82 | 35.89 | 36.42 | 36.41 | 36.37 |
|  | Anchor | 31.56 | 34.16 | 34.44 | 36.21 | 36.16 | 36.23 |
|  | Diff | 4.16 | 1.66 | 1.45 | 0.21 | 0.25 | 0.14 |
| hall | ER | 41.89 | 41.93 | 42.10 | 42.30 | 42.27 | 42.33 |
|  | Anchor | 39.13 | 39.52 | 39.1 | 42.03 | 42.06 | 42.12 |
|  | Diff | 2.76 | 2.41 | 3 | 0.27 | 0.21 | 0.21 |
| container | ER | 38.79 | 38.87 | 38.88 | 39.12 | 39.19 | 39.19 |
|  | Anchor | 37.21 | 37.37 | 36.14 | 39.04 | 38.89 | 38.97 |
|  | Diff | 1.58 | 1.5 | 2.74 | 0.08 | 0.3 | 0.22 |
| news | ER | 37.49 | 36.37 | 37.80 | 38.26 | 38.20 | 38.25 |
|  | Anchor | 33.5 | 34.52 | 34.67 | 37.32 | 37.14 | 37.24 |
|  | Diff | 3.99 | 1.85 | 3.13 | 0.94 | 1.06 | 1.01 |
| silent | ER | 37.97 | 37.97 | 38.05 | 38.37 | 38.36 | 38.27 |
|  | Anchor | 36.34 | 36.84 | 37.16 | 38.2 | 38.22 | 38.23 |
|  | Diff | 1.63 | 1.13 | 0.89 | 0.17 | 0.14 | 0.04 |
| glasgow | ER | 36.26 | 36.52 | 36.60 | 37.16 | 37.17 | 37.21 |
|  | Anchor | 34.75 | 35.65 | 35.87 | 36.3 | 37.08 | 37.17 |
|  | Diff | 1.51 | 0.87 | 0.73 | 0.86 | 0.09 | 0.04 |

(b)

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 37.13 | 36.68 | 40.96 | 40.98 |
|  | Anchor | 23.07 | 28.90 | 38.31 | 38.30 |
|  | Diff | 14.06 | 7.78 | 2.65 | 2.68 |
| silent | ER | 34.78 | 35.79 | 37.01 | 37.01 |
|  | Anchor | 27.24 | 28.33 | 34.74 | 36.85 |
|  | Diff | 7.54 | 7.46 | 2.27 | 0.16 |
| news | ER | 29.62 | 32.79 | 34.90 | 36.64 |
|  | Anchor | 25.84 | 24.47 | 30.93 | 25.21 |
|  | Diff | 3.78 | 8.32 | 3.97 | 11.43 |
| container | ER | 34.74 | 36.54 | 37.34 | 37.48 |
|  | Anchor | 26.98 | 33.17 | 35.40 | 36.69 |
|  | Diff | 7.76 | 3.37 | 1.94 | 0.79 |

Number of dropped frames

| Sequence | Mode | WCDMA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 |
| foreman | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 53 | 53 | 39 | 7 | 10 | 8 |
|  | Diff | 53 | 53 | 39 | 7 | 10 | 8 |
| hall | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 45 | 45 | 45 | 45 | 45 | 45 |
|  | Diff | 45 | 45 | 45 | 45 | 45 | 45 |
| container | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 67 | 41 | 50 | 4 | 8 | 3 |
|  | Diff | 67 | 41 | 50 | 4 | 8 | 3 |
| news | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 62 | 63 | 60 | 7 | 10 | 6 |
|  | Diff | 62 | 63 | 60 | 7 | 10 | 6 |
| silent | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 61 | 54 | 46 | 12 | 6 | 7 |
|  | Diff | 61 | 54 | 46 | 12 | 6 | 7 |
| glasgow | ER | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Anchor | 48 | 45 | 40 | 6 | 7 | 3 |
|  | Diff | 48 | 45 | 40 | 6 | 7 | 3 |

| Sequence | Mode | 10dB |  | 20dB |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 14km/h | 1.4km/h | 14km/h | 1.4km/h |
| hall | ER | 0 | 262 | 0 | 2 |
|  | Anchor | 1261 | 981 | 198 | 143 |
|  | Diff | 1261 | 719 | 198 | 141 |
| silent | ER | 30 | 382 | 0 | 7 |
|  | Anchor | 625 | 501 | 127 | 59 |
|  | Diff | 595 | 119 | 127 | 52 |
| news | ER | 0 | 216 | 0 | 3 |
|  | Anchor | 644 | 463 | 89 | 71 |
|  | Diff | 644 | 247 | 89 | 68 |
| container | ER | 0 | 268 | 0 | 0 |
|  | Anchor | 615 | 488 | 114 | 62 |
|  | Diff | 615 | 220 | 114 | 62 |