| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Seventh Meeting: Monterey, 16-19 February, 1999 | Document Q15-G-32 Filename: q15g32.doc Generated: 11 Feb ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Michael Gallant and Faouzi Kossentini University of British Columbia 2356 Main Mall Vancouver, BC, Canada V6T 1Z4 | Tel: Fax: Email: | (604) 822 4988 (604) 822 5949 mikeg@ee.ubc.ca |
| Title: | Inclusion of chrominance distortion for high complexity mode of test model |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Summary

This document proposes a small modification to the high complexity mode decision algorithm described in the Test Model.

The high complexity mode decision described in Section 5.1.3 of the current revision of the Test Model defines the Lagrangian functional as

J = D + l MODE R

The distortion, D, is defined as the sum of squared difference (SSD) between the luminance component coefficients of the target macroblock and its corresponding reconstructed luminance component coefficients.

The SSD cost does not include the distortion for the chrominance blocks. Subjective assessment revealed noticeable color artifacts for some sequences at very low bit rates, particularly for the News anchors at QP values of 25, 15, and 10. Modifying the SSD to include the distortion for chrominance blocks eliminated the particular artifacts that were noted, increased the chrominance PSNR, and did not significantly increase the average bit rate.

We therefore propose that the SSD be defined as the sum of squared difference between the luminance and chrominance component coefficients of the target macroblock and the corresponding reconstructed luminance and chrominance component coefficients.

# Results

We compare results for two sequences with the anchors generated in July 1998. The simulation conditions are the same as used for the anchors, specified in document Q15-D-62. The TMN-10 high complexity coder is modified to include the distortion for the chrominance blocks in the SSD.

| News, QCIF, SKIP=2, Average Bit-Rate and PSNR |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Anchors |  |  |  |  | With Chrominance |  |  |  |
| QP | Rate [kbps] |  | Y [dB] | Cr [dB] | Cb [dB] | Rate [kbps] |  | Y [dB] | Cr [dB] | Cb [dB] |
| 4 | 96.56 |  | 38.29 | 41.28 | 41.92 | 96.05 |  | 38.32 | 41.49 | 42.16 |
| 5 | 73.10 |  | 36.53 | 39.94 | 40.74 | 72.73 |  | 36.56 | 40.08 | 40.87 |
| 7 | 51.43 |  | 34.45 | 39.42 | 40.14 | 51.01 |  | 34.46 | 39.54 | 40.24 |
| 10 | 33.82 |  | 32.45 | 37.35 | 38.24 | 33.75 |  | 32.47 | 37.50 | 38.33 |
| 15 | 20.83 |  | 30.00 | 36.25 | 37.38 | 20.66 |  | 29.96 | 36.52 | 37.49 |
| 25 | 11.42 |  | 27.16 | 34.36 | 36.20 | 11.30 |  | 27.19 | 35.32 | 36.61 |
| Foreman, QCIF, SKIP=2, Average Bit-Rate and PSNR |  |  |  |  |  |  |  |  |  |  |
|  |  | Anchors |  |  |  |  | With Chrominance |  |  |  |
| QP | Rate [kbps] |  | Y [dB] | Cr [dB] | Cb [dB] | Rate [kbps] |  | Y [dB] | Cr [dB] | Cb [dB] |
| 4 | 159.13 |  | 37.62 | 41.47 | 42.20 | 158.83 |  | 37.65 | 41.63 | 42.32 |
| 5 | 118.42 |  | 36.05 | 40.45 | 41.07 | 118.36 |  | 36.06 | 40.57 | 41.19 |
| 7 | 81.95 |  | 34.13 | 39.94 | 40.37 | 81.80 |  | 34.13 | 39.98 | 40.48 |
| 10 | 54.40 |  | 32.24 | 38.33 | 38.49 | 54.07 |  | 32.23 | 38.39 | 38.56 |
| 15 | 34.17 |  | 29.91 | 37.57 | 37.66 | 33.69 |  | 29.89 | 37.70 | 37.77 |
| 25 | 19.28 |  | 27.33 | 36.69 | 36.54 | 18.90 |  | 27.34 | 36.66 | 36.70 |