| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Seventh Meeting: Monterey, 16-19 February, 1999 | Document Q15-G-41 Filename: q15g41.doc Generated: 18 Feb ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Marko Luomi, Nokia | Tel: Fax: Email: | marko.luomi@research.nokia.com |
| Title: | WCDMA Error Patterns |  |  |
| Purpose: | Information |  |  |

_____________________________

# introduction

As a response to a request set by Q15/SG16, we are providing a set of simulated Wideband CDMA error patterns for error resilience simulations. The channel models used in generating the error patterns are based on the latest air interface specifications by ETSI.  ARIB has similar air interface specification. The major change since summer 1998 has been to replace convolutional codes by turbo codes at bit-rates >= 32 kbps. The reason behind that decision is the fact that turbo codes make the errors more bursty than convolutional codes do, and thus in the same bit error rate, typically better video quality can be achieved. The final parameters for turbo codes are still open. In addition to the bit rates who did not have any error patterns available, i.e., 128 and 384 kbps, the set covers also other important bit rates in third generation networks; 32 and 64 kbps, thus providing up-to-date patterns for 64 kbps, and new type of patterns for 32 kbps.

The format of the patterns is the same as previously; '1' means error, there are eight bits in a byte, and the least significant bit is transmitted first.

# Simulation parameters

Simulation parameters are listed in Table 1. For more information about the listed parameters, see [1].

Table 1. Simulation parameters

| Frequency | 1920 MHz |
| --- | --- |
| Chip rate | 4.096 Mcps |
| Transmission direction | Uplink |
| Multipath profile | ITU Outdoor-to-indoor A |
| Mobile speed | 3 km/h and 50 km/h |
| Bit rate | 32, 64, 128, 384 kbps |
| Interleaving depth | 40 ms |
| DPDCH spreading factor | 32, 16, 8, 4 |
| RLC-PDU = frame with CRC bits. FER is calculated per RLC-PDU | 640 bits |
| Number of RLC-PDUs per interleaving period | 2-24 |
| CRC bits | 16 |
| Coding | 1/3-rate Turbo code, 4 states |
| DPCCH spreading factor | 256 |
| Power difference between DPCCH and DPDCH | 32, 64 and 128 kbps: -6 dB 384 kbps: -9 dB |
| Power control step | 1 dB |
| Power control signalling errors | 0 % |
| RX-antenna diversity | yes |
| Simulation length | 180 s |
| FER targets | 1e-2 4e-3 1e-3 4e-4 |

Table 2 Simulation results 3 km/h

| Bit rate | Interleaving | FER | BER | Eb/N0 |
| --- | --- | --- | --- | --- |
| 384 kbps | 40 ms | 1.5e-2 | 5.1e-3 | -0.6 dB |
|  |  | 4.8e-3 | 7.9e-5 | -0.5 dB |
|  |  | 2.1e-3 | 5.0e-5 | -0.4 dB |
|  |  | 5.1e-4 | 2.2e-6 | -0.3 dB |
| 128 kbps | 40 ms | 1.2e-2 | 5.0e-4 | 0.1 dB |
|  |  | 3.6e-3 | 1.3e-4 | 0.2 dB |
|  |  | 1.5e-3 | 3.1e-5 | 0.3 dB |
|  |  | 5.3e-4 | 5.1e-5 | 0.4 dB |
| 64 kbps | 40 ms | 1.0e-2 | 5.4e-3 | 0.7 dB |
|  |  | 3.2e-3 | 1.6e-4 | 0.8 dB |
|  |  | 1.3e-3 | 7.2e-5 | 0.9 dB |
|  |  | 2.2e-4 | 3.4e-6 | 1.2 dB |
| 32 kbps | 40 ms | 9.8e-3 | 9.1e-4 | 1.6 dB |
|  |  | 2.8e-3 | 3.0e-4 | 1.7 dB |
|  |  | 8.9e-4 | 7.5e-5 | 1.9 dB |
|  |  | 2.2e-4 | 2.1e-5 | 2.1 dB |

Table 3. Simulation results 50 km/h

| Bit rate | Interleaving | FER | BER | Eb/N0 |
| --- | --- | --- | --- | --- |
| 384 kbps | 40 ms | 1.2e-2 | 4.7e-4 | 0.1 dB |
|  |  | 4.0e-3 | 1.4e-4 | 0.3 dB |
|  |  | 1.6e-3 | 4.9e-5 | 0.5 dB |
|  |  | 6.0e-4 | 9.8e-6 | 0.5 dB |
| 128 kbps | 40 ms | 1.2e-2 | 6.3e-4 | 0.6 dB |
|  |  | 3.7e-3 | 2.0e-4 | 0.7 dB |
|  |  | 1.5e-3 | 3.8e-5 | 1.0 dB |
|  |  | 5.3e-4 | 2.7e-6 | 1.2 dB |
| 64 kbps | 40 ms | 1.1e-2 | 6.6e-4 | 1.3 dB |
|  |  | 3.6e-3 | 1.7e-4 | 1.4 dB |
|  |  | 1.3e-3 | 6.0e-5 | 1.7 dB |
|  |  | 6.7e-4 | 3.4e-6 | 1.7 dB |
| 32 kbps | 40 ms | 1.0e-2 | 6.3e-4 | 2.1 dB |
|  |  | 3.1e-3 | 1.7e-4 | 2.3 dB |
|  |  | 1.1e-3 | 5.4e-5 | 2.5 dB |
|  |  | 2.2e-4 | 5.2e-7 | 2.5 dB |

# References

| [1] | UMTS Terrestrial Radio Access from ETSI SMG2, at http://www.itu.int/imt/2-radio-dev/proposals/ |
| --- | --- |