| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Seventh Meeting: Monterey, 16-19 February, 1999 | Document Q15-G-45 Filename: q15g45.doc Generated: 19 Feb ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Gary Sullivan PictureTel Corp. M/S 635 100 Minuteman Rd. Andover, MA 01810 | Tel: Fax: Email: | +1 978 623 4324 +1 978 749 2804 garys@pictel.com |
| Title: | Common Conditions for Latency Measurement Comparisons |  |  |
| Purpose: | Information |  |  |

_____________________________

These are the common conditions for measuring latency as agreed at the Monterey meeting.  As these are our initial attempt at defining such common conditions, they may require refinement as we gain experience with them.

Anchor:

- Use rate control as described in test model document

- Use high complexity motion estimation and mode decision

- Use option configuration DFIJT

- Set QP=20 for the first picture (skip pictures to reduce delay after this picture as described in test model)

Sequences and bit rates:

| Sequence | Resolution | Max Frame Rate (fps) | Channel Bit Rate (kbps) |
| --- | --- | --- | --- |
| Hall | CIF | 10 | 48 |
| Foreman | QCIF | 10 | 48 |
| Glasgow | QCIF | 15 | 144 |
| Irene | CIF | 30 | 256 |

Delay measurement: per Fryer measurement method (Q15-D-43)

Must report both the number of frames coded and the following

quickie metric = (variance of bits per frame)/(square of average bits per frame) +
(variance of time distance between encoded frames)/(square of average time distance between encoded frames)

where the variance sX2of a set of samples Xii=1N is calculated as sX2=1N−1i=1NXi−X2,

and the sample average X is calculated as X=1Ni=1NXi.