UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

16	  999

E

Question: 11/16

SOURCE:	United States of America *

TITLE:	Correction of problems related to MCV in the white draft of H.230

______________________________________________________________________________

### Introduction

There is a problem with the H.243 mechanism for broadcasting a still image (H.261 annex D) . There is no reliable means for a terminal to know when its video signal is being broadcast to all remote sites, so there is a risk that transmission may begin before all sites are ready to receive the still image.  The MIV indication is only an “on-air” signal indication that one or more sites are receiving the video stream.  We propose three new H.230 C&I BAS codes: ‘MVA’ (Multipoint Visualization Achieved) and ‘MVR’ (Multipoint Visualization Refused/Revoked) and an associated ‘MVC’ capability (Multipoint Visualization Capability) to resolve the problem.

These changes were presented to the joint Q11-15/16 Experts meeting in Monterey, CA (Feb. 15-19,1999) and the expert’s group recommended the changes to be accepted.

Please note that corresponding changes are being proposed for H.243, specifying the use of these new codepoints.

### Text to be added to the white draft of H.230

The following clarifications and additions should be incorporated in H.230 (change marks are against current revisions of H.230):

Section 3.5: Add new MVA and MVR messages as unique answers to MCV.  Add a capability MVC to indicate support of these new messages.  After receiving MVA the terminal can transmit a still image (graphic transfer):

| MVC | Multipoint Visualization Capability – Included in the cap-set of an MCU or terminal to show that it can properly generate or process the codes MVA and MVR |
| --- | --- |
| MVA | Multipoint Visualization Achieved – Transmitted by an MCU to indicate that it has granted visualization in response to the command MCV |
| MVR | Multipoint Visualization Refused/Revoked – Transmitted by an MCU when it cannot comply with the command MCV, when visualization status has been withdrawn due to a higher priority switch request, e.g. VCB, or as a response to Cancel-MCV |

Section 3.5: Clarify use of MIV and Cancel-MIV as "On Air"/”Off Air” indications only (change marks are against current H.230):

| MIV | Multipoint Indication Visualization – Transmitted by an MCU to indicate to a terminal that its video signal is being seen by at least one other terminal (otherwise known as "On-air" or “seenByAtLeastOneOther” indication). |
| --- | --- |

We propose the following new BAS code values (to be updated in Table 1/H.230 and Table 4/H.230):

MVC		(010)[12]

MVA		(010)[13]

MVR		(010)[14]

-- END --

* Contact:	Patrick Luthi

Phone: +1 978-623-4354

Fax: +1 978-749-2804

E-mail: luthip@pictel.com