| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eighth Meeting: Berlin, Germany, 3-6 August, 1999 | Document Q15-H-10 Filename: q15h10.doc Generated: 9 July ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | G. Bjontegaard Telenor Satellite Services P.O.Box 6914 St.Olavs plass N-0130 Oslo, Norway | Tel: Fax: Email: | +47 23 13 83 81 +47 22 77 79 80 gisle.bjontegaard@international.telenor.no |
| Title: | Improvements to the Telenor proposal for H.26L: More blocksizes for prediction and RD constrained quantization of transform coefficients. |  |  |
| Purpose: | Proposal/Test model |  |  |

_____________________________

# 1	Use of 7 different blocksizes in motion compensated prediction

So far 3 block sizes have been used for motion compensation: 16x16, 8x8 and 4x4 vectors.    We have added 4 more vector sizes as indicated below.

Mode:    1                            2                          3                        4                        5                        6                          7

Signalling of the new blockstructure for prediction is made by the Mode parameter.    The new mode table is listed below.

| Code no. | Mode |
| --- | --- |
| 0 | Skip |
| 1 | 16x16 |
| 2 | 16x8 |
| 3 | 8x16 |
| 4 | 8x8 |
| 5 | 8x4 |
| 6 | 4x8 |
| 7 | 4x4 |
| 8 |  |
| 9 | Intra |

In the present implementation, separate search is made for all block sizes (see detailed description in previous documentation).    This obviously result in additional complexity on the encoder side.    For the decoder it is assumed that additional blocksizes for prediction comes 'free'.

There are various ways for the encoder to reduce complexity when deciding on which block structure to use.

One of the reasons for implementing the additional block sizes - and distribute the source code - was to obtain information of potential gain with this extension and probably also try other subsets than 16x16, 8x8 and 4x4.

The overall gain by using 7 blocksizes compared to 3 is typically about 1/4 dB (see RD curves).

# 2	RD constrained quantization

## 2.1	Relation between reconstruction- and decision levels

Assume that we quantize each transform coefficient separately.    This is illustrated below.    The coefficient size is indicated by the arrow .    There are two possible reconstruction levels: R0 and R1 connected to the current QP.    The distance between R0 and R1 is d.    If the number of bits for using R0 and R1 is the same, the decision level between the two should be in the middle to minimize distortion (D0 = R0 +d/2).    However, reconstruction to R1 requires more bits than reconstruction to R0 .    Therefore the decision level D0 is located closer to R1 than R0 to minimize the RD function.

R1 -

D0 -                          d

R0 -

In the test model we use a fixed relation between R0 R1 and D0 even if the difference in bits connected to R0 and R1 varies (depending of LEVEL and RUN).

## 2.2	RD constrained quantization

Let me give a simple example of Inter quantization.    If the choice for the first coefficient is LEVEL = 0 or 1,we must assume that LEVEL=1 'costs' 3 bits compared to 0 bits with LEVEL=0.    This will be reflected in the decision between the two (see above).

However, assume that the second coefficient in the scan will (definitely) be quantized to LEVEL=1.    If the first coefficient is quantized to 0 the (LEVEL,RUN) event is (1,1) which is coded with 5 bits.    On the other hand, if the first coefficient is quantized to 1, this will result in two events (1,0) (1,0) which are coded with 3 + 3 = 6 bits.    This is only 1 bit in addition, which should be taken into account when choosing the value of LEVEL for the first coefficient.

## 2.3	Quantization procedure

First I go through all the coefficients and do a sort of prequantization.    If the coefficient to be quantized is between R0 and R0 + 0.6d the reconstruction level is set to R0 . If the coefficient is between and R0 + 0.6d and R1 we still keep the possibility open for both R0 and R1 .    After this procedure, n (typically 0 to 3) coefficients will still have the choice of 2 reconstruction values.    This means that we have m = 2n possible quantization strategies for the complete block.

I then go through the m (typically 1 to 8) possibilities to minimize the rate distortion function:

RD = Bits + lx((Coeff/Q - Level)2 - Coeff/Q)2) = Bits + lxLevelx(Level-2xCoeff/Q)

l is set to 12.    Q is the divisor used in the quantization process (to obtain Level).    If all the m quantization strategies result in positive values for RD, we set all reconstructed coefficients for the block equal to 0.

The procedure may look like a 'brute force' approach.    Still the increase in time to run the encoder was hardly noticeable.    As this procedure is an encoder option, the encoder is obviously free to implement its own 'tricks' to optimize the quantization process.

The overall gain by using this RD constrained quantization is typically about 1/4 dB (see RD curves).

# 3	Rate Distortion curves