| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Ninth Meeting: Red Bank, New Jersey, 19-22 October, 1999 | Document Q15-I-19 Filename: q15i19.doc Generated: 10 Oct. ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Michael Gallant, Guy Côté, and Faouzi Kossentini University of British Columbia 2356 Main Mall Vancouver, BC, Canada V6T 1Z4 Stephan Wenger TU Berlin / TELES AG Franklinstr. 28-29 D-10587 Berlin | Tel: Fax: Email: | (604) 822-4988 (604) 822-5949 {mikeg, guyc, faouzi} @ece.ubc.ca +1-415-297-2063 +1-604-736-7853 stewe@cs.tu-berlin.de |
| Title: | Normative Error Concealment Method: Repeat Macroblock |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

We present results for an error concealment method for inclusion in H.263++ Annex L. The method is similar to the TCON model, except that no motion compensation is performed.

# Description of the error concealment scheme

Data from the previously (temporal) displayed picture is used for concealment of parts of the picture where data is lost. Missing macroblocks in the current frame are concealed by simply repeating the macroblock in the previous displayed frame at the same spatial location.

# Results

Results were generated using the Internet/H.323 common conditions as outlined in Q15-G-39r1 and the source packetization/depacketization simulator of Q15-I-09, with packet loss error patterns in Q15-I-16. All PSNR values are averaged over all frames, including skipped frames where the previous coded frame is repeated.

Two sets of average results are presented in each of Tables 1, 2, and 3 for the Internet packet loss error patterns. In all tables, the first set of results is for the high-complexity mode decision algorithm for error-prone environments described in the Test Model (Q15-G-16), which repeats macroblocks for concealment, as described in the previous section.  The second set of results is also for the high complexity mode decision algorithm for error-prone environments described in the Test Model (Q15-G-16), but uses a different concealment mechanism, based on TCON and described in Q15-I-20.

Table 1 presents results for decoded PSNR after applying the error patterns. In this case, the encoder uses knowledge of the packet loss rate and the decoder error concealment mechanism in its mode decision algorithm. Note that in both cases this should produce different data streams for each different loss situation, except for the zero loss case.  Based on the results, we suspect there is a small problem in the encoding algorithm, as we do not in fact obtain identical streams for the zero loss case. This problem is currently under investigation.

Table 2 also presents results for decoded PSNR after applying the error patterns. In this case, the encoder does not use knowledge of the packet loss rate or the decoder error concealment mechanism to compute the concealment distortion in its mode decision algorithm. This produces the same data stream for both cases for all loss situations. Again, based on these results we suspect there is a small problem in the encoding algorithm, as we do not in fact obtain identical streams for both the repeated macroblock and TCON concealment cases.

Table 3 presents results for decoded PSNR without having applied the error patterns. Both sets of results are from data streams generated by the high-complexity mode decision algorithm for error-prone environments described in the Test Model (Q15-G-16). These results serve as a benchmark against which the corrupted streams can be compared.

A comparison of the first set of results in Tables 1 and 2 illustrates that there are still clear benefits in having a normative mechanism for concealment, where in this case the mechanism simply repeats lost macroblocks. We believe that a simpler alternative to TCON also has merit and should be included as normative content.

| Sequence | Total Bit Rate | Frame Rate | Video Bit Rate | PLR | Decoded and COPY concealed stream (high complexity mode decision with PLR and COPY concealment knowledge) |  |  | Decoded and TCON concealed stream (high complexity mode decision with PLR and TCON concealment knowledge) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Y PSNR | Cb PSNR | Cr PSNR | Y PSNR | Cb PSNR | Cr PSNR |
| Foreman | 64.00 | 7.5 | 59.20 | 0 | 26.25 | 37.62 | 37.97 | 27.49 | 39.21 | 39.29 |
|  | 64.00 | 7.5 | 59.20 | 3 | 24.72 | 37.28 | 37.51 | 24.80 | 37.26 | 37.27 |
|  | 64.00 | 7.5 | 59.20 | 5 | 25.21 | 37.44 | 37.67 | 25.34 | 37.44 | 37.49 |
|  | 64.00 | 7.5 | 59.20 | 10 | 22.91 | 36.72 | 36.79 | 22.92 | 36.72 | 36.53 |
|  | 64.00 | 7.5 | 59.20 | 20 | 21.65 | 36.22 | 36.15 | 21.56 | 36.20 | 36.08 |

Table 1.

| Sequence | Total Bit Rate | Frame Rate | Video Bit Rate | PLR | Decoded and COPY concealed stream (high complexity mode decision without PLR or COPY concealment knowledge) |  |  | Decoded and TCON concealed stream (high complexity mode decision without PLR or TCON concealment knowledge) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Y PSNR | Cb PSNR | Cr PSNR | Y PSNR | Cb PSNR | Cr PSNR |
| Foreman | 64.00 | 7.5 | 59.20 | 0 | 26.25 | 37.62 | 37.97 | 27.49 | 39.21 | 39.29 |
|  | 64.00 | 7.5 | 59.20 | 3 | 23.78 | 36.88 | 36.99 | 24.26 | 37.00 | 36.93 |
|  | 64.00 | 7.5 | 59.20 | 5 | 24.38 | 37.11 | 37.33 | 24.82 | 37.21 | 37.23 |
|  | 64.00 | 7.5 | 59.20 | 10 | 19.93 | 35.19 | 34.77 | 20.84 | 35.54 | 35.05 |
|  | 64.00 | 7.5 | 59.20 | 20 | 17.68 | 33.67 | 33.10 | 18.60 | 34.06 | 33.55 |

Table 2.

| Sequence | Total Bit Rate | Frame Rate | Video Bit Rate | PLR | Encoded stream (high complexity mode decision with PLR and COPY concealment knowledge) |  |  | Encoded stream (high complexity mode decision with PLR and TCON concealment knowledge) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Y PSNR | Cb PSNR | Cr PSNR | Y PSNR | Cb PSNR | Cr PSNR |
| Foreman | 64.00 | 7.5 | 59.20 | 0 | 26.25 | 37.62 | 37.97 | 27.49 | 39.21 | 39.29 |
|  | 64.00 | 7.5 | 59.20 | 3 | 26.11 | 37.60 | 37.93 | 27.54 | 39.06 | 39.23 |
|  | 64.00 | 7.5 | 59.20 | 5 | 26.00 | 37.60 | `37.89 | 27.46 | 38.98 | 39.16 |
|  | 64.00 | 7.5 | 59.20 | 10 | 25.86 | 37.58 | 37.89 | 27.37 | 38.89 | 39.08 |
|  | 64.00 | 7.5 | 59.20 | 20 | 25.67 | 37.54 | 37.82 | 27.23 | 38.73 | 38.94 |

Table 3.