| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Ninth Meeting: Red Bank, New Jersey, 19-22 October, 1999 | Document Q15-I-30 Filename: q15i30.doc Generated: 12 Oct. ’99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Kook-yeol Yoo and Yung-Lyul Lee Information Media Lab. Samsung Electronics 416, Maetan-3Dong Paldal-Gu, Suwon, Kyungki-Do 442-742, Korea | Tel: Fax: Email: | +82-331-200-3674 +82-331-200-3195 mailto:kyoo@mmrnd.sec.samsung.co.kr mailto:yllee@mmrnd.sec.samsung.co.kr |
| Title: | Enhanced Samsung Entropy coding |  |  |
| Purpose: | Proposal |  |  |

_____________________________

- Use of local statistics information

In the Berlin meeting, we presented an DSR (Dynamic Symbol Reordering)-based entropy coding method to enhance Telenor’s proposal (TML-1), only applying the method to MB type and CBP (Coded Block Pattern) [1]. In this proposal, we present a modified and improved version of our previous algorithm and its application to all other syntax elements, except reference picture selection symbol. To get a more accurate symbol probability, the symbol probabilities from the MBs neighboring the current MB is used. So, the proposed algorithm is divided into two stages as follows (See also Fig. 1):

Stage 1: Global DSR, based on the probability measured using several previous frames (the same as [1])

The mapping table is obtained by the following relationship:

bmi≥bmj if pi≤pjfor i,j∈0,L'−1mi=iotherwise                                                 (1)

where bi and mirepresent the number of bits for the i-th symbol by VLC table and mapping table for the i-th symbol, respectively. The parameter L is the total number of symbols in the VLC table and L' satisfies the following relationship:

L'≤L                                                                                        (2)

The probability for i-th symbol is computed by using following equation:

pi=t=n−1n−Wcti/j=0L'−1t=n−1n−Wctj, for i∈0,L'−1                                                 (3)

where, cti represents the number of i-th symbols occurred in the t-th frame. Wis the number of previous frames used in the probability computation.

Stage 2: Local DSR, based on the probability measure using several encoded MBs in the current frames.

The probability in the stage 2 is measured by using the following equation:

pi=cn'mi/j=0L''−1cn'mj,for i∈0,L''−1                                       (4)

where cn'i is number of I-th symbols occurred in the causal portion among the eight-connected neighbor MBs. Tha mapping table m is obtained in the stage 1. And the parameters L' andL''satisfy the following relationship:

L''≤L'                                                                                (4)

We used relatively small L'' since the size of sample space for the stage 2 is small. It should be noted that in the stage 2, the symbol probability is measured for the output symbol of Stage 1.

Global Symbol

Mapper

Local Symbol Mapper

VLC Encoder

Symbol

Encoded

Bitstream

Frame-based

Symbol Counter

MB-based

Symbol Counter

MB Delay

Frame Delay

Stage 1

Stage 2

Fig. 1: Combined Global-Local DSR-based VLC encoder

- Simulation results

For the comparison, we followed the common test condition for H.26L [2] and also we additionally used several other high bitrate sequences to evaluate the proposed algorithm for the high bitrate application, which was issued in the Berlin meeting [3]. In summary, the test condition used in this simulation is as follows:

Table 1: Expanded simulation condition

| QINTER | 4, 5, 7, 10, 15, 25 | 4, 5, 7, 10, 15, 25 |
| --- | --- | --- |
| Frame skip | 2 | 1 |
| Resolution | QCIF | CIF |
| Sequences (as defined in H.26L CFP) | Mother-and-Daughter(MAD) Hall Container Silent Voice News Foreman | Foreman |
| Expanded sequences (for high bitrate application) | Flower Garden Carphone Stefan Costguard Mobile & Calendar(Mob&Cal) | Flower Garden Carphone News Hall Costguard |

  - Improvement over previous Samsung proposal for reference

To know the improvement of this modification over the previous Samsung proposal, Berlin, we compared the performance measured in relative to TML-1 algorithm. Note that Berlin version only applied global DSR algorithm to MB type and CBP and the current version applied global-local DSR to all the syntax elements. The Figure 2 shows that the modified version gives additionally 6% reduction in total bitrate, while the previous Samsung proposal gives only 1.81% bitrate reduction over the TML-1 algorithm.

  - Bitrate reduction (for a given PSNR)

The bitrate reduction performance of proposed method over TML-1 in percentage (%) is given in Table 1 The sequences giving more reduction than average ratio are marked in bold faces, showing that the proposed method is more effective for relatively high bitrate or complex sequences. Note that Hall sequence has only small moving objects and static background for the most part of each image, thereby the statistics is dominated by the edge components. So, the proposed method which is adaptive to local statistics gives good results, though the sequence is not high bitrate sequences. In Average, proposed method gives 8.29% and 8.88% reduction for QCIF and CIF sequences, respectively.

Table 2: Bitrate reduction relative to TML-1 (%)

|  | QCIF | CIF |
| --- | --- | --- |
| Container | 6.08 | NA |
| Hall | 9.26 | 8.67 |
| Foreman | 7.76 | 7.78 |
| News | 8.16 | 7.40 |
| Silent Voice | 7.58 | NA |
| Stefan | 11.03 | NA |
| Flower Garden | 9.34 | 12.67 (SIF) |
| Carphone | 8.09 | 7.71 |
| Costguard | 7.96 | 9.02 |
| Mob&Cal | 9.13 | NA |
| MAD | 6.77 | NA |
| Average | 8.29 | 8.88 |

  - Rate-Distortion curves for object quality evaluation

Fig. 3: R-D curves for various sequences (QCIF, 10Hz) (continue)

Fig. 3: R-D curves for various sequences (QCIF, 10Hz)

Fig. 4: R-D curves for various sequences (CIF)

From the Figs. 3 and 4, we can observe obvious improvement in PSNR for a given PSNR. The improvements are dependent on the sequence characteristics, as previous explained in the previous section (Table 2). For the complex sequences, such as ‘Flower Garden’ and ‘Stefan’, etc, the proposed method gives about 2dB improvement in PSNR for the smallest quantization stepsize. For the relatively simple sequences, the proposed method still gives about 0.5dB improvement. From the comparison for CIF (or SIF) and QCIF sequences, we observed no significant performance differences.

# Summary

In this proposal, we present modified DSR-based entropy coding by using local statistics of syntax elements. For the more extensive comparison, we used more sequences than the ones in the common test condition. The simulation results show that the proposed algorithm gives about 8.3 and 8.9% bitrate reduction over TML-1 method for QCIF and CIF sequences, respectively. And the proposed method is highly effective for the high bitrate applications. From these results, we believe that the proposed method can be a powerful tool, which enables TML-1 to achieve 50% bitrate reduction over H.263++.

# References

- Kook-yeol Yoo, Byung Sun Choi, and Yung-Lyul Lee, “Improvements to the Telenor proposal for H.26L: Preliminary results on Dynamic Symbol Reordering (DSR) method for Universal VLC encoding / decoding,” ITU-T Q.15/SG16 Doc. # Q15-H-19, Berlin, 3-6 August, 1999

- G. Sullivan and K. Hibi, “Draft call for proposal for H.26L video coding,” ITU-T Q.15/16, Geneva, Jan. 1998.

- G. Sullivan, “Draft Meeting Report of the Eighth Meeting (Meeting H) of the ITU-T Q.15/16,” ITU-T Q.15/16 Doc. # Q15-H-37d 1, Berlin, 3-6 August 1999.