| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eighth Meeting: Berlin, Germany, 3-6 August, 1999 | Document Q15-I-44 Filename: q15i44.doc Generated: 4. Aug ‘99 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wiegand Telecommunications Laboratory University Erlangen-Nuremberg Germany | Tel: Fax: Email: | (9131) 85-27117 (Germany) (9131) 85-28849 (Germany) wiegand@nt.e-technik.uni-erlangen.de |
| Title: | Proposed Changes to Draft for Annex U on Enhanced Reference Picture Selection |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Summary

This document presents a proposed draft for Annex U specifying the optional Enhanced Reference Picture Selection modeThe proposed changes are highlighted.

Annex U
Enhanced Reference Picture Selection mode

(This annex forms an integral part of this Recommendation.)

## U.1	Introduction

This annex describes the optional Enhanced Reference Picture Selection mode of H.263. The capability of this mode of H.263 is signaled by external means (for example Recommendation H.245). The amount of additional picture memory accommodated in the decoder is also signaled by external means to help the memory management at the encoder. The use of this mode is indicated in the PLUSPTYPE field of the picture header. The mode provides benefits for both error resilience and coding efficiency.

For error resilience, it can use backward channel messages, which are defined and signaled by external means (e.g. Recommendation H.245) sent from a decoder to an encoder to inform the encoder which part of which pictures have been incorrectly decoded..

For coding efficiency, motion compensation can be extended to multi-frame prediction. The extension of motion compensation to multi-frame prediction is achieved by extending the motion vector by a picture reference parameter which may be used to address a block for motion compensation in each of the multiple reference pictures. The picture reference parameter is a variable length code specifying a buffer index. For that, the reference pictures are assembled in a buffering scheme that can be controlled by the encoder. The combination of the buffering scheme with the error resilience capabilities enables the use of Reference Picture Selection (Annex N). Moreover, a CRC is specified to detect buffer misalignment between encoder and decoder. The specifications determine the extensions to the syntax for the frame buffering scheme and the macroblock syntax.

Annex U contains one sub-mode called Reference Picture Pruning. The purpose of Reference Picture Pruning is to reduce on average, the amount of memory required to save multiple reference pictures. The memory reduction is accomplished by partitioning each reference picture into sub-frames. While encoding, the encoder may discard (“prune”) sub-frames from reference pictures in the multi-frame buffer. A detailed description of the Reference Picture Pruning sub-mode may be found in Section U.5.

## U.2	Video source coding algorithm

The source coder of this mode is shown in generalized form in Figure U.1. This figure shows a structure which uses a number of picture memories.

The video source coding algorithm can be extended to multi-frame motion compensation. Enhanced coding efficiency may be achieved by allowing reference picture selection on the macroblock level. A picture buffering scheme with relative indexing is employed for efficient addressing of pictures in the multi-frame buffer. The multi-frame buffer control may work in several modes of operation. For example, a sliding window over time can be accommodated by the buffer control unit. Past decoded and reconstructed pictures starting with the immediately preceding one ending with the picture which is decoded M time instants before are collected in the picture memories PM0... PMM-1. If the number of pictures maximally accommodated by the multiple reference picture buffer corresponds to M, the motion estimation when coding picture m, if 0£m£M-1, can utilize m pictures. In case m³M, the maximum number of pictures M can be used. Alternatively, an adaptive buffer control can be utilized.

The source coder may select one or several of the picture memories to suppress temporal error propagation caused by inter-frame coding. The Independent Segment Decoding mode (see Annex R), which treats boundaries of GOBs with non-empty headers or slices as picture boundaries, can be used to avoid spatial error propagation due to motion compensation across the boundaries of the GOBs or slices when this mode is applied to a smaller unit than a picture, such as a GOB or slice. The information to signal which picture is selected for prediction is included in the encoded bitstream.

The strategy used by the encoder to select the picture to be used for prediction is out of the scope of this Recommendation.

FIGURE U.1/H.263

Source coder for Enhanced Reference Picture Selection

## U.3	Back-Channel messages

### An out-of-band channel, which does not have to be reliable, can be used to convey backchannel messages. The syntax of this out-of-band channel (which will usually be a separate logical channel of H.223 or H.225.0) should be the one defined in Annex N.

## U.4	Syntax

The syntax is altered in the Picture, Group of Blocks (GOB), slice, and macroblock layer. On the Picture, GOB, and Slice layer, an Enhanced Reference Picture Selection layer (ERPS layer) is optionally inserted. Furthermore, a TR check is optionally inserted at the Picture or GOB or Slice layer as well.

The syntax for the PLUS header is shown in Figure U.2. The fields of ERPSI, TRPI, TRP, ERPS, and TRCI are inserted into the PLUS header. The fields ERPSI, TRPI, TRP, ERPS, and TRCI are added to Figure 8/H.263.

FIGURE U.2/H.263

Structure of PLUS Header.

The syntax for the Picture is shown in Figure U.3. The field of TRC is inserted into the Picture layer. The field TRC is added to Figure 7/H.263.

FIGURE U.3/H.263

Structure of Picture layer.

The syntax for the GOB layer is shown in Figure U.4. The fields of ERPSI, TRI, TR, TRPI, TRP, ERPS, TRCI, and TRC are added to Figure 9/H.263.

FIGURE U.4/H.263

Structure of Gob layer.

When the optional Slice Structured mode (see Annex K) is in use, the syntax of the slice layer is modified in the same way as the GOB layer. The syntax for the Slice layer is shown in Figure U.5.

FIGURE U.5/H.263

Structure of Slice layer.

The ERPS layer is shown in Figure U.6.

NRPA

RPBS

NRI

RPS

RPB

RPP

API

APP

RPI

PII

SPI

NPP

SPB

RPS

SDI

HSD

VSD

FIGURE U.6/H.263

ERPS layer.

Variable length codes for the NRPA, NRI, RPS, RPP, APP, NPP, and PR fields are given in Table U.1.

Table U.1/H.263

Variable length codes for NRPA, NRI, RPS, RPP, APP, NPP, and PR.

| Absolute position | number of bits | Codes |
| --- | --- | --- |
| 0 | 1 | 1 |
| ‘x0’+1 (1:2) | 3 | 0x00 |
| ‘x1x0’+3 (3:6) | 5 | 0x11x00 |
| ‘x2x1x0’+7 (7:14) | 7 | 0x21x11x00 |
| ‘x3x2x1x0’+15 (15:30) | 9 | 0x31x21x11x00 |
| ‘x4x3x2x1x0’+31 (31:62) | 11 | 0x41x31x21x11x00 |
| ‘x5x4x3x2x1x0’+63 (63:126) | 13 | 0x51x41x31x21x11x00 |
| ‘x6x5x4x3x2x1x0’+127 (127:254) | 15 | 0x61x51x41x31x21x11x00 |
| ‘x7x6x5x4x3x2x1x0’+255 (255:510) | 17 | 0x71x61x51x41x31x21x11x00 |
| ‘x8x7x6x5x4x3x2x1x0’+511 (511:1022) | 19 | 0x81x71x61x51x41x31x21x11x00 |
| ‘x9x8x7x6x5x4x3x2x1x0’+1023 (1023:2046) | 21 | 0x91x81x71x61x51x41x31x21x11x00 |
| ‘x10x9x8x7x6x5x4x3x2x1x0’+2047 (2047:4094) | 23 | 0x101x91x81x71x61x51x41x31x21x11x00 |

#### U.4.1.1	Enhanced Reference Picture Selection layer Indicator (ERPSI) (1 bit)

ERPSI is a bit that is present only if Enhanced Reference Picture Selection is indicated in the PLUS header. ERPSI indicates whether the ERPS layer is present. If ERPSI is set to ‘1’, the ERPS layer is present. If ERPSI is set to ‘0’, the ERPS layer is not present. When the ERPS layer is not present, the default Annex U operates in Reference Picture Selection mode (Annex N) but without VideoMux back-channel. The fields TRI, TR, TRPI, and TRP are used as in Annex N of this Recommendation. The decoder operates as specified in Annex N. The macroblock syntax remains unaltered to Annex N.

U.4.1.2	Temporal Reference Indicator (TRI) (1 bit)

A code word that is only present on the GOB or Slice layer. TRI indicates whether or not the following TR field is present.

0: TR field is not present.
1: TR field is present.

U.4.1.3	Temporal Reference (TR) (8/10 bits)

When present, TR is an eight bit number unless a custom picture clock frequency is in use, in which case it is a ten bit number consisting of the concatenation of ETR and TR of the picture header.

#### U.4.1.4	Temporal Reference for Prediction Indicator (TRPI) (1 bit)

TRPI indicates whether or not the following TRP field is present.

0: TRP field is not present.
1: TRP field is present.

TRPI shall be equal to zero whenever the picture is an I or EI picture.

#### U.4.1.5	Temporal Reference for Prediction (TRP) (10 bits)

TRP is a ten bit number. If a custom picture clock frequency was not in use for the reference picture, the two MSBs of TRP are zero and the LSBs contain the eight bit TR found in the picture header of the reference picture. If a custom picture clock frequency was in use for the reference picture, TRP is a ten bit number consisting of the concatenation of ETR and TR from the reference picture header.

When present (as indicated in TRPI), TRP has two meanings:

- When the ERPS layer is not present (ERPSI is set to ‘0’), TRP indicates the Temporal Reference which is used for prediction of the encoding, except in the case of B pictures and the B-picture part of an Improved PB frame. For B pictures or the B-picture part of an Improved PB frame, the picture having the temporal reference TRP is used for the prediction in the forward direction. (Prediction in the reverse-temporal direction always uses the immediately temporally-subsequent picture.) When TRP is not present, the most recent temporally-previous anchor picture shall be used for prediction, as when not in the Reference Picture Selection mode. TRP is valid until the next PSC, GSC, or SSC.

- When the ERPS layer is present (ERPSI is set to ‘1’), TRP specifies that all pictures in the complete multi-frame buffer having buffer-index less than the picture associated with TRP are not used for motion compensation. The picture in the multi-frame buffer assigned to TRP has buffer index 0, the pictures in the multi-frame buffer having buffer-index greater than the picture associated with TRP are available for motion compensation for P-pictures. It is assumed that the decoder replicates the multi-frame buffer of the encoder according to the specified buffering mode. The TRs are attached to the buffered frames. For B-pictures or the B-picture parts of an Improved PB frame, the picture having the temporal reference TRP is used for the prediction in the forward direction. (Prediction in the reverse-temporal direction always uses the immediately temporally-subsequent picture.) If TRP is not present, the buffer remains as assembled.

#### U.4.1.5	Enhanced Reference Picture Selection layer (ERPS) (Variable Length)

The ERPS layer is present only if Enhanced Reference Picture Selection is indicated and if ERPSI is set to ‘1’. The ERPS layer specifies the buffer indexing to decode the current frame. The Number of Reference Picture Active (NRPA) indicates how many reference pictures out of the multi-frame buffer are used for motion compensation. The Reference Picture Buffer Sub-sampling mode (RPBS) indicates what kind of pointer table is used to reference the NRPA frames in the multi-frame buffer. If RPBS indicates the use of a pointer table other than incrementally indexing according to the assembled buffering scheme, the fields of Number of Indices to be Remapped (NRI) and Reference Picture Selection (RPS) are present to specify the used pointer table. Moreover, the ERPS layer specifies the buffering of the currently decoded frame. The Reference Picture Buffering mode specifies the buffering of the currently decoded frame. Two modes are specified. The Sliding Window mode operates as a (First-In-First-Out) FIFO buffer. The Adaptive Remove/Add Picture (ARAP) mode permits a more general buffering scheme. The fields of RPI, RPP, API, and APP are present if the ARAP mode is selected permitting flexible buffer handling.

#### Number of Reference Pictures Active (NRPA) (Variable Length)

NRPA is a variable length code word that is present only if Enhanced Reference Picture Selection layer is present and the picture coding type indicates a P-picture. NRPA specifies the number of active reference frames for decoding the current picture. The NRPA field is transmitted using Table U.1. The corresponding index in Table U.1 is equal to NRPA-1. If NRPA indicates the use of only 1 reference picture (index=0), the macroblock syntax remains unchanged and the syntax extentions in section U.1.4.6 do not apply.

#### Reference Picture Buffer Sub-sampling mode (RPBS) (Variable length)

RPBS is a variable length code word that is present only if the Enhanced Reference Picture Selection layer is present and the picture coding type indicates a P-picture. RPBS indicates the multiple reference picture buffer sub-sampling mode. The RPBS code word does not change the buffering of decoded reference frames. It provides a pointer table to use for decoding the currently transmitted frame. The RPBS mode is applied relative to the shifted buffer indexing if TRP is present. Otherwise, it is applied to the non-subsampled buffer.

Table U.2/H.263.

Modes of operation for Reference Picture Buffer Sub-sampling.

| Code word | RPBS mode |
| --- | --- |
| ‘0’ | No sub-sampling |
| ‘10’ | Sub-sampling |
| ‘11’ | Reserved for future use |

If RPBS is set to ‘0’, meaning no sub-sampling, all pictures in the multiple reference picture buffer starting from index 0 ending with index NRPA-1 may be utilized for prediction. The picture indexing is kept the same. If RPBS is set to ‘10’, meaning sub-sampling, the multiple reference picture buffer is sub-sampled. The sub-sampled NRPA reference pictures are indexed as 0 through NRPA-1 in the order of their selection using the following two fields.

### Number of Remapped Indices (NRI)

A variable length code word that is present only if Enhanced Reference Picture Selection layer is indicated and the RPBS is set to ‘10’. NRI is transmitted using Table U.1 indicating the number of indices for which an index mapping is transmitted. The indices for which no index mapping is transmitted are used in their default incremental order by removing those index values with index mapping from the non-subsampled multiframe buffer.

### Reference Picture Selection (RPS) (Variable length)

A variable length code word that is present only if Enhanced Reference Picture Selection layer is indicated and the RPBS is set to ‘10’. RPS is transmitted using Table U.1 indicating which picture is to be sampled out of the multiple reference picture buffer. The RPS code word is transmitted as many times as the value of the NIR symbol. The buffer-indexing applied to decode the current picture follows the order of the selection in increasing numbers.

### Reference Picture Buffering mode (RPB) (Variable length)

A variable length code word that is present only if the Enhanced Reference Picture Selection layer is indicated. RPB specifies the buffering of the currently decoded picture. RPB is carried out relative to the indexing of the non-subsampled buffer. The multiple reference picture modes are signaled using Table U.3.

Table U.3./H.263

Modes of operation for multiple reference picture buffer management.

| Code word | RPB mode |
| --- | --- |
| ‘0’ | Sliding Window |
| ‘10’ | Adaptive Remove/Add Picture |
| ‘11’ | Inherited RPB mode |

The Sliding Window mode stands for removing the picture with index NRPA-1 in the buffer and adding the current decoded picture at position 0 to the multiple reference picture buffer. The Adaptive Remove/Add Picture mode stands for a more general version of the sliding window mode. Here any picture can be removed from the multiple reference picture buffer while the currently decoded picture can be inserted at any position into the buffer. This mode should be used to build-up a multiple reference picture buffer. Finally, the Inherited RPB mode stands for applying the RPB mode specified in the Picture layer in the GOB or Slice layer. This mode is not valid if chosen in the Picture layer. If the Inherited RPB mode is chosen on the GOB or Slice layer, the RPB mode is either set to ‘0’ or ‘10’.

### Remove Picture Indication (RPI) (1 bit)

RPI is a fixed length code word that is present only if the Enhanced Reference Picture Selection layer is present and if the RPB code is set to ‘10’. If RPI is set to ‘0’, no picture is removed from the memory unless a picture is added and the physical multiple reference picture buffer size is exceeded. In that case, the picture with the largest index in the complete physical buffer is removed from the multiple reference picture buffer. If RPI is set to ‘1’, a picture is removed from the multiple picture memory and the RPP field is following.

### Remove Picture Position (RPP) (Variable length)

RPP is a variable length code word that is present only if the Enhanced Reference Picture Selection layer is present and if RPI is set to ‘1’. The code table for the picture reference parameter (PR) as given in Table U.1 is used to specify the index m of the picture that is to be removed from the multiple reference picture buffer. After removing the indexed picture from the mth position in the multiple reference picture buffer, the indexes greater than m are decrement by one.

### Add Picture Indication (API) (1 bit)

API is a fixed length code word that is present only if the Enhanced Reference Picture Selection layer is indicated and if the RPB code is ‘10’. If API is set to ‘0’, the currently decoded picture is not added to the multiple picture memory. If API is set to ‘1’, the currently decoded picture is added to the multiple picture memory and the APP field is following.

### Add Picture Position (APP) (Variable length)

APP is a variable length code word that is present only if the Enhanced Reference Picture Selection layer is present and the API is set to ‘1’. The VLC as given in Table U.1 is used to indicate at which position the currently decoded frame is to be added to the multiple frame memory. After adding the currently decoded frame at the m’th position in the multiple frame memory, the indexes greater or equal than m are incremented by one.

### Pruning Information Indicator (PII) (1bit)

The PII is a bit that is present only if the ERPS layer is present. PII indicates whether or not data fields describing the sub-frame pruning process are present. If the PII is set to “1,” the pruning information data fields are present. If the PII is set to “0,” the data fields are not present. No pruning information may be sent in an ERPS layer occurring in a GOB or slice header. Consequently, the PII bit shall be set to ‘0’ in any ERPS layer occurring in a GOB or slice header. If Reference Picture Pruning is in use, the PII must be set to “1” in the ERPS layer in the PLUS header of an intra-frame to indicate the presence of sub-frame dimension information. For details regarding Reference Picture Pruning, see Section U.5.

### Sub-Frame Dimension Indicator (SDI) (1 bit)

The SDI is a bit that is present only if the PII is set to “1.”  SDI indicates whether or not data fields describing sub-frame dimensions are present. If the SDI is set to “1,” the data fields describing sub-frame dimensions are present. If the SDI is set to “0,” the data fields are not present. The encoder must send sub-frame dimension information every intra-frame but may send the information more frequently. A change in the sub-frame dimensions must be indicated in the ERPS layer of an intra-frame.

### Horizontal Sub-frame Dimension (HSD) (6 bits)

A 6 bit codeword that is present only if the SDI is set to “1.”  The HSD specifies the horizontal dimension of a sub-frame in units of macroblocks. Each frame has a width of ceil(fwMB / sfwMB) sub-frames where fwMB is the width of a frame in macroblocks and sfwMB is the width of a sub-frame in macroblocks. For positive numbers, the ceiling function, ceil(x), equals x if x is an integer otherwise ceil(x) equals one plus the integer part of x. The horizontal dimension must be an integer multiple of the horizontal size of the MPU.

### Vertical Sub-frame Dimension (VSD) (6 bits)

A 6 bit codeword that is present only if the SDI is set to “1.”  The VSD specifies the vertical dimension of a sub-frame in units of macroblocks. Each frame has a height of ceil(fhMB / sfhMB) sub-frames where fhMB is the height of a frame in macroblocks and sfhMB is the height of a sub-frame in macroblocks. For positive numbers, the ceiling function, ceil(x), equals x if x is an integer otherwise ceil(x) equals one plus the integer part of x. The vertical dimension must be an integer multiple of the vertical size of the MPU.

### Sub-frame Pruning Indicator (SPI) (1 bit)

The SPI is a bit that is present only if the PII is set to “1.”  SPI indicates whether or not data fields describing sub-frame pruning are present. If the SPI is set to “1,” the data fields describing sub-frame pruning are present. If the SPI is set to “0,” the data fields are not present.

### Number of Pictures to Prune (NPP) (Variable Length)

A variable length codeword that is present only if the SPI is set to “1.”  The NPP uses the code table for the picture reference parameter given in Table U.1 of the draft Annex U specification to indicate the number of reference pictures to be pruned.

### Reference Picture Selection (RPS) (Variable Length)

A variable length codeword that is present only if the SPI is set to “1.”  The RPS uses the code table for the picture reference parameter given in Table U.1 of the draft Annex U specification to indicate which reference picture is to be pruned. The RPS codeword and associated sub-frame pruning bitmap (described below) are always transmitted as pairs and are transmitted N times where N is the value of NPP.

### Sub-frame Pruning Bitmap (SPB)

A fixed length codeword that contains one bit for each sub-frame and is present only if the SPI is set to “1.”  The sub-frames are numbered in raster scan order starting from the upper-left of the frame. For example, consider a case in which a reference picture, specified by RPS, is partitioned into six sub-frames. Let “s1 s2 s3 s4 s5 s6” represent the six bits in the SPB. If bit si = “1”, then the decoder should prune the ith sub-frame in the reference picture. For instance, if the SPB = “000110” then sub-frames 4 and 5 are pruned. To prevent start code emulation, a “1” is inserted (if necessary) into the bitmap in the 15th, 30th, 45th, etc. bit position. The SPB codeword and associated RPS are always sent as pairs and are transmitted N times where N is the value of NPP.

### TR Check Indicator (TRCI) (1 bit)

TRCI is a fixed length code word that is present only if the Enhanced Reference Picture Selection layer is present. TRCI indicates whether the TRC field which is placed after the macroblock layer is present. A value of 0 indicates that TRC is not present. A value of 1 indicates that TRC is present.

### TR Check (TRC) (12 bit)

A fixed length code word that is present only if TRCI has a value of 1. When present, TRC is a 12 bit field that is generated by calculating a cyclic redundancy check (CRC) over a set of TR fields. Its purpose is to assure that the encoder and decoder use the same reference frames for prediction when using the picture reference parameter (PR) at the macroblock layer. Though this is not necessary in error free environments, a mismatch between the content of the reference picture buffer at the encoder and decoder may result from transmission errors. In this case it is important to detect the mismatch at the decoder in order to initiate appropriate recovery mechanisms, e.g. requesting a fast INTRA update. Using TRC it is possible to detect the mismatch by comparing the received value with the locally generated CRC.

The CRC is calculated as follows. First, the appropriate set of TRs must be determined. For each transmitted PR in the macroblock layer, the corresponding TR is selected. If a PR is selected, it is labeled not to be used for CRC again. Second, a message block is built from the selected set of TRs. The TR values are represented by 10-bit numbers and concatenated to the message block in the order of their transmission in the macroblock layer. Finally, in the third step, the CRC is calculated from the message block. The generator polynomial is g(x) = x12 + x11 + x3 + x2 + x + 1. The resulting 12-bit CRC is assigned to TRC.

### U.4.1.6	Macroblock layer syntax

The macroblock layer syntax is modified against H.263 version 2 syntax for P-pictures if the number of selected reference pictures (NRPA value) is greater than one. The field NRPA is signaled in the ERPS layer.

If the COD bit is set to "1", no further information is transmitted for this macroblock. In that case, the decoder shall treat the macroblock as an INTER macroblock with motion vector for the whole block equal to zero, picture reference parameter equal to zero, and with no coefficient data. If the COD bit is set to "0", indicating that the macroblock is coded, the syntax of the macroblock layer is depicted in Figure U.8 with the fields PR0, PR, PR2, PR3, and PR4 being inserted into Figure 10. PR0, PR, PR2, PR3, and PR4 consist of a variable length code word as given in Table U.1.

If the field PR0 is not set to zero, no further information is transmitted for this macroblock; in that case the decoder shall treat the macroblock as an INTER macroblock with motion vector for the whole block equal to zero, picture reference parameter equal to PR0, and with no coefficient data. If three times PR0 is set to 1 (code word ‘000’ is transmitted), a bit ‘1’ is inserted to prevent start code emulations.

If the field PR0 is set to zero, the macroblock is coded. The meaning of fields MCBPC, MODB, CBPB, CBPY, and DQUANT remains unaltered. The field PR is included together with the field MVD for all INTER macroblocks (in PB frames mode also for INTRA macroblocks).

The three code words PR2-4 are included together with MVD2-4 if indicated by PTYPE and if MCBPC specifies the INTER4V type. PR2-4 and MVD2-4 are only present when in Advanced Prediction mode (see Annex F) or Deblocking Filter mode (see Annex J).

FIGURE U.7/H.263

H.263 macroblock syntax layer.

## U.5  Reference Picture Pruning

The Reference Picture Pruning sub-mode reduces, on average, the amount of memory required to save multiple reference pictures. The memory reduction is accomplished by partitioning each reference picture into sub-frames. While encoding, the encoder may discard (“prune”) sub-frames from a reference picture in the multi-frame buffer. The strategy used by the encoder to decide which of the sub-frames to prune is outside the scope of this document. The encoder signals to the decoder the size of the sub-frames and which of the sub-frames to prune using the enhanced reference picture selection (ERPS) layer whose syntax is described in Section U.4. The decoder shall not prune the most recent frame checked into the Annex U multi-frame buffer to ensure that at least one complete reference picture resides in the buffer at all times.

While it is suggested that the encoder refrain from referencing pixels in sub-frames that it has pruned, it is sometimes difficult to avoid doing so (e.g. when OBMC of Annex F is in use). In the event that the encoder references a pixel in a sub-frame that it has pruned, the decoder shall reference the pixel at the same position in the closest temporally succeeding picture from which a sub-frame containing the pixel has not been pruned. The temporal order of the pictures in the multi-frame buffer is given by their TR field which is attached to them during the buffer management process as specified in this annex. If no temporally succeeding picture contains the pixel, the closest temporally preceding picture containing the pixel is used instead.

As an example of how Reference Picture Pruning may be used effectively, consider two reference pictures whose upper-right quadrant are nearly identical. If all sub-frames completely contained within that quadrant were pruned from the older reference picture, then an encoder wishing to reference pixels in the upper-right quadrant could reference those pixels in the newer picture with little or no loss in coding efficiency.

The Reference Picture Pruning capability is negotiated by external means (e.g. Recommendation H.245). In addition, the decoder signals, also by external means, the minimum partition unit (MPU) which is described in terms of a minimum vertical and horizontal size (in macroblocks) of a sub-frame and the amount of memory it has available for its multi-frame buffer. Memory management is facilitated by the simple partition rules described below.

Each reference picture is partitioned into rectangular sub-frames of equal size. The encoder specifies the sub-frame size which must be an integer multiple of the MPU. That is, the sub-frame’s vertical and horizontal dimensions must be integer multiples of the minimum sizes signaled externally by the decoder. The sub-frame dimensions must be sent with every intra-frame but may be sent more frequently if desired. In addition, the upper-left-hand corner of the first sub-frame is coincident with the upper-left-hand corner of the reference picture. Consequently, the entire partition may be described by specifying the horizontal and vertical dimensions of a sub-frame. Sub-frames may extend beyond the right and bottom boundaries of the reference picture. When a sub-frame that extends past the reference picture boundary is saved, the decoder may wish to set aside enough memory to save the entire sub-frame and not just the memory necessary to save the portion of the reference picture that lies within that sub-frame. A decoder designed such that each sub-frame occupies the same amount of memory will prevent the possibility of memory fragmentation.

Example partition

Sub-frame 2

Picture Border

FIGURE U.8/H.263

Example Partition with Sub-frames Extending Past the Picture Border.

Finally, whenever the reference picture partition is redefined, which may occur when the source picture format changes, the encoder must send the new sub-frame dimensions using the ERPS layer as specified in the following section. A partition redefinition is recognized only if it is sent in the PLUS header of an intra-frame. All reference pictures that have been pruned using the old partition should be discarded.

## U.6	Decoder Process

The decoder of this mode may need an additional number of picture memories to store the decoded video signals and their Temporal Reference (TR) information. If the Enhanced Reference Picture Selection layer is present, the decoder stores the reference pictures for inter-frame decoding in a multi-frame buffer-based scheme with buffer indexes. When the field of TRP is present, the buffer indexes are modified. The reference picture that is addressed with TRP is associated with buffer index 0. The reference pictures associated with indexes greater than the TRP picture are assigned new indexes. These indexes are given by subtracting the old indexes of these pictures by an offset that is the value of the old index of the TRP picture. The indexes into the buffer can be further re-arranged by the reference picture buffer sub-sampling mode. The buffering of the currently decoded picture can be specified using the reference picture buffering mode. The buffering may follow a first-in, first-out mode. Alternatively, the buffering may follow a customized adaptive scheme specified by the encoder in the forward channel.

When any employed reference picture is not available at the decoder or the CRC for the TRs in the multi-frame buffer indicates an error, the decoder may send a forced INTRA update signal to the encoder by external means (see for example Recommendation H.245). The decoder replicates the multi-frame buffer of the encoder according to the specified buffering mode. The TRs are attached to the buffered frames. The buffering scheme is operated also when partially erroneous frames are decoded according to the specified buffer mode information.

Multi-frame motion compensation proceeds in a similar way as Reference Picture Selection. They differ in that, the reference picture is variable as indicated in the PR. In case four motion vectors per macroblock are used and the NRPA field indicates the use of more than one reference picture, the motion vector for both chrominance blocks is inherited from the first of the four motion vectors. For that, the motion vector for both chrominance blocks is derived by dividing the component values of the first of the four macroblock values by two, due to the lower chrominance format. The component values of the resulting quarter pixels resolution values are modified towards the nearest half pixel position as indicated in Table 18.

If the Enhanced Reference Picture Selection layer is not present, the mode is operated as if Reference Picture Selection (see Annex N) were enabled without VideoMux back-channel.

Finally, let us give an example for a possible operation of this mode: